3GPP TSG RAN WG4 Meeting #114-bis	R4-2504032

Wuhan, China, 7th – 11th April, 2025

Agenda item:			8.2

Source:	vivo

Title:	Discussion on measurement quantities for IoT NTN

Document for:	Discussion

# Introduction

In the RAN1#120 meeting, an LS [1] on measurement quantities for IoT NTN was approved. RAN4 is requested to answer the questions and provide feedback.

- Overall description RAN1 discussed whether to support measurement quantities (i.e., DL RS TX power, Received Interference Power, and Thermal Noise Power) for IoT NTN measurement, and agreed to consult RAN4 based on the following agreement: Agreement Send an LS to ask RAN4 the following: - Whether any of the following measurement quantities, described in clause 5.2 of TS 36.214, are applicable for NB-IoT-based NTN and/or eMTC-based NTN: - DL RS TX power - Received Interference Power - Thermal Noise Power - For the measurement quantitie(s) that are applicable, if any, whether reference point(s) need to be defined.. Question1: Whether any of the following measurement quantities, described in clause 5.2 of TS 36.214, are applicable for NB-IoT-based NTN and/or eMTC-based NTN: - DL RS TX power - Received Interference Power - Thermal Noise Power Question2: For the measurement quantitie(s) that are applicable, if any, whether reference point(s) need to be defined? - Actions To RAN4 ACTION: RAN1 respectfully asks RAN4 to take the above information into consideration and provide feedback on the above questions.

In this contribution, we provide our views on measurement quantities for IoT NTN.

# Discussion

## 2.1 Background

During RAN1 discussions, it was found out that the supported E-UTRN measurements for IoT NTN were unclear. It is not clear whether the E-UTRN measurements defined in TS 36.214 is applicable to IoT NTN or not. Furthermore, reference point for the measurement is also unclear if it is applicable.

RAN1 identified the following E-UTRN measurements may be applicable for NB-IoT-based NTN and/or eMTC-based NTN and asks RAN4 for confirmation.

After reviewing the historical discussions on E-UTRAN measurement in LTE TN, the purposes of these measurements and the corresponding spec versions introduced for LTE TN were summarized in Table 1.

Table 1. Purposes of E-UTRAN measurement for TN

| Measurement quantities | 1st Rel | Purposes |
| --- | --- | --- |
| DL RS TX power | R8 | Mobility and UL power control |
| Received Interference Power | R8 | Related to generation of Overload Indicator |
| Thermal noise power | R8 | Related to triggering of the interference overload indication |

DL RS Power: for mobility and UL power control

DL TX power was proposed by R1-072977/R1-073214 in RAN1#49b. It is closely related to the RSRP measurement for mobility, as well as to the uplink power control. eNodeB must configure the DL TX power to the UE so that the UE can derive the pathloss between eNB and UE based on RSRP. Since the transmit power of the DL reference signal is sent to the UE through a standardized interface, it meets the conditions for eNodeB-related measurements introduced in TS 36.214.

Received Interference Power: Related to Overload Indication generation

In addition to DL TX power, several other L1 eNB measurements, including their purposes (e.g., Load Balancing), receiving nodes, and definitions, were discussed in R1-073787. RAN1 was subsequently tasked by RAN3, via LS R3-072444, to analyze the characteristics of the information needed to estimate the load on the radio interface to optimize handover/cell reselection parameters. Ultimately, in RAN1#52 bis, RAN1 agreed that Overload Indication should be generated based on a standardized eNodeB measurement of noise + interference (yellow highlighted below). As a result, the Received Interference Power, defined as the uplink received interference power including thermal noise, was introduced in 36.214 in RAN1#53.

RAN1#52 bis agreement UL ICIC - Overload Indication details - Way forward: OI generation on the basis of a standardized eNode B measurement of noise + interference - RSRP measurements reporting triggers - Way forward: To introduce an additional trigger for RSRP measurements for ICIC - The trigger should in addition to the existing RSRP triggering criteria also take buffer status and valid grant of the UE into account. - For DL ICIC: If the UE has a DL grant but no UL grant, a timer could be used instead of the UL grant to trigger a report. - It is foreseen that also the RSRP reports triggered for ICIC purposes should be transmitted by higher layer signalling. - Details on how to design such a trigger and the reporting would be the responsibility of the RAN2 working group.

Thermal noise power: Related to triggering of Overload Indication

The triggering of Overload Indication was also discussed, with a proposal in R1-081973 suggesting that the trigger is based on eNB measurements of interference relative to thermal noise. If the defined threshold is met, an Overload Indication is signaled via X2. In the end, thermal noise power was introduced by CR R1-082187.

## 2.2 Support of E-UTRN measurement for IoT NTN

In TS 36.108, the requirements for DL RS power for IoT NTN are specified in clause 6.5.4.

#### TS 36.108 for IoT NTN ### 6.5.4 DL RS power #### 6.5.4.1 General For E-UTRA, DL RS power is the resource element power of the Downlink Reference Symbol. The absolute DL RS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL RS power indicated on the DL-SCH and the DL RS power of each E-UTRA carrier at the SAN TAB connector for SAN type 1-H. For NB-IoT, DL NRS power is the resource element power of the Downlink Narrow-band Reference Signal. The absolute DL NRS power is indicated on the DL-SCH. The absolute accuracy is defined as the maximum deviation between the DL NRS power indicated on the DL-SCH and the DL NRS power of each NB-IoT carrier at the SAN TAB connector for SAN type 1-H. #### 6.5.4.2 Minimum requirements For E-UTRA, DL RS power of each E-UTRA carrier shall be within 2.1 dB of the DL RS power indicated on the DL-SCH. For NB-IoT, DL NRS power of each NB-IoT carrier shall be within 2.1 dB of the DL NRS power indicated on the DL-SCH.

In current specification TS 36.108 for IoT-NTN, the measurement of DL RS Tx power is already supported and accuracy requirement is also specified. Therefore, it can be confirmed that measurement of DL RS Tx power is supported for IoT-NTN.

Proposal 1. RAN4 confirms that E-UTRN measurement of DL RS Tx power is supported for IoT-NTN.

For measurement of received interference power and thermal noise power, it is mainly used for load balancing in TN networks. In general, the scheme for load balancing would also be needed in IoT-NTN networks. To keep consistency between TN and NTN networks, it is suggested such scheme is also supported for IoT-NTN.

Proposal 2. RAN4 confirms that E-UTRN measurements of received interference power and thermal noise power are supported for IoT-NTN.

## 2.3 Reference point

In TS 36.214, the reference point for E-UTRAN measurement is the TX or RX antenna connector, whereas "antenna connector" refers to the "BS antenna connector" test port A and test port B of non-AAS base station as described in TS 36.104, which is for TN only. It does not refer to TS 36.108 for the reference point of the E-UTRAN measurement. Consequently, the definition of E-UTRAN measurement for IoT NTN is not complete.

TS 36.214 [10] 3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception ". ## 5.2 E-UTRAN measurement abilities The structure of the table defining a E-UTRAN measurement quantity is shown below. Column field; Comment. Definition; Contains the definition of the measurement.. The term "antenna connector" used in this clause to define the reference point for the E-UTRAN measurements refers to the "BS antenna connector" test port A and test port B as described in [10]. The term "antenna connector" refers to Rx or Tx antenna connector as described in the respective measurement definitions. ### 5.2.1 DL RS TX power Definition; Downlink reference signal transmit power is determined for a considered cell as the linear average over the power contributions (in [W]) of the resource elements that carry cell-specific reference signals which are transmitted by the eNode B within its operating system bandwidth. For DL RS TX power determination the cell-specific reference signals R0 and if available R1 according TS 36.211 [3] can be used. The reference point for the DL RS TX power measurement shall be the TX antenna connector.. ### 5.2.2 Received Interference Power Definition; The uplink received interference power, including thermal noise, within one physical resource block's bandwidth of resource elements as defined in TS 36.211 [3]. The reported value shall contain a set of Received Interference Powers of physical resource blocks as defined in TS 36.211 [3]. The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches.. ### 5.2.3 Thermal noise power Definition; The uplink thermal noise power within the UL system bandwidth consisting of resource blocks as defined in [3]. It is defined as (No x W), where No denotes the white noise power spectral density on the uplink carrier frequency and denotes the UL system bandwidth. The measurement is optionally reported together with the Received Interference Power measurement, it shall be determined over the same time period as the Received Interference Power measurement, The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches..

The reference points for requirement are specified in TS 36.108 as below.

TS 36.108 SAN type 1-H: Satellite Access Node operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB. SAN type 1-O: Satellite Access Node operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB. ====omitted==== ## 4.3 Requirement reference points ### 4.3.1 SAN type 1-H For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. ====omitted==== How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause. ### 4.3.2 SAN type 1-O For SAN type 1-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Figure 4.3.2-1: Radiated reference points for SAN type 1-O

The above reference points for requirement can be used to define the reference point for E-UTRN measurements for IoT-NTN.

Proposal 3: The reference point for DL RS Tx power shall be:

-	for SAN type 1-H TS 36.108: the Tx Transceiver Array Boundary connector,

-	for SAN type 1-O TS 36.108: the Tx antenna (i.e., the centre location of the radiating region of the Tx antenna).

Proposal 4: The reference point for received interference power and thermal noise power shall be:

-	for SAN type 1-H TS 36.108: the Rx Transceiver Array Boundary connector,

-	for SAN type 1-O TS 36.108: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna).

# Summary

In this contribution, we provided views on measurement quantities for IoT NTN. Following proposals are present.

Proposal 1. RAN4 confirms that E-UTRN measurement of DL RS Tx power is supported for IoT-NTN.

Proposal 2. RAN4 confirms that E-UTRN measurement of received interference power and thermal noise power is supported for IoT-NTN.

Proposal 3: The reference point for DL RS Tx power shall be:

-	for SAN type 1-H TS 36.108: the Tx Transceiver Array Boundary connector,

-	for SAN type 1-O TS 36.108: the Tx antenna (i.e., the centre location of the radiating region of the Tx antenna).

Proposal 4: The reference point for received interference power and thermal noise power shall be:

-	for SAN type 1-H TS 36.108: the Rx Transceiver Array Boundary connector,

-	for SAN type 1-O TS 36.108: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna).

- References

- R1-2501512 LS on measurement quantities for IoT NTN, vivo, RAN1

- Appendix: Draft reply LS on measurement quantities for IoT NTN

3GPP TSG RAN WG4 Meeting #114-bis	R4-25x

Wuhan, China, 7th – 11th April, 2025

Title:	Reply LS on measurement quantities for IoT NTN

Response to:	R1-2501512

Release:	Rel-18

Work Item:	IoT_NTN_enh-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 Thanks RAN1 for the LS on measurement quantities for IoT NTN. RAN4 discussed the questions in the LS and made following agreement.

- RAN4 confirms that E-UTRN measurement of DL RS Tx power is supported for IoT-NTN.

- RAN4 confirms that E-UTRN measurements of received interference power and thermal noise power are supported for IoT-NTN.

- The reference point for DL RS Tx power shall be:

  - -	for SAN type 1-H TS 36.108: the Tx Transceiver Array Boundary connector,

  - -	for SAN type 1-O TS 36.108: the Tx antenna (i.e., the center location of the radiating region of the Tx antenna).

- The reference point for received interference power and thermal noise power shall be:

  - for SAN type 1-H TS 36.108: the Rx Transceiver Array Boundary connector,

  - for SAN type 1-O TS 36.108: the Rx antenna (i.e., the center location of the radiating region of the Rx antenna).

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above agreements into consideration in the future work.

3. Reference:

- R1-2501512 LS on measurement quantities for IoT NTN, vivo, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta