3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504035

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.1.1.3.2

Source:	Ericsson

Title:	Discussion on the MIMO layers for 6Rx UE

Document for:	Approval

# Introduction

With advancements in hardware and design, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. For instance, certain foldable mobile phones have adopted 6Rx antennas. Incorporating 6Rx facilitates a higher peak data rate by supporting more DL MIMO layers than 4Rx configurations. On the other hand, due to the higher diversity, it can also help increase the downlink coverage by improving the receiver sensitivity.

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1], and later on, RAN revised the WI to make it clearer to support 6Rx for FWA devices [2].

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Introduce 6 MIMO layers support as an optional feature only for FWA.

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The following sections will discuss the open issues on the 6Rx MIMO layers.

# Support of MIMO layers

The progress of MIMO 6 layers has been very slow during the last meeting, the WF on the MIMO layers for 6Rx are captured below [3]:

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices

Way Forward:

-	RAN4 Chair to take note of the following in the meeting report:

-	“RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19”

-	6-layer performance requirements will be only defined for FWA.

-	4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

-	The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4.

Issue 2-1-2: 6-layer Support as optional feature for FWA

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following.

-	Introduce 6 MIMO layers support as an optional feature only for FWA.

As we have pointed out in the introduction, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. Numerous simulation results show that supporting a higher number of DL MIMO layers can improve the DL peak throughput.

The radio channel and several other factors impact the number of layers that can be supported, and these should be thoroughly studied and analyzed.

One factor is the receive antenna correlation. When antenna elements are closely spaced, the correlation between them increases.  In the previous 8Rx discussions, the objective is to define the requirement for FWA and CPE UEs. Given their larger form factor compared to the handheld UE, the demodulation performance study assumed a low antenna correlation [4].

Another factor is the likelihood of getting a rank 6 channel in a practical deployment scenario. Rank distributions depend on SINR distribution and propagation conditions. For example, the likelihood of getting a higher rank is low in a deployment scenario with a larger ISD and rural or suburban areas. With a relatively small ISD and dense urban area, the probability of getting a higher rank is probably high.

In the latest version of 6Rx WI, the objective was revised to reflect what has been agreed in the RAN4 #114 WF, with the addition of ‘‘Introduce 6 MIMO layers support as an optional feature only for FWA’’ [2][3]. To our understanding, MIMO 6 layers are supported for FWA devices as an optional feature, the key question is how to address the applicability for FWA.

- MIMO 6 layers are supported for FWA devices as an optional feature, the key question is how to address the applicability for FWA.

Regarding the simulation assumption, RAN4 is unable to conclude on a common set of simulation assumptions to determine the feasibility of 6 layers for handheld UE in Rel-19, and this was captured in the chairmen’s note shown below. The detailed discussion will be carried out in the RAN4 Demod session with additional contributions forthcoming.

R4-2503014	WF on 6Rx requirements

Type: other		For: Approval
					Source: AT&T

Chair: RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19

Decision:		Approved.

There were some discussions on whether new signaling of UE types is needed. From the network perspective, gNB does not necessarily need to know the UE type but the UE capabilities to manage resource, enable features, etc.

Similar to 8Rx, the applicability rule could apply to 6 layers for FWA as well, and this can be captured in the RAN4 Demod discussion and no new signaling is needed.

- The applicability of 6 layers for FWA can be captured in the RAN4 Demod discussion and no new signaling is needed.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	MIMO 6 layers are supported for FWA devices as an optional feature, the key question is how to address the applicability for FWA.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The applicability of 6 layers for FWA can be captured in the RAN4 Demod discussion and no new signaling is needed.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2503014, WF on 6Rx requirements, AT&T.

- R4-2405154, Draft BigCR on 38.101-4 for introduction of 8Rx performance requirements