3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504038

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.12.1.1

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE inter-band CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the inter-band are shown below:

Issue 1-1-2: UE antenna type for inter-band CA

Agreement:

- Refine the agreement from last meeting as:

  - For intra-band contiguous CA, same antenna type should be applied on a band.

  - For inter-band CA, either different or same antenna type can be applied on different bands.

  - For inter-band CA, consider the following cases:

    - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC.

    - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC.

    - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

    - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.]

    - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

Further check which understanding is correct:

- Understanding #1: Case 4 is excluded in RF session.

Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

Agreement:

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

  - Inter-band CA

### ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam

If both the PCC and SCC are using the same receive beam, that implies that the UE is using a single beamforming to receive both bands. However, since they are inter-band, the frequencies are different. Beamforming is frequency-dependent, as the beam direction and shape depend on wavelength.

For inter-band CA (where PCC and SCC are in different frequency bands), using a single common Rx beam would require the same phase shifts to align signals across different frequencies. This is generally infeasible unless the band bands are very close in frequency. The bands n3 and n39 are defined in the table below.

Table 1 Operating bands for n3 and n39

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |

In most practical inter-band CA deployments, e.g. combing low-band and mid-band, separate beam configurations are necessary to optimize reception for each band.

- The feasibility of the ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam is highly dependent on the frequency separation

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The feasibility of the ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam is highly dependent on the frequency separation

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502952, WF on UE RF requirements for ATG

- TS 38.101-1, V18.5.0