3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504039

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.12.1.2

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, and the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the UL MIMO are shown below:

Issue 3-1: ULFPTx mode for UL-MIMO

- Proposals

- Option 1: ULFPTx Mode is not relevant to this feature. (ZTE)

- Option 2: ULFPTx mode still applies for ATG UE to support different implementation (CMCC, LGE, Ericsson)

- Option 3: Depend on ATG UE implementation whether to leverage the existing ULFPTx mode. (Huawei)

WF:

FFS

Issue 3-2: whether preclude the single antenna port related requirement

- Proposals

- Option 1: no (CMCC, LGE, Ericsson, Huawei)

- Option 2: not to consider the single antenna port transmission TxD for ATG UE operation. (ZTE)

WF:

FFS

There are still many open issues that need to be addressed. The following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

For ATG UE UL-MIMO, in the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors, the uplink full power transmission (ULFPTx) mode is applicable. From Rel-16 onwards, with the support of ULFPTx mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full-power PA in full power mode. This ensures that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power. For ATG UE, the power is based on the declaration, and the declaration is per band, not per layer [3]. Without the ULFPTx support, UE needs a new signaling to inform the network when it is transmitting at full power mode.

An example of ATG UE ULFPTx mode was proposed in [4], which could be adopted into the specification.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

- ULFPTx mode should be supported for ATG UL-MIMO

### Single antenna port related requirement for ATG UE

As stated in the previous session, the ATG UE with one antenna port would still be configured, i.e. single layer UL MIMO operation, but with two Tx antenna connectors.

The key question would be under what circumstances or scenarios the single antenna port would be scheduled.

ATG channels often involve high-altitude aircraft with minimal scatters, e.g. at cruising altitude. This results in a rank-deficient channel matrix (rank 1), preventing spatial multiplexing. Another factor is the antenna configuration or arrangement. If the UE antenna is closely spaced, spatial correlation increases, reducing the effective MIMO rank. This forces the network to schedule a single layer for reliability. At cell edges or in noisy environments, the network may prioritize reliability over throughput. Single-layer transmission with robust modulation and coding is preferred to avoid errors in multi-layer transmissions.

All in all, the ATG UEs are likely to use single-layer operation in LoS-dominated, lower-multipath environments.

- The ATG UEs are likely to use single-layer operation on LoS-dominated, lower multipath environments.

- Includes single antenna port-related requirements for ATG UE.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The ATG UEs are likely to use single-layer operation on LoS-dominated, lower multipath environments.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	Includes single antenna port-related requirements for ATG UE.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502952, WF on UE RF requirements for ATG.

- TS 38.306 v18.3.0

- R4-2415897, ATG UE RF requirements for UL MIMO, LG Electronics.