3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504043

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.23.1

Source:	Ericsson

Title:	Discussion on MPR impact on SBFD UL resource muting

Document for:	Discussion

# Introduction

During the last RAN1 #119 meeting, an LS on the impact on transmit signal quality/MPR requirement was triggered to RAN4 [1].

The agreements concerning the UL resource muting are captured below:

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

This following session will analyze the LS and potential impact on the transmit signal quality/MPR for the SBFD UL resource muting.

# Discussion

Based on the RAN1 technical report, the following options for UL resource muting are studied to enhance gNB-to-gNB co-channel CLI measurement and/or channel measurement.

-	Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource)

-	Option 2: Non-transparent UL resource muting method (e.g., define UL resource muting pattern with one or more RE/RB muting patterns)

For option 2, A comb offset {0, 1} is configured for up to two UL muting symbols for CP-OFDM, which means that resources are muted in an alternative pattern of up to two uplink symbols. It is illustrated in Figure 1.

Figure 1. An example combo offset {0, 1} configuration for UL PUSCH muting with two muting symbols.

Only up to two symbols out of 14 were applied, and the alternating pattern distributes power evenly in the muting symbol. The physical RB locations remain unchanged. MPR is primarily tied to the frequency-domain RB allocation, e.g., edge versus inner RBs, and other factors, such as power classes, modulation and coding scheme, channel bandwidth, etc.

Another agreement was that “3 dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.” It means there is a 3 dB power boost on the active REs in muted symbols, the total transmitted power and average power remain the same.

The impact can be assessed by comparing the transmitting power and calculating the PAPR for the symbols with and without UL resource muting. For simplicity, QPSK with PA clipping is used, and PRB allocation is 16, each with 12 subcarriers. The SCS is 15 kHz.

The comparisons in transmitted signal and PAPR between muting and without muting are shown in Figure 2 and Figure 3. Boosting the active subcarriers by 3 dB means each active subcarrier has double the power. However, in the time domain, the amplitude is determined by the sum of the IFFT of the subcarriers. If the active subcarriers are spaced out with comb-2 pattern, their contributions in the time domain might not add up as coherently as when all subcarriers are active. It means with fewer active subcarriers, even though each is boosted, the likelihood of all of them adding up in phase to create a high peak is reduced. This could result in lower peak amplitudes, hence lower PAPR. However, as long as the UE’s PA can handle this without introducing non-linearities, MPR remains unaffected.

Figure 2. Signal envelope for the first 14 symbols.

Figure 3. Simulated PAPR per symbol w and w/o UL resource muting.

- MPR remains unaffected by the introduction of UL resource muting.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	MPR remains unaffected by the introduction of UL resource muting.

# References

- R1-2410889, LS on impact on transmit signal quality/MPR requirement