# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504057

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.12.1.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on Rel-19 ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#114, UE RF requirements for Rel-19 ATG enhancement were discussed, and the agreements are captured in WF [1]. In this paper, we provide our further views on remaining issues.

# 2. Discussion

## 2.1 Inter-band CA

The assumed parameters when supporting n39 UL for CA_n3A-n39A were agreed in [1].

Agreement: - Companies are encouraged to provide MSD values based on the following parameters. Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; [35dB]. n39 Tx filter rejection at 1805~1830MHz; [50dB]. - Above parameters are only applicable for ATG UE with dedicated filter design

With about assumptions, the MSD values can be calculated with different output power as following table:

Table 1: CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | 5.8 |
| 24dBm | 6.3 |
| 25dBm | 6.8 |
| 26dBm | 7.3 |
| 27dBm | 8.0 |
| 28dBm | 8.6 |
| 29dBm | 5.8 |
| 30dBm | 10.1 |
| 31dBm | 11.0 |
| 32dBm | 11.8 |
| 33dBm | 12.8 |
| 34dBm | 13.7 |
| 35dBm | 14.7 |
| 36dBm | 15.7 |
| 37dBm | 16.8 |
| 38dBm | 17.9 |
| 39dBm | 19.0 |
| 40dBm | 20.2 |

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | >ACLR2 |

Proposal 1: RAN4 to take the MSD values in Table 1 to define the MSD requirements for CA_n3-n39 with band n39 UL MSD requirements for different output powers.

Proposal 2: RAN4 should only define one set of MSD requirements with common assumptions.

Another issue discussed in RAN4#114 is whether preclude Case 4 and the following agreements was reached.

Agreement: - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

The UE antenna type is highly depending on the band combination (e.g., whether two bands are close to each other) and the implementation (e.g., different vendors might have different antenna design even for the same band or band combination). Therefore, it’s not possible to conclude a unified antenna design for the UE antenna type for the specific band combination from RF perspective. Based on our understanding, RRM session are discussing the MRTD requirements with different cases on the UE antenna type. Since it is hard to conclude UE antenna type for inter-band CA from RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

Proposal 3: From RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

Proposal 4: There is no need to further discuss ATG UE antenna type in RAN4 RF session which is highly depending on UE implementation.

# 3.	Conclusion

In this paper, we provide our views on UE RF requirements in ATG enhancements. The following observations and proposals are made:

Proposal 1: RAN4 to take the MSD values in Table 1 to define the MSD requirements for CA_n3-n39 with band n39 UL MSD requirements for different output powers.

Proposal 2: RAN4 should only define one set of MSD requirements with common assumptions.

Proposal 3: From RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

Proposal 4: There is no need to further discuss ATG UE antenna type in RAN4 RF session which is highly depending on UE implementation.

# 4.	References

- R4-2502952, WF on UE RF requirements for ATG, CMCC

- TS 38.101-1 v18.6.0