3GPP TSG-RAN WG4 Meeting #114bis		                R4-2504064

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	Consideration on the OTA test of ambient IoT device 1

Agenda item:	7.21.3.2

Document for:	Approval

# Background

The OTA test of ambient IoT (AIoT)device type 1 was initially discussed in RAN#114 [1] and some preliminary agreements were made [2]. In this contribution, we highlight the key issues that we think are critical to be studied for OTA test setup.

# Basic test setup

As a basic consideration, whether to test the tag under free space needs to be determined. In real life, the tag is likely to be installed on surface of a container with metal or other materials. However, such a condition is also highly related with the core requirement discussion. Without further delaying the test setup discussion, it is proposed that RAN4 can take the free space test as starting point to define the OTA test setup for AIoT device 1 in Rel-19.

Proposal 1: RAN4 can take the free space test as starting point to define the OTA test setup for AIoT device 1 in Rel-19.

With such free space test assumption, the basic test setup of AIoT device type 1 can be illustrated as in Fig. 1. In this case the size of quite zone (QZ) of 30 cm which is used for current FR1 OTA test setup can be re-used for AIoT testing considering the size of the tag is not likely to be larger than a smart phone. If other test condition than free space would be required later, RAN4 can revisit this QZ zone.

Proposal 2: Define the QZ size of AIoT device type 1 as 30 cm (same as FR1 TRP/TIS test) as starting point, and RAN4 can revisit this QZ zone size if other test condition than free space would be required.

Similarly, the minimum range between the range between the CW antenna and the centre of QZ (d1) and also the minimum range between the test antenna and the centre of QZ (d2) can be reused from the legacy TRP/TIS test setup, which guarantee the planewave condition inside the quite zone.

Proposal 3: re-sue the same minimum range as legacy TRP/TIS test for the test antenna and also the CW/energy source antenna in AIoT device type 1 in Rel-19.

Another key consideration for the OTA test setup is that whether 3D scanning would be needed to perform. Although this is highly related with the core requirement parameter discussion (e.g., if we test EIRP or TRP), without further delaying the OTA test development, it is proposed to take the 3D scanning based test approach as baseline to develop the OTA test method for AIoT device type 1 in Rel-19, which is more general and can meet the demand of testing different performance metric.

Proposal 4: take 3D scanning test approach as baseline to develop the OTA test method for AIoT device type 1 in Rel-19.

With 3D scanning measurement assumption, a measurement grid also needs to be determined. The original measurement grid was defined up to 30° per step size but it was extended to 45° in later release. Consideration the radiation pattern of the AIoT tag is likely to be a dipole like pattern without strong directivity, it is proposed to consider 45° measurement grid to reduce the test time and cost.

Proposal 5: consideration the radiation pattern of AIoT tag is dipole like without strong directivity, it is proposed to adopt 45° step size on measurement grid to reduce the test time and cost.

Then, the follow up question is on the scanning scheme of CW antenna and testing antenna. Ideally, one would like to measure the tag performance under all the possible combination of CW angle and reader angle (a full bi-static RCS measurement), but such a test is clearly too time consuming.

Therefore, to reduce the scanning sets that the TE needs to perform, it needs to be determined whether both testing antenna and CW antenna shall scan together around the DUT or only testing antenna needs scan the DUT. The advantage of former solution is that isolation between the testing antenna and CW antenna are stable over the whole measurement. However, in this case, for each scanning, it can only test DUT performance under a given angle offset between CW antenna and testing antenna, which may not reflect the real-life scenario well. The corresponding solution here is to repeat the measurement multiple times with different offset angles.

An alternative solution is to fix the CW antenna but scan the testing antenna only, which can naturally create a test condition that CW and reader are located under different angle offsets. However, the issue of this test approach is that the cross talk between the CW antenna and testing antenna will change along the scanning and this aspect needs to be carefully studied. A maximum coupling level between the CW antenna and testing antenna needs to be defined as well.

Proposal 6: RAN4 adopt one of the options as baseline to develop the OTA test method for AIoT device in Rel-19:

- Option 1: Both testing antenna and CW antenna has a fixed angel offset and scan together around the DUT

- Option 2: The position of CW antenna is fixed but testing antenna is scanning around the DUT

Proposal 7: The maximum allowed coupling level between CW antenna and testing antenna needs to be defined.

Figure. 1. OTA test chamber setup

# Other aspect of OTA test

## Impact of RF energy source and CW

the power density of the CW and the power of the energy source that exist within the imagined quiet zone (QZ). To trigger the device during the test, the CW power level that impinged on the device needs to be above certain threshold. Therefore, a power density level of CW signal and the energy source needs to be defined in OTA test.

Proposal 8: the power density of CW signal and energy source needs to be defined for the OTA test of AIoT device 1.

## Impact of energy harvesting

Another major difference between AIoT devices and legacy UEs are the device behaviour due to the energy harvesting. In [2], it is assumed that the duration of one device's unavailability due to charging by energy harvesting can be assumed to be up to several tens of seconds, which was endorsed in RAN#103.

Observation 1: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds.

It is worth highlighting that this is an entirely different situation than for normal UEs in a test. Regular IoT devices or normal smartphones can operate over hours in conformance tests when intensive transmission and reception are performed. In addition, it is known that the conformance test can be very time-consuming, especially for OTA tests, as each test may last tens of minutes or even over an hour since the probe needs to scan a full space over the device. If an AIoT device were tested similarly, the test time may explode, considering that the device would be unavailable from time to time.

Observation 2: The transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

As an example, we illustrate in Fig. 2 how the energy harvesting would affect the test procedure, where it assumes the device needs to charge before each measurement, in this case, the TE needs to understand when to start to initiate the connection with device, and when to start performance measurement. Therefore, the different time period illustrate in Fig. 1 may need to be declared to the TE during the conformance test. Such an impact is particularly critical if multiple points need to be measured.

Figure. 2. Impact on energy harvest on the device test procedure.

Proposal 9:  RAN4 needs to confirm if any information related to energy harvest needs to be specified or declared during the OTA test.

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been made:

Observation 1: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds.

Observation 2: The transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 1: RAN4 can take the free space test as starting point to define the OTA test setup for AIoT device 1 in Rel-19.

Proposal 2: Define the QZ size of AIoT device type 1 as 30 cm (same as FR1 TRP/TIS test) as starting point, and RAN4 can revisit this QZ zone size if other test condition than free space would be required.

Proposal 3: re-sue the same minimum range as legacy TRP/TIS test for the test antenna and also the CW/energy source antenna in AIoT device type 1 in Rel-19.

Proposal 4: take 3D scanning test approach as baseline to develop the OTA test method for AIoT device type 1 in Rel-19.

Proposal 5: consideration the radiation pattern of AIoT tag is dipole like without strong directivity, it is proposed to adopt 45° step size on measurement grid to reduce the test time and cost.

Proposal 6: RAN4 adopt one of the options as baseline to develop the OTA test method for AIoT device in Rel-19:

- Option 1: Both testing antenna and CW antenna has a fixed angel offset and scan together around the DUT

- Option 2: The position of CW antenna is fixed but testing antenna is scanning around the DUT

Proposal 7: The maximum allowed coupling level between CW antenna and testing antenna needs to be defined.

Proposal 8: the power density of CW signal and energy source needs to be defined for the OTA test of AIoT device 1.

Proposal 9:  RAN4 needs to confirm if any information related to energy harvest needs to be specified or declared during the OTA test.

# References

- R4-2500686 	Topic summary for [114][134] A-IoT_device​ Moderator	(CMCC)

- R4-2502858 	WF on requirements for A-IoT device 	CMCC

- RP-240116		View on Ambient IoT scope 	Qualcomm.