# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2504065

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	7.26.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were discussed in RAN4 #113 [1], a WF was agreed [2], and a LS was sent to RAN1 to check the detail of wake up sequence design to facilitate the REFSENS discussion [3].

In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- Type of LP-WU receiver

In RAN4#114 meeting, the following agreements for LP-WUS/WUR have been reached for deriving REFSENS:

- Set 1: LP-WUR IM+NF = 18.0 dB

- Set 2: LP-WUR IM+NF = 13.5 dB

However, the mapping between different sets and different receiver types is still under RAN4 discussion. Per our understanding, the two sets of NF are based on OOK type receiver and OFDM type receiver, respectively. Pending the RAN1 design, it is proposed to consider mapping the different Rx requirements to a receiver that can support OOK waveform, and the receiver can support overlaid OFDM waveform.

Proposal 1: If multiple sets of requirements are specified in the end, RAN4 shall strive to associate the Rx requirements with the receiver that can only decode OOK waveform, and the receiver can decode OFDM waveform if it is in line with the different receiver types defined in RAN1.

- REFSENS for LP-WUR

## REFSENS requirement for OOK receiver

Based on our simulation results, the target SNR can reach about -7 dB for 1% MDR and -3 dB for 1% BER under the AWGN channel (see appendix). Based on our design, we have about 4dB processing gain (1% MDR vs. 1% BER). Considering that the different receiver implementations may present different processing gains, an SNR value between -7 and -3 dB can be considered.

Observation 1	 Based on our sequence and receiver design, the target SNR is about -7 dB for 1% MDR and -3 dB for 1% BER.

Proposal 2	RAN4 adopts a target SNR value in the range between -7 dB to -3 dB for LP-WUS.

With this SNR assumption and agreed NF+IM from the last meeting, the REFSENS of OOK LP-WUS for 11 RB with 30 kHz can be calculated as below:

REFSENS=-174 dBm+10*log3.96MHz+18 NF+IM+-3SNR=-93 dBm

Observation 2	 With the proposed SNR, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -93 dBm.

## REFSENS requirement for OFDM receiver

Different to the OOK receiver, the OFDM receiver can decode the overlaid OFDM sequence. Therefore, from this aspect, the LP-WUR works very similarly to the main receiver (MR). Although the performance metric is different to the legacy receiver, the SNR of -1 dB on the legacy receiver can be used as a starting point.

Proposal 3 	The -1 dB SNR can be taken as a starting point for OFDM receiver.

With this SNR assumption and agreed NF+IM from the last meeting, the REFSENS of OFDM LP-WUS for 11 RB with 30 kHz can be calculated as below:

REFSENS=-174 dBm+10*log3.96MHz+13.5 NF+IM+-1SNR=-95.5 dBm

Observation 3	 With the proposed SNR, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -95.5 dBm.

- ACS and ASCS Requirement

## Requirement for ACS and ASCS

For ACS requirements, the following agreements have been made in RAN4#114:

- Same interference signal level as MR for LP-WUR ACS requirements

  - FFS at least for one set of Rx requirements.

- The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point

  - FFS whether two sets of LP-WUS wanted signal level can be defined.

- Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation

- FFS number of guard RBs.

From the network co-existence aspect, regardless the receiver type, the LP-WUS is expected to co-exist with other signals and suffer the same level of interference as a normal NR receiver. Thus, setting the same interference signal level for the LP-WUR as MR would be reasonable for the ACS requirement.

Proposal 4	Define the same interference signal level as MR for LP-WUR for ACS requirements for all LP-WUS receiver type.

The OFDM-based receiver is expected to operate similarly to the main receiver, and the interference cancellation capability are also expected to be similar. Therefore, the same wanted signal level as legacy, e.g., REFSENS+ 14 dB can be reused.

Proposal 5	 Same wanted signal level with REFSENS + 14 dB can be specified for OFDM-based LP-WUS receiver.

Meanwhile, it is understood that a more aggressive power-saving gain is targeted for OOK-based LP-WUR, and the capability of LP-WUR to tolerate interference could be worse than that of MR. Therefore, it is possible to specify a higher wanted signal level in the core requirements to balance the power consumption and the interference mitigation for the LP-WUR.

Proposal 6	 A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

For the ASCS requirement, based on our simulation results (see appendix), the number of guard RBs does not significantly affect ACSC performance. Therefore, it is proposed that 0 RB be adopted for ACSC test cases.

Proposal 7 	RAN4 adopts 0 guard RB for ASCS test case with 0 dB desens on LP_WUR.

- Testability of LP-WUS

From the testability aspect, different from the legacy REFSENS or ACS test for the MR, there will be no ACK/NACK for the LP-WUS receiver, and therefore, how the test equipment (TE) shall be able to count the missed detection rate is unclear.

It is clear that if a full cycle of wake-up and connection needs to be performed (wake-uppaging random access  connected mode then go back to Idle), the test time will be excessive.

Observation 4	 The test time may be excessive if a full cycle of wake-up and connection needs to be performed.

As an alternative, 3GPP may also investigate whether succeeding transmissions (e.g., PRACH) on the main radio can be detected by the TE once the UE has been woken up. For the sake of test time, it would be preferred that no connection mode needs to be created. However, it would be a question of how the UE LP-WUR would behave if there is no response from the TE on the MR after a few attempts.

Observation 5	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

One possible solution is to implement a dedicated test mode to allow the UE to record whenever it detects an intended wake-up sequence and report this value to TE once the test is done.

Observation 6	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

But in general, we think the details of the test method can be left to RAN5 to decide, while RAN4 can focus on the issue of what needs to be tested.

Proposal 8 	The detail of the test method can be left to RAN5, while RAN4 focuses on what needs to be tested.

In this aspect, it is possible for a UE to support the LP-WUS in both idle mode and connected mode. For the RF requirement, it is sufficient to test one of the modes in our view as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both modes.

Proposal 9  	For RF requirement, it is sufficient only to verify the UE performance under one state (e.g., either Idle or connected) as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both Idle and Connected modes.

As idle mode is the most important use case, it is proposed that the test be taken under idle mode as a baseline for further discussion.

Proposal 10  	Test under IDLE mode can be taken as a baseline since this is likely to be the most important use cases for Rel-19 LP-WUS.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1	 Based on our sequence and receiver design, the target SNR is about -7 dB for 1% MDR and -3 dB for 1% BER.

Observation 2	 With the proposed SNR, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -93 dBm.

Observation 3	 With the proposed SNR, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -95.5 dBm.

Observation 4	 The test time may be excessive if a full cycle of wake-up and connection needs to be performed.

Observation 5	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

Observation 6	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

Proposal 1: If multiple sets of requirements are specified in the end, RAN4 shall strive to associate the Rx requirements with the receiver that can only decode OOK waveform, and the receiver can decode OFDM waveform if it is in line with the different receiver types defined in RAN1.

Proposal 2	RAN4 adopts a target SNR value in the range between -7 dB to -3 dB for LP-WUS.

Proposal 3 	The -1 dB SNR can be taken as a starting point for OFDM receiver.

Proposal 4	Define the same interference signal level as MR for LP-WUR for ACS requirements for all LP-WUS receiver type.

Proposal 5	 Same wanted signal level with REFSENS + 14 dB can be specified for OFDM-based LP-WUS receiver.

Proposal 6	 A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Proposal 7 	RAN4 adopts 0 guard RB for ASCS test case with 0 dB desens on LP_WUR.

Proposal 8 	The detail of the test method can be left to RAN5, while RAN4 focuses on what needs to be tested.

Proposal 9  	For RF requirement, it is sufficient only to verify the UE performance under one state (e.g., either Idle or connected) as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both Idle and Connected modes.

Proposal 10  	Test under IDLE mode can be taken as a baseline since this is likely to be the most important use cases for Rel-19 LP-WUS.

- References

R4-2500688		Topic summary for [114][136] NR_LPWUS_UERF	Moderator(Vivo)

R4-2503028 		WF on UE RF requirements for LP-WUS​ 	Vivo

R4-2503003 		LS on LP-WUS UE RF​ 	Vivo

- Appendix LLS

In this section, we present initial simulation results for ASCS analysis with OOK-4 waveform. Our simulations are based on the simulation assumption for link level simulation (LLS) agreed during RAN4#111, shown below:

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Chip rate | M=4 |
| WUS duration | 126 bits ~ 1mSec |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0 |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 |
| WUS RB | - Fixed 11RB ~ 3.96MHz |
| Position within channel | - Center of the channel |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth |
| Filter | - 5th Order Butterworth matching fixed 3.96MHz RF bandwidth |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same average PSD with WUS signal |
| Sampling rate | 7.68MHz |
| ADC bit width | Ideal ADC |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Power boosting | No power boosting |
| Channel Model | TDL-C 300 and AWGN |
| Performance metric | 1% MDR with false alarm rate below 1% |

In our simulation, LP-WUS design has the following characteristics:

- Kasami sequences of equal length of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 1 msec.

- Overlaid OFDM sequence with length 16 consisting of an m-sequence of length 15 and a random single bit. The overlaid OFDM sequence is generated based on option 1-1 agreed in RAN1, i.e., before DFT/LS block.

- The LP-WUS occupies 16 OFDM-symbols, 11RBs (1 RB GB at each side of the LP-WUS bandwidth) with subcarrier spacing of 30 kHz.

We have used ideal ADC but it is expected that the ADC performance would not degrade our MDR performance significantly. The order of lowpass Butterworth is set to 5 and we assume no frequency and timing error.

We study the results, BER and MDR, for AWGN and TDL-C 300 channel for two guard band settings of 0 RB and 1 RB on each side of the LP-WUS bandwidth. The achieved false-alarm rate for the studied scenarios is below 0.1%, thanks to LP-WUS structure, sequence design, and DBB processing.

Fig. A1  Bit error rate (BER) for AWGN (left) and TDLC (right) channel models.

Fig. A2    Miss detection rate (MDR) for AWGN (left) and TDLC (right) channel models.

Based on this initial simulation results, it can be observed that the number of guard RB does not affect the MDR performance significantly. Additionally, the processing gain from the sequence design can compensate for performance loss of the LP-WUR due to its ultra-low power design, allowing to reach the target coverage level even at relatively high BER.