3GPP TSG-RAN WG4 Meeting #114bis	R4-2504072

Wuhan, China, April 7th – April 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for LTM event-triggered L1 requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The draftCR captures requirements for event-triggered L1 reporting for Rel-19 LTM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update applicability of: - Cell switch delay requirements in 6.3 and 8.20 - TCI state activation delay requirements in 8.25 Introduce event-triggered and event-triggered periodic reporting requirements for: - Intra-frequency measurements in section 9.14.3 - Inter-frequency measurements in section 9.15.3 Note: The changes from RAN4#114 meeting that are not yet included in the specification are shown with change marks in this draft CR. New changes are introduced as “Nokia”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Event-triggered L1 measurement requirements are not included in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 8.20.1, 8.25.1 and new clauses 9.14.3.x1, 9.14.3.x2, 9.15.3.x1, 9.15.3.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

## 6.3	L1/L2-Triggered Mobility

### 6.3.1	LTM PCell Cell Switch

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the UE has sent an event-triggered L1 measurement report with SSB associated to the target TCI state and the condition for triggering the measurement report remains fulfilled at the time of receiving the cell switch command, and

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

Change 2

## 8.20	LTM PSCell Cell Switch

### 8.20.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this clause are applicable to LTM PSCell switch.

The requirements in this clause are applicable to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to NR-DC for the following scenarios:

PSCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PSCell switch to an LTM candidate cell that is a serving SCell in SCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM TCI state activation command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the UE has sent an event-triggered L1 measurement report with SSB associated to the target TCI state and the condition for triggering the measurement report remains fulfilled at the time of receiving the cell switch command, and

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The definition of known LTM target cell and the definition of known target joint DL/UL TCI state or separate DL and UL TCI states are as stated in clause 6.3.1.2.

Change 3

## 8.25	TCI state activation for LTM candidate cell

### 8.25.1	Introduction

The requirements in this clause apply for a UE configured with one or more joint or separate uplink/downlink TCI state configurations on LTM candidate cell. UE shall complete the activation of the target joint or separate uplink/downlink TCI states within the delay defined in this clause after receiving a MAC CE indicating LTM candidate cell TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321 [7].

The requirements in this clause apply for

-	Known downlink TCI state in FR1 and FR2 specified in clause 8.25.2, and

-	Unknown downlink TCI state in FR1 specified in clause 8.25.2, provided the following conditions are met:

-	UE has reported L3 measurement result with the associated SSB index of the downlink TCI state within 5 seconds before the LTM TCI state activation command, or

-	the UE has sent an event-triggered L1 measurement report with SSB associated to the target TCI state and the condition for triggering the measurement report remains fulfilled at the time of receiving the cell switch command.

-	SNR of the associated SSB is above -3 dB.

-	Unknown downlink TCI state in FR2 specified in clause 8.25.2, provided the following conditions are met:

-	the UE has sent an event-triggered L1 measurement report with SSB associated to the target TCI state and the condition for triggering the measurement report remains fulfilled at the time of receiving the cell switch command.

Change 4

#### 9.14.3.x1	Event-triggered reporting

Reported L1-RSRP measurements contained in event-triggered L1-RSRP reports shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2.

The UE shall not send any event triggered L1-RSRP measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time period between an event that will trigger the measurement report and the time when the UE starts to transmit the measurement report over the air interface.

The event triggered measurement reporting delay shall be less than

-	TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.14.5, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

-	TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.14.5, otherwise,

where Treport = 0 and TSSB_NBC is the periodicity of the SSB that triggers the report.

#### 9.14.3.x2	Event-triggered periodic reporting

Reported L1-RSRP measurements contained in event-triggered L1-RSRP reports shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14.3.x1.

Change 5

#### 9.15.3.x1	Event-triggered reporting

Reported L1-RSRP measurements contained in event-triggered L1-RSRP reports shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2.

The UE shall not send any event triggered L1-RSRP measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time period between an event that will trigger a measurement report and the time when the UE starts to transmit the measurement report over the air interface.

The event triggered measurement reporting delay shall be less than

-	TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.15.5 or 9.15.6, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

-	TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined in clause 9.15.5 or 9.15.6, otherwise,

where Treport = 0 and TSSB_NBC is the periodicity of the SSB that triggers the report.

#### 9.15.3.x2	Event-triggered periodic reporting

Reported L1-RSRP measurements contained in event-triggered L1-RSRP reports shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2, respectively.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.15.3.x1.

End of changes