3GPP TSG RAN WG4 Meeting #114bis		 R4-2504101

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.7.2

Source: 	MediaTek Inc.

Title: 	Discussion on low-band CA switching

Document for:	Approval

1. Introduction

The low band carrier aggregation has been discussed and WF was agreed in last RAN4 meeting[3]. The contribution provide continue discussion on the low-band CA with the new carrier switching feature.

2. Discussion

Issue 1: Scope clarification

In last RAN4 meeting, WF of scope clarification was agreed and copied below:

-	Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI

-	It is not precluded that such two low bands are further combined with the additional band

-	The combination of those two low bands with other band can be further discussed in the future release

From the above agreement, the UE capability, if specified, shall consider not only the switching of the two-low-bands, but also when the two-low-band pair is fallback as part of higher order configuration. As stated in the note1 of WI scope “additional band combinations to be handled via the basket work item approach”, the low band pair applicable for LB CA switching capability shall be clearly indicated in the UE capability.

Observation 1: From the agreed WF, the LB CA switching can be further combined with additional band. The low band pair applicable for LB CA switching capability need to be clearly indicated in the UE capability.

Issue 3: Switching period capability

The issue was left open for further discussion. Since it was agreed RAN4 can consider the two bands can be further combined with the additional band, the UE capability and signalling design shall indicate the details such as which of the two bands pair can support switching capability. For example, CA_n5-n29 is further combined with n12 while the CA_n12-n29 does not support LB switching capability, the signalling design shall be able to indicate the difference clearly. With above consideration, it may not be proper if the capability is simply per band combination reporting. RAN4 may need consider per band pair per BC reporting for the UE capability. Furthermore, even if the CA_n12-n29 can also support LB switching capability, to simplify RRC configuration procedure and RAN1 switching pattern design workload, there should be only one low-band pair operating switching at a time.

Proposal 1: The UE capability and signalling design need to consider when the two low band is fallback of higher order combination. Per band pair per BC is proposed for the UE capability \

Proposal 2: If there are more than one low-band-pair reporting support of the switching capability, there shall be only one low-band pair operating LB CA switching at a time.

And since this is new feature start from release-19. It is expected new UE capability and signalling are also introduced. If the release independence manner is considered, to accommodate NBC issue, it is better to begin with release-19 only.

Proposal 3: If release independence manner is considered, it is proposed release independent starts from release-19

Issue 6: UE reference architecture

First of all, we would assume the reference RF front-end architecture would consider sharing low-band antenna (referring to [2]) as illustrated below as baseline:

Figure 1. RF antenna and front-end architecture assumption

Though it does not preclude that UE can implement with luxury hardware capability with more DL Rx paths and triplexer for the example band combination since there’s large enough Tx-Rx freq gap that makes triplexer feasible. There are still some cases, such as SDL_n12-n29, that don’t have enough T/R freq gap thus triplexer would not be feasible. So the architecture in Figure 1 can be the baseline to specify the minimum requirements.

Proposal 4: The below front-end architecture is proposed for specifying the minimum requirements for low band DL switching:

Issue 4: Time mask for switching

In last RAN4 meeting, time mask for switching is still open for discussion. The options in the WF are copied:

-	Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)

-	Option 2: Switching period is always within the switch-from carrier

-	Option 3: Switching period is contained within the switching gap configured by the network

-	Other options are not precluded

As discussed in [4] in last RAN4 meeting, we support option 2 with illustration below and timing advance (TA) shall be considered additionally on top of the switching period

Proposal 5: We support option 2: Switching period is always within the switch-from carrier. Timing advance (TA) shall be considered additionally on top of the switching period

Figure 2. Time mask for the LB CA switching

Issue 5: Switching period location

Two options in the agreed WF[3]:

-	Option 1: Switching period location is entirely up to RAN1 design

-	Option 2: Switching period location is defined relative to the slot boundary

-	Other options are not precluded

As our discussion in [4], and in the WI scope stated switching pattern shall be based on RRC configuration, we propose the switching pattern is slot-based. We support option 2

Proposal 6: We support option 2: Switching period location is defined relative to the slot boundary with below sub-bullet

- Switching period always locate at the end of “switch-from” carrier to avoid impact on the PDCCH.

3. Conclusion

Issue 3: Switching period capability

Proposal 1: The UE capability and signalling design need to consider when the two low band is fallback of higher order combination. Per band pair per BC is proposed for the UE capability \

Proposal 2: If there are more than one low-band-pair reporting support of the switching capability, there shall be only one low-band pair operating LB CA switching at a time.

Proposal 3: If release independence manner is considered, it is proposed release independent starts from release-19

Issue 6: UE reference architecture

Proposal 4: The below front-end architecture is proposed for specifying the minimum requirements for low band DL switching:

Issue 4: Time mask for switching

Proposal 5: We support option 2: Switching period is always within the switch-from carrier. Timing advance (TA) shall be considered additionally on top of the switching period

Issue 5: Switching period location

Proposal 6: We support option 2: Switching period location is defined relative to the slot boundary with below sub-bullet

- Switching period always locate at the end of “switch-from” carrier to avoid impact on the PDCCH.

Reference:

- RP-243315 New WID on low band carrier aggregation via switching, RANP#106

- RP-241537 Rx switched nX-n29 low-low carrier aggregation, RANP#104

- R4-2503016 WF on low-low band switching, RAN4#114

- R4-2501121 Discussion on low-band CA switching, RAN4#114