3GPP TSG-RAN WG4 Meeting #114bis	R4-2504107

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on RRM core requirements and test framework for beam management

Agenda Item:	7.19.3

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #114, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

Issue 2-1: Metrics/KPIs for beam prediction

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Proposals - Option 1: - In case of the AI/ML model with label (beam ID) output type - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted. - In case of the AI/ML model with label (beam ID) + RSRP output type - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously. - Option 2: Option 2 with a combination of Top-K/1 and Top 1/K - Option 2a: update option 2 to Top-N/K, the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". - Option 3: Option 3 as it does not have the issue of UE failing if there are multiple beams within a small RSRP margin - Option 4: combinations of options, concrete proposals should be disucssed - Recommended WF - To be discussed The number of options should be downselected to make some progress or a clear path to choose an option should be discussed/agreed

If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

The inclusion of the parameter N does not present compelling justification. While incorporating the top-N predicted beams may enhance the robustness of beam prediction beyond the top-1 beam in certain scenarios, that said, we shall think about it from implementation perspective. Even the UE provides the top-N predicted beams all satisfying a certain threshold, up to 4 beams in legacy reporting, the NW may only choose the top-1 prediction by default.

Regarding Option 3 ‘The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB.’, in our opinion, Specifically, Option 3, which may be as the complement of Option 2, ensures that the ground truth RSRPs of the top-K predicted beams remain close to the ground truth RSRP of the strongest beam, thereby preventing significant deviation between the predicted beam and the actual received RSRP. This approach enhances the reliability of beam prediction while maintaining a balance between accuracy and practical implementation constraints.

However, Option 3 presents a potential issue in testing, particularly from a conformance testing perspective. The term “successful rate” is inherently ambiguous, as it suggests that a certain number of failures may be permissible within the test. This raises concerns regarding the evaluation criteria for compliance, as it remains unclear how the success rate should be measured and whether it can be converted into a mandatory requirement that must be strictly satisfied. Therefore, further investigation is needed to establish a well-defined framework or to reformulate Option 3 into a clear and enforceable requirement for conformance testing.

We consider the metrics/KPIs including the options as follow:

- In the case of the AI/ML model with label (beam ID) output type, the metrics/KPIs for beam management requirements/tests are derived by the beam prediction accuracy, i.e., the UE shall only fulfil beam prediction accuracy, i.e., Option 2 + Option 3.

- In the case of the AI/ML model with label (beam ID) + RSRP output type, the Metrics/KPIs for beam management requirements/tests shall take both of the beam prediction accuracy and RSRP accuracy into account, i.e., the UE shall fulfil beam prediction accuracy and RSRP accuracy simultaneously, i.e., Option 1 + Option 2 + Option 3.

Proposal 1: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

Observation 1: RSRP in Option3 of Beam prediction KPIs is groundtruth RSRP, i.e., the maximum RSRP among top-K predicted beams and the RSRP of the strongest beam are groundtruth RSRP.

Proposal 2: In Option2 of Beam prediction KPIs, RAN4 considers the below options.

- Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

- Not introduce the parameter N, since K equals to or is less than 4 typically.

As a reference, it’s observed that regarding performance monitoring, RAN1 is discussing the performance metric of Top 1 or Top K beam prediction accuracy, which is defined as: at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

- Where K is the number of predicted beam(s) in the corresponding inference report.

- Where Top M beam(s) are the beam(s) within X dB of the beam with largest measured value of L1-RSRP of the resource set(s) for monitoring

Compare the RAN1’s performance monitoring metrics and Option 3, the difference is:

- RAN1 defines X dB, then sorts out the Top-M beams and checks if there is at least one beam within Top-M beams and at same time within Top-K predicted beams.

- Option 3 in RAN4 defines the maximum RSRP among Top-K predicted beams fulfilling a RSRP threshold in essence.

If we assume M=1 and the monitoring RS(s) is identical to the predicted RS(s) in RAN1’s performance monitoring metrics, then apparently, RAN1’s performance monitoring metrics and Option 3 are equivalent (expect for the RSRP acquisition approach). Since we suggest that Option2 shall prioritize Top-K/1 rather than Top-K/N, our standing on Option 3 is to maintain the current wording: "the maximum RSRP among the top-K predicted beams is greater than the RSRP of the strongest beam minus x dB." In other words, it is unnecessary to verify whether more than one beam within the Top-K predicted beams exceeds the RSRP of the strongest beam by x dB.

Observation 2: The term ‘successful rate’ in Option 3 of Beam prediction KPIs shall be removed, as Option3 represents a mandatory requirement that must be strictly satisfied.

Proposal 3: Maintain the current wording of Option 3 of Beam prediction KPIs without modification, as no further improvements are necessary.

Issue 2-2: Relative RSRP accuracy

Relative RSRP accuracy Relative RSRP accuracy has been discussed for a few meetings without any clear agreement - Proposals - Option 1: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 2: Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n , where the beam index n owns the largest value - Option 3: Relative RSRP is the RSRP difference between the predicted RSRP values, FFS what the accuracy should be - Option 4: Others - Recommended WF - To be discussed

Firstly, recap the legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 of TS38.133, which is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The relative accuracy requirement is designed for the comparison between multiple RSs, relative accuracy ensures that any measurement error in a single RSs is less impactful. In this way, satisfying the relative accuracy requirement essentially reduces the likelihood of measurement inconsistencies across SSBs. Hence, for serving the same purpose, the reference L1-RSRP of relative accuracy shall be assessed in prediction area.

In legacy, there is no doubt that the reference L1-RSRP is the largest measured value of L1-RSRP. Differently, in AI/ML based beam management, a more straightforward approach is to define the reference L1-RSRP as the largest predicted value of L1-RSRP, facilitating a peer-to-peer comparison.

Proposal 4: Support Option 1 of the relative RSRP accuracy, relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Issue 3-1-1: Absolute RSRP accuracy

Agreement in RAN4#112: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

There exists a leftover regarding the absolute RSRP accuracy.

In our opinion, the beams that shall be taken care are the predicted beams. In other words, the prediction (accuracy) shall focus on the beams that the UE reports but not the beams supplied by the genie way.

If there are offsets between the two approaches, it's questionable to adopt the top-K beams w.r.t. the ground truth RSRP. Two instances that reflect our concerns are provided below:

- If the requirement is only relevant to the top-K beams with respect to the ground truth RSRP, does the beam's RSRP have no requirements if it is present in the top-K predicted beams but not in the genie beams?

- Conversely, why should we need the RSRP accuracy of a beam that the UE doesn't even report if it is present in the top-K genie beams but not in the predicted beams?

Proposal 5: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting.

Issue 2-3: Measurement period for inference

Measurement period for inference Requirements for the measurement period are discussed by multiple companies and should be defined as core requirements Issue 2-3: Measurement period for inference - Proposals - Option 1: Use “legacy” L1-RSRP measurement as baseline - FFS whether same values for M,N,P should be reused - Option 2: Differentiate case 1 and case 2 - Reuse L1 measurement period for Case 1 - Introduce configurable number of measurement occasions for case 2 - Option 3: Study the measurement period based on simulations - Option 4: Others - Recommended WF - Option 1

In conventional way, measurement delay normally refers to Clause 9.5.4 of TS38.133, ‘When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.’. Given that, the requirement derives the minimal delay in which the UE shall be able to complete an entire L1-RSRP measurement while satisfying the accuracy.

However, those requirements may be different in the predication case. The timeline concerning measurement and inference phases in prediction in Figure 1. Note: those variants in the figure are just for interpretation demonstration.

Figure 1 Timeline of measurement and inference phase in prediction

It’s noted that BM case 1 and BM case 2 may have different set A and set B configurations. Also, the length of measurement/inference for BM case 1 and BM case 2 are typically different, e.g.,

- m, n for BM case 1 typically equal to 1, i.e., one-shot measurement and one-shot inference.

- On the contrary,  m, n for BM case 2 typically are larger than 1, for tracking beam changes.

- Wherein, m, n is the number of reference signal measurement occasions or predication occasions respectively. Normally, the exact number of m, n shall be configured by the NW or up to UE implementation/capability or predefined in specification.

Regarding measurement delay, which may be defined by one of the below options:

- Delay of single/one shot measurement occasion. Note: it is identical to measurement delay in legacy.

- Delay of all measurement occasions for prediction, including m measurement occasions.

Regarding measurement accuracy, a unified accuracy requirement shall be upheld for each L1-RSRP measurement delay in both BM-case 1 and BM-case 2. In other words, from an accuracy perspective, there is no distinction between BM-case 1 and BM-case 2. Furthermore, no necessity for accuracy enhancement has been identified thus far.

A single-shot measurement occasion shall be applicable to both BM-case 1 and BM-case 2, ensuring that an entire measurement is completed to obtain a qualified RSRP result. Consequently, further restrictions on the accumulated measurement accuracy—averaged across multiple measurement occasions—are unnecessary, provided that the accuracy of each individual measurement occasion is guaranteed as today.

In our opinion, multiple measurement occasions for BM case 2, typically are not intended to obtain an accumulated higher measurement accuracy. Instead, they mainly target facilitating beam characteristic analysis, which is typically a proprietary process managed by AI/ML model vendors, such as analyzing UE movements.

Proposal 6: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied, with the assumption that the measurement accuracy remains consistent.

Proposal 7: RAN4 to study whether specific requirements are necessary to evaluate the capability of a UE in tracking changes in L1-RSRP through measurements in BM-case 2.

Issue 2-6:	RSRP Error modeling in simulations

In this section, we first simulated the LLS based on the simulation assumption in R4-2420480 and obtained the absolute values for L1-RSRP differences between the ideal and practical channel estimation, as listed in Table 1.

Table 1 The L1-RSRP differences of SSB

| SNR (dB) | L1-RSRP differences (dB) |
| --- | --- |
| -15 | 2.58 |
| -12 | 1.48 |
| -9 | 0.87 |
| -6 | 0.55 |
| -3 | 0.38 |
| 0 | 0.26 |
| 3 | 0.18 |
| 6 | 0.13 |
| 9 | 0.09 |

From the table we can see that the RSRP differences can be up to 2.5 dB with a worse channel condition, such as SNR = -15 dB, but the L1-RSRP differences is around 0.38 dB with the suggested SNR value -3dB, which could have minor impact on the performance metric compared to the +/- 4.5dB RF error on prediction accuracy.

Observation 3: The L1-RSRP differences of SSB with the SNR value -3 is only a few tenths of dBs so its impact would be minor as the baseband (BB) error.

We then generated the BB error at SNR 0 dB with TDL-C channel assumption. Results show that the BB error distribution follows the Gaussian distribution as the below figure shows, with zero mean.

Figure 2 BB error distribution of SSB L1-RSRP with 0 dB SNR

Therefore, we support to model the BB error with Gaussian distribution if the BB error is applied into the measurement error. The sigma value can be further discussed after the BB error distribution is confirmed.

Proposal 8: The distribution of baseband error can be fitted with zero mean Gaussian distributions, the sigma value can be further discussed after the BB error distribution is confirmed.

Issue 2-7: Measurement error impact evaluation

The impact of the measurement errors on the Top-K/1 prediction accuracy is shown in Table 2 when K is 1,3,5. In addition to the parameter settings mentioned in the R4-2420480, 1 dB margin is considered in the beam prediction accuracy. Same RF errors are applied for each Tx.

Table 2 The impact of RF errors on AI/ML prediction accuracy for Top-K/1

|  | Prediction accuracy no measurement error | Prediction accuracy with RF errors +/-4.5dB (Gaussian) | Prediction accuracy with RF errors +/-4.5dB (Uniform) |
| --- | --- | --- | --- |
| Top-1/1 | 94.3 | 85.7 | 79.4 |
| Top-3/1 | 98.3 | 98.2 | 96.7 |
| Top-5/1 | 99.3 | 99 | 98.7 |

From the table, by considering ±4.5 dB RF error with Gaussian distribution and uniform distribution, the beam prediction accuracy degrades ~10% (from 94.3  85.7) and ~18% (from 94.3  79.4) respectively, in terms of Top-1 beam prediction accuracy when comparing to the one without measurement error. While its impact on Top-3 is relatively small and can even be neglectable for Top-5 case.

Observation 4: Both ±4.5 dB RF error with Gaussian distribution and uniform distribution can bring large performance degradation on prediction accuracy especially for Top-1 beam, where the impact of uniform distributed RF error is even worse.

The 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1 predicted beam(s) and the RSRP of the genie aided strongest beam is also evaluated. Results show that ~5.18 dB degradation can be caused by ±4.5 dB RF error with Gaussian distribution. Therefore, RAN4 should further discuss whether the degradation is acceptable or not.

It might be worth to mention that, the 90%-tile L1-RSRP difference mentioned in the R4-240480 is defined as “90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))”. In our understanding, the “95%-tile L1-RSRP” and “5%-tile L1-RSRP” should be the “95%-tile L1-RSRP difference” and “5%-tile L1-RSRP difference”. It is suggested to clarify this metric during the meeting.

Proposal 9: the definition of “90%-tile L1-RSRP difference” should be clarified, such as: 90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP difference), abs (5%-tile L1-RSRP difference)).

Proposal 10: The performance degradation from ±4.5 dB RF error is not neglectable, further discussion is needed for whether accept this performance degradation or not.

Issue: Performance monitoring

Based on RAN1’s agreements, we may focus on Type 1, Option 2 of UE-side model monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN WG1 #118, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN WG1 #118bis, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN WG1 #119, the below agreements were achieved.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Our understanding is that for both BM-Case 1 and BM-Case 2, when utilizing a UE-sided AI/ML model, in the context of Option 2 (UE-assisted performance monitoring), the UE  should be capable of comparing the differences between prediction results and measurement results across a designated resource set or specific resources used for monitoring.

Based on RAN1 agreements to date, we do not identify any fundamental differences between measurement/inference delay for prediction and measurement/inference delay for monitoring, given that the evaluation metric remains the same (e.g., Top-1 or Top-K beam prediction accuracy). Furthermore, no proposals or agreements have indicated a distinction between the two ones. However, the final selection of the metrics may differ, considering their respective objectives and use cases.

Proposal 11: RAN4 shall define measurement delay for monitoring.

Observation 5: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

## Testability and interoperability

Issue 4-1: Baseline test setup

- Proposals - Option 1: Single AoA setup (different Tx beams emulated only with power difference?) - Is it possible to test if UE can also make a prediction when set B beams come from different directions? - How many Tx beams can be emulated when the spatial domain properties are lost? - Will UE need just a simple interpolation model or is a more complex model necessary to pass such a test? - Can additional (L1-)RSRP measurement accuracy requirement (e.g. spherical coverage) be used to complement Single AoA testing with directional/spatial UE performance? - Option 2: Single AoA setup with “two stage approach” - Will a UE which uses Rx beam information in the prediction model have to also implement a model which does not use the Rx beam information? - Are there any implementation constraints on a UE to execute/pass this test? - How will the TE know the UE Rx beam pattern? - Option 3: multi AoA setup– RRM test setup as baseline (including enhancement for up to 4AoA active probes) - How many Tx beams can be emulated with such a setup, what is the relationship between number of probes and number of set B Tx beams? - Can AoAs between the probes also be emulated (is it possible to “make it look like” signals are coming from other angles than where the probes are placed?) - Can a simplified CDL be emulated or only TDL from each probe? - Option 4: multi AoA setup – 3D MPAC system as baseline - How many Tx beams can be emulated with the baseline setup? - 4Tx beams with dual pol(set B?) can be emulated with 2 additional probes Pros&cons of different test setups - Option 5: multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. Single AoA; Single AoA with two stage; Multi AoA – RRM test setup as baseline; Multi AoA 3D MPAC. Tests that UE uses spatial information(UE actually makes measurement in multiple directions to make a prediction); No; Maybe?; Yes; Yes. Relevance for field performance; Low; Maybe?; Good/High; Good/High. Simplified CDL emulation; No; Yes; Yes. Test setup complexity; Low; Low; Medium; Medium/High?. Test method complexity; Low; High; Medium; Medium/High?. Reuse of test setup for other tests; Medium (multi AoA testing already exists in RRM); High?; Low/medium? Can only reuse the test setup for FR2 MIMO OTA?. - Recommended WF - Do not further discuss single AoA test setups, continue to study the two multi AoA candidate setups

Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

Issue 4-2: Channel model

- Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others - Recommended WF - To be discussed If simplified CDL is to be further considered, more detailed proposals on what to study/analyze further should be discussed (e.g. RX PAS, number of relevant clusters to emulate, dependency on number of clusters and number of probes in the chamber, etc)

Agreement: - Use CDL-based channel model as starting point

From AI/ML model testing perspective, it’s clear that there exist two options in last meetings.

- Option 1:  only inputting L1-RSRP of beams, single AoA with CDL/TDL channel can be applied.

- Option 2:  inputting beam information comprising L1-RSRP, Tx beam spatial distribution and Rx beam spatial distribution, multiple AOA with CDL channel shall be applied.

Option1 supposes that by emulating artificial Tx/Rx beam into the incoming beams, the single AoA test configuration can generate the needed L1-RSRP of beams.

Figure 3 Single AOA flow

However the practical Rx beam plays a crucial role in FR2 UE performance, a question to Option 1 is whether the difference between really tested Tx/Rx beam and the artificial Tx/Rx beam can impact AI/ML model.

Proposal 12: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model?

The simplification of the CDL channel is achieved by constraining the directions Tx beams and and AOA/AOD. Observations regarding possible approaches to simplifying AOA/AOD can be found in R4-2500825, R4-2411254, and other relevant contributions. The simplified CDL channel consolidates similar AOA/AOD angles to enable spatial evaluation with a reduced number of probes.

Given that the maximal number of beams to be measured is 32, a important consideration is whether the 32 beams after simplified CDL channel can be produced by 8 or 12 probes thereby achieving the balance between performance and implementation complexity.

Proposal 13: RAN4 to check if the 32 beams after simplified CDL channel can be produced by 8 or 12 probes which may achieve the balance between performance and implementation complexity.

A test solution involving the measurement of different Tx/Rx beams using a time-division multiplexing (TDM) approach was proposed in previous meetings. While we remain open to this approach, the method for controlling Rx switching requires further clarification.

Proposal 14: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

The simplified CDL channel can, in principle, be applied to both training and inference. However, within the scope of common test configurations, only inference needs to be specified, as training is not part of the conformance test. Consequently, a comparison between the simplified CDL channel and the reference CDL channel during training is not required. In other words, training data may be generated using the reference CDL channel.

Proposal 15: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 16: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Issue: Beam prediction testing process

- Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others - Recommended WF - Take Option 1 as baseline, discuss further details of tests such as under what condition should the ground truth be measured, how to ensure the UE does not cheat, etc.

Below are some preliminary analyses on the condition of ground truth, e.g., the SNR condition.

The purpose of defining the SNR condition is to get an ideal RSRP measurement accuracy by minimizing (most of) measurement errors caused by low SNR. Hence, determining the SNR condition may rely on the configurations of reference signal, e.g., periodicity, density, bandwidth, subcarrier spacing and channel model in order to obtain a SNR level which is applicable for most typical field scenarios.

Recap the procedure deriving the accuracy requirement on the measured L1-RSRP in Rel-15, the achievable accuracy at certain SNR level is derived by simulations. In a same way, it is rational to extend those simulations to a higher SNR level where the L1-RSRP estimation error can be less than a threshold with X percentile.

Generating such SNR level in the test environment is another issue to be addressed, a concern is that the realization of SNR level is not only about generating a proper signal level but also the noise floor level generated as a side effect needs to be considered. To realize high SNR, the probes in test environment shall be able to generate signals at high output power level over the air. An underlying problem is that the wanted signal level can sometimes become too high to be created in the OTA environment. In that case, the noise floor becomes dominant which may deteriorate the test performance. Test equipment vendors are encouraged to comment on the SNR level from a test environment perspective.

Proposal 17: RAN4 to evaluate the SNR condition for the ground truth by simulation.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute the RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 18: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Issue: OTA Test setup for beam management

In this issue, we discuss the setup of channels to be applied in measurement and inference.

For BM-case1, which predicts the best beams in Set A (different from Set B) with respect to the measurements on Set B, at the same time domain with respect to spatial correlation between Set B and Set A. Since the time delay of measurement delay and inference delay is in dozens or hundreds of milli-seconds order, there’s no chance to rearrange random distributions and variations between measurement and prediction, the channel for measurement and prediction have to apply a same channel.

For BM-case2, which harnesses the historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A, from test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

A question is relevant to measurement window and the inference window, which are configured by NW, may have respective minimal length with respect to a certain prediction performance. Such that a set of consequent changes of channel parameters shall be designed during measurement window and the inference window. At the least, establishing a set of channel instants from channel #a+1 to channel #a+m for measurement window and channel #b+1 to channel #b+n for inference window.

These channels contain a set of parameters which update sequentially and gladfully starting from measurement until end of inference. The parameters wherein to be updated may comprise one or more than one of the below:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters, in order to mimic the UE’s movement, may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

Proposal 19: For BM-case1, same channel is applied for measurement and inference.

Proposal 20: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Issue:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The rationale behind the agreements on the issue is that the coverage of datasets for training/testing shall be broad adequately to meet or be close to the practical field deployment scenario as much as possible. In this manner, we suggest considering defining the test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 21: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

Since the training data are uniquely affected by the specific UE hardware and digital processing units, the error of training data could be left for implementation from each UE vendor, the effect to the predict accuracy may be covered by the overall measurement error.

Observation 6: the measurement error in datasets for training/testing may be covered by the overall measurement error.

# Conclusion

Observation 1: RSRP in Option3 of Beam prediction KPIs is groundtruth RSRP, i.e., the maximum RSRP among top-K predicted beams and the RSRP of the strongest beam are groundtruth RSRP.

Observation 2: The term ‘successful rate’ in Option 3 of Beam prediction KPIs shall be removed, as Option3 represents a mandatory requirement that must be strictly satisfied.

Observation 3: The L1-RSRP differences of SSB with the SNR value -3 is only a few tenths of dBs so its impact would be minor as the baseband (BB) error.

Observation 4: Both ±4.5 dB RF error with Gaussian distribution and uniform distribution can bring large performance degradation on prediction accuracy especially for Top-1 beam, where the impact of uniform distributed RF error is even worse.

Observation 5: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

Observation 6: the measurement error in datasets for training/testing may be covered by the overall measurement error.

Proposal 1: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

Proposal 2: In Option2 of Beam prediction KPIs, RAN4 considers the below option.

- Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

- Not introduce the parameter N.

Proposal 3: Maintain the current wording of Option 3 of Beam prediction KPIs without modification, as no further improvements are necessary.

Proposal 4: Support Option 1 of the relative RSRP accuracy, relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

Proposal 5: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting.

Proposal 6: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied, with the assumption that the measurement accuracy remains consistent.

Proposal 7: RAN4 to study whether specific requirements are necessary to evaluate the capability of a UE in tracking changes in L1-RSRP through measurements in BM-case 2.

Proposal 8: The distribution of baseband error can be fitted with zero mean Gaussian distributions, the sigma value can be further discussed after the BB error distribution is confirmed.

Proposal 9: the definition of “90%-tile L1-RSRP difference” should be clarified, such as: 90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP difference), abs (5%-tile L1-RSRP difference)).

Proposal 10: The performance degradation from ±4.5 dB RF error is not neglectable, further discussion is needed for whether accept this performance degradation or not.

Proposal 11: RAN4 shall define measurement delay for monitoring.

Proposal 12: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model?

Proposal 13: RAN4 to check if the 32 beams after simplified CDL channel can be produced by 8 or 12 probes which may achieve the balance between performance and implementation complexity.

Proposal 14: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

Proposal 15: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 16: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Proposal 17: RAN4 to evaluate the SNR condition for the ground truth by simulation.

Proposal 18: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Proposal 19: For BM-case1, same channel is applied for measurement and inference.

Proposal 20: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Proposal 21: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

# References

- R4-2414323 WF on AI/ML, Qualcomm