3GPP TSG-RAN WG4 Meeting #114bis	R4-2504108

Wuhan, China, April 7th-11th, 2024

Source: 	Ericsson

Title:  	Initial discussion on RRM requirements for Ambient-IoT

Agenda Item:	7.24.3

Document for:	Discussion

# Introduction

The WI Solutions for Ambient IoT (Internet of Things) in NR [1] was approved from at RAN#106 meeting. Some relevant content is duplicated in the below as:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In this contribution, we present our initial views on ambient-IoT.

# Discussion

### Regarding the device definition of Device 1, referring to TR 38.769 V19.0.0, it appears that a device hasn’t active reception/measurement capability other than backscattering.

- Antenna could be either shared or separate for RF energy harvester and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks). - RF energy harvester can include rectifier performing RF signal (AC) to DC conversion. - Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and supplying power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - RF Envelope Detector converts RF signal to baseband. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator. - Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study. - Comparator determines high/low of input signal. - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with transmitted signal from BB logic.

We aim to analyze the impact of Ambient-IoT on RRM requirements from multiple perspectives in the below content.

First of all, from high level, protocol stack and functionality aspects are considered in TR 38.769 V19.0.0, which is duplicated as follow.

As to the A-IoT required functionalities, the following functionalities are supported: - A-IoT paging (see clause 6.3.3) - A-IoT random access procedure (see clause 6.3.4) - A-IoT data transmission (see clause 6.3.5) As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): - AS security (The AS layer design assumes no support of AS security, unless the study in [152] further concludes differently.) - RRC states - RRC connection management - RRM L3 measurement reporting - Mobility - ASN.1 encoding/decoding - Periodical system information and MIB - Tracking/RAN area update procedure - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. - Multiple A-IoT logical channels for upper layer data - NR SR, as in TS 38.300 [153] - NR BSR, as in TS 38.300 [153]

Furthermore, the D1T1-B scenario, also captured by TR 38.769 V19.0.0, is shown in the below table:

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-B | CW outside topology |  | - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW |  |

Observation 1: Mobility isn’t applicable for Ambient-IoT device.

Observation 2: RRM L3 measurement isn’t applicable for Ambient-IoT device.

Observation 3: Signalling characteristics (covered by Clause 8 in TS38.133) isn’t applicable for Ambient-IoT device.

R2D signal:

In the context of Ambient-IoT communication between device and reader, any dedicated physical broadcast channel, e.g. PBCH-like, and reference signals including at least DMRS, PTRS, CSI-RS/TRS, are not considered for R2D. Consequently, no explicit measurement requirements are defined for reference signals, which also is agreed in RAN1 as duplicated in the below:

The Physical R2D Channel (PRDCH) serves as the sole physical channel for R2D communication, as duplicated in the below RAN1 agreements, responsible for carrying any higher-layer payload and L1 R2D control information (if defined).

SI agreements in RAN1#117:

Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

SI agreements in RAN1#118:

Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

PRDCH utilizes an OFDM-based OOK waveform with a subcarrier spacing of 15 kHz. The device receives R2D messages, such as paging information, through PRDCH and responds via backscattering. Due to its reliance on envelope detection, the device is unlikely to perform conventional signal quantity/quality measurement procedures that require reference signal-based assessments.

However, we, from RAN4 perspective, shall evaluate the measurement from a practical application perspective, specifically examining whether a device can derive tangible benefits from it.

A possible use case concerning measurement is that the device may determine D2R transmission with respect to the received signal quantity/quality.

Observation 4:  No conventional DL reference signal, which can be used for meaurement purpose, is applied for Ambient-IoT device.

Observation 5:  Ambient-IoT device may assess D2R transmission based on the received signal's quantity/quality; however, its mechanism also is contingent upon agreements by other WGs.

Proposal 1: RAN4 to check whether measurement is needed for Ambient-IoT device, with a focus on the potential benefits the device can obtain.

- An example of underlying usage of measurement is the device may determine D2R transmission with respect to the received signal quantity/quality. Requirements may be needed for specifying the received signal quantity/quality.

R2D timing:

An R2D timing acquisition signal (R-TAS), immediately preceding the transmission of PRDCH, is included at least for timing acquisition and indicating the start of R2D transmission in the time domain. Below RAN1 agreements were reached:

SI agreements in RAN1#117:

Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

SI agreements in RAN1#118bis:

Agreement For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. Agreement For R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used.

An A-IoT device may apply the R-TAS for internal oscillator calibration, it means the device possesses the capability to manage the downlink (DL) timing necessary for the reception of the Physical R2D Channel (PRDCH).

In legacy, we don’t define the DL timing requirement, because downlink timing is entirely controlled by the NW, and the UE is expected to synchronize with the received downlink signals rather than adjust them. But it’s observed that TD2R_min, which limits the minimum time between a D2R transmission and the corresponding R2D transmission, may bring demands to the device.

Proposal 2: RAN4 to check if DL timing requirements shall be defined for Ambient-IoT device, including going through existing timing requirements and check if they are still relevant for A-IoT, e.g., whether they are relevant and can they be done using R-TAS /PRDCH?

Observation 6: A possible limitation to DL timing synchronization is the reception of R-TAS, may depend on the last R2D transmission TD2R_min.

D2R signal:

For D2R, a physical channel PDRCH carries any higher-layer payload, the response transmitted from device to reader during the contention-based access procedure, and L1 D2R control information, if defined. RAN1 obtained the below agreement:

D2R timing:

Regarding D2R timing, A D2R timing acquisition signal (D-TAS), preceding each PDRCH, is included at least for timing acquisition.

In legacy the SSB minimum period (160 ms) contributes to some clock drift between reception of PSSS/SSS (PBCH) and this adds to UL timing uncertainty (Te) in legacy.

The A-IoT device will drift rather fast since the initial sampling frequency offset (SFO) is 10^x ppm from WID. If an oscillator can get some external sync, then it is MUCH better than SFO, it improves fast. Given the R2D timing signal, the desired A-IoT UL timing accuracy, represented by D-TAS, is a function of R-TAS DL BW, device SFO and the periodicity of R-TAS. Since the UL timing will drift a lot between R-TAS receptions, so the TR2D_min time might play a role in determining acceptable UL time accuracy.

We notice the below two aspects:

- The transmission timing of D-TAS and PDRCH relies on determination of the R2D timing acquisition signal (R-TAS), the UE at least may be able to maintain the timing accuracy of D-TAS by calibrating and adjusting the internal oscillator, which comprises:

  - The transmission timing error, which indicates the UL timing error referring to the DL timing.

  - The adjustment error, which indicates the capability for UE adjusting UL timing error.

- Moreover, even though the D2R is backscattered, the device may have the capability to manage the transmission timing of the D-TAS to some extent, such as through the use of delay circuits in its implementation.

  - The delay timing (or timing shift) error, which indicates the offset between the DL timing and the UL timing.

  - The adjustment error, which indicates the capability for UE adjusting the offset between the DL timing and the UL timing.

Proposal 3:  RAN4 shall evaluate whether an uplink (UL) timing requirement needs to be defined for Ambient-IoT device, potentially encompassing, but not limited to, the following aspects:

- The transmission timing of D-TAS and PDRCH relies on determination of the R2D timing acquisition signal (R-TAS), the UE at least may be able to maintain the timing accuracy of D-TAS by calibrating and adjusting the internal oscillator, which comprises:

  - The transmission timing error, which indicates the UL timing error referring to the DL timing.

  - The adjustment error, which indicates the capability for UE adjusting UL timing error.

- Moreover, even though the D2R is backscattered, the device may have the capability to manage the transmission timing of the D-TAS to some extent, which comprises:

  - The delay timing (or timing shift) error, which indicates the offset between the DL timing and the UL timing.

  - The adjustment error, which indicates the capability for UE adjusting the offset between the DL timing and the UL timing.

Some relevant agreements in RAN1 are duplicated as follow:

SI agreements in RAN1#117:

Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

SI agreements in RAN1#116:

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

SI agreements in RAN1#117:

Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ³ TR2D_min - FFS the maximum value(s) for TR2D

SI agreements in RAN1#118bis:

Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Agreement Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3 Agreement RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception.

CW:

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1 from WID, the following cases for CW transmission are studied, referring to TR 38.769 V19.0.0:

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Continuous Wave (CW) signals are unmodulated carrier waves, meaning they do not carry actual data or signaling information. From measurement perspective, no requirements can capture the continuous wave characteristics.

As we know, energy harvesting threshold is under discussion in RF session, the purpose is to request the ambient-IoT device being able to adopt CW for backscattering.

That said, an underlying use case concerning CW reception is the one or more backscattering operational behavior may depend on the received CW quantity/quality. If the case is valid, CW reception may be specified with certain requirements.

Proposal 4:  RAN4 to check whether requirements concerning CW reception shall be defined besides of energy harvesting threshold studied in RF session, e.g.,

- The device may determine D2R transmission with respect to the received CW quantity/quality. Requirements may be needed for specifying the CW quantity/quality.

Random access:

From the perspective of the physical layer, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used. The response transmitted from the device to the reader during this procedure is transmitted on PDRCH.

The random access procedure is summarized by steps as follow, referring to TR 38.769 V19.0.0 and RAN1/2 agreements in the below,

- Receives A-IoT paging message, which may contain an identifier or not.

- Transmits Msg1, which may contain random ID and device ID, if

- Receives Msg2, which is used for contention resolution.

- Transmits Msg3, which may contain device ID and/or any other upper layer data

In random access for Ambient-IoT, Msg2 monitoring is agreed to be studies with some progresses in RAN1 and RAN2, which are duplicated in below.

SI agreements in RAN1#119:

Agreement For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring - For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device, - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. - For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources. Agreement For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring - Option 1: Predefined - Option 2: Indicated in the R2D transmission triggering random access - Note: Option 1 and Option 2 may not be mutually exclusive. Agreement For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options for a device to determine the reference time for the starting time for Msg2 monitoring - Option 1: End of time domain resource where the device transmitted the Msg1 - Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) - Option 3: End of the R2D transmission triggering random access - Option 4: End of a R2D transmission carrying Msg2 - Note the reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

SI agreements in RAN 2#126:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements.

- A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

- RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information.

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed.

SI agreements in RAN 2#127bis:

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

SI agreements in RAN 2#128:

Agreements

- For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

- FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA.

- Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

- RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

- If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

- Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

Observation 7:  Random access is applicable for Ambient-IoT, which is defined in a way of unification of CBRA and CFRA.

Proposal 5: RAN4 to check if requirements regarding random access (including 2step and 3step CBRA) shall be defined, e.g.,

- The correct behaviour after receiving paging.

- The correct behaviour after receiving Msg2.

- The correct behaviour after the monitoring window expires without receiving Msg2.

# Conclusion

Observation 1: Mobility isn’t applicable for Ambient-IoT device.

Observation 2: RRM L3 measurement isn’t applicable for Ambient-IoT device.

Observation 3: Signalling characteristics (covered by Clause 8 in TS38.133) isn’t applicable for Ambient-IoT device.

Observation 4:  No conventional DL reference signal, which can be used for meaurement purpose, is applied for Ambient-IoT device.

Observation 5:  Ambient-IoT device may assess D2R transmission based on the received signal's quantity/quality; however, its mechanism also is contingent upon agreements by other WGs.

Observation 6: A possible limitation to DL timing synchronization is the reception of R-TAS, may depend on the last R2D transmission TD2R_min.

Observation 7:  Random access is applicable for Ambient-IoT, which is defined in a way of unification of CBRA and CFRA.

Proposal 1: RAN4 to check whether measurement is needed for Ambient-IoT device, with a focus on the potential benefits the device can obtain.

- An example of underlying usage of measurement is the device may determine D2R transmission with respect to the received signal quantity/quality. Requirements may be needed for specifying the received signal quantity/quality.

Proposal 2: RAN4 to check if DL timing requirements shall be defined for Ambient-IoT device, including going through existing timing requirements and check if they are still relevant for A-IoT, e.g., whether they are relevant and can they be done using R-TAS /PRDCH?

Proposal 3:  RAN4 shall evaluate whether an uplink (UL) timing requirement needs to be defined for Ambient-IoT device, potentially encompassing, but not limited to, the following aspects:

- The transmission timing of D-TAS and PDRCH relies on determination of the R2D timing acquisition signal (R-TAS), the UE at least may be able to maintain the timing accuracy of D-TAS by calibrating and adjusting the internal oscillator, which comprises:

  - The transmission timing error, which indicates the UL timing error referring to the DL timing.

  - The adjustment error, which indicates the capability for UE adjusting UL timing error.

- Moreover, even though the D2R is backscattered, the device may have the capability to manage the transmission timing of the D-TAS to some extent, which comprises:

  - The delay timing (or timing shift) error, which indicates the offset between the DL timing and the UL timing.

  - The adjustment error, which indicates the capability for UE adjusting the offset between the DL timing and the UL timing.

Proposal 4:  RAN4 to check whether requirements concerning CW reception shall be defined besides of energy harvesting threshold studied in RF session, e.g.,

- The device may determine D2R transmission with respect to the received CW quantity/quality. Requirements may be needed for specifying the CW quantity/quality.

Proposal 5: RAN4 to check if requirements regarding random access (including 2step and 3step CBRA) shall be defined, e.g.,

- The correct behaviour after receiving paging.

- The correct behaviour after receiving Msg2.

- The correct behaviour after the monitoring window expires without receiving Msg2.

# References

- RP-250796, New WID on Rel-19 Ambient IoT