3GPP TSG-RAN WG4 Meeting #114bis	R4-2504110

Wuhan, China, 7th-11th, April 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Agenda Item:	7.30.3

Document for:	Discussion

# Introduction

In this contribution, we share our perspectives on the remaining issues in the WF [1] that were documented in the last meeting.

# Discussion

Issue 1-1-1: Procedural requirements for CB-MSG3 transmission

Agreement: - Include procedural requirements for CB-MSG3 transmission in a new subclause of 6.6A in TS36.133. - Further discuss the content to include in the next meeting.

RAN2 agreements in RAN2#128 are excerpted as below:

Agreements: 1. Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) 2. Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels 3. The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) 4. There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) 5. The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific 6. There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) 7. For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time 8. For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) 9. The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) 10. Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) 11. Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. 12. RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

RAN2#129 reached the agreements as below:

Agreements: 1. It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used 2. RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC 3. RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS 4. For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. 5. The UE shall at most have one ongoing CB EDT procedure at any time. 6. The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. 7. The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. 8. CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. 7. As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. 8. RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) 2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In TS 36.133, the relevant requirements are duplicated in the below:

#### 6.6A.2.1 Correct behaviour when receiving Random Access Response reception The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. #### 6.6A.2.2 Correct behaviour when not receiving Random Access Response reception The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 TS 36.321. #### 6.6A.2.3 Correct behaviour when receiving a NACK on msg3 The UE shall re-transmit the msg3 upon the reception of a NACK on msg3. #### 6.6A.2.4 Correct behaviour when receiving a message over Temporary C-RNTI The UE shall send ACK if the Contention Resolution is successful. The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message. #### 6.6A.2.5 Correct behaviour when contention Resolution timer expires The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

To our understanding, RAN4 may be aware of UE’s behaviors regarding two scenarios after sending out CB-msg3:

- Receives Msg4 with contention resolution within monitoring window(s)

  - UE shall send ACK if the contention resolution is successful.

- Monitoring window expires.

  - UE may attempt another CB-msg3 transmission (may be indicated by CB-Msg4) after waiting for a random back-off with an increased Tx power.

  - UE may fallback to 4-step random access EDT after a number of failed CB-msg3 transmission attempts.

Observation 1: RAN4 may be aware of UE’s possible behaviours, which still under RAN2’s discussions, regarding two scenarios:

- If the UE receives Msg4 with contention resolution within monitoring window (s),

  - UE may send ACK if the contention resolution is successful.

- If monitoring window(s) expires,

  - UE may attempt another CB-msg3 transmission (may be indicated by CB-Msg4) after waiting for a random back-off with an increased Tx power.

  - UE may fallback to 4-step random access EDT after a number of failed CB-msg3 transmission attempts.

Proposal 1: RAN4 can update the procedural requirements for CB-MSG3 transmission after RAN2 achieves agreements.

# Conclusion

Observation 1: RAN4 may be aware of UE’s possible behaviours, which still under RAN2’s discussions, regarding two scenarios:

- If the UE receives Msg4 with contention resolution within monitoring window (s),

  - UE may send ACK if the contention resolution is successful.

- If monitoring window(s) expires,

  - UE may attempt another CB-msg3 transmission (may be indicated by CB-Msg4) after waiting for a random back-off with an increased Tx power.

  - UE may fallback to 4-step random access EDT after a number of failed CB-msg3 transmission attempts.

Proposal 1: RAN4 can update the procedural requirements for CB-MSG3 transmission after RAN2 achieves agreements.

# References

- R4-2502606, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek