3GPP TSG-RAN WG4 Meeting #114bis	R4-2504115

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for FR2-1 L3 measurement enhancement

Agenda Item:	7.17.4

Document for:	Discussion

# Introduction

This contribution outlines our point of view on performance requirements for FR2-1 L3 measurement enhancement based on the relevant core requirements which have been agreed in previous meetings.

# Discussion

## Topic #1: Performance requirements for FBS L3 measurement

Below scenarios are impacted by FBS L3 measurement as per the agreements on core requirements.

- SSB based Intra-frequency measurement without MG

- SSB based Intra-frequency measurement with MG

- SSB based Intra-frequency measurement with MG

- SSB based Inter-frequency measurement without MG

- SSB based Inter-frequency measurement with MG

- Handover

- RRC Re-establishment

- RRC Connection Release with Redirection

Accordingly, new performance requirements need to be introduced to verify the FBS L3 measurement core requirements. As per the WI’s content,  the tests only take the single FR2 serving cell configuration into account.

Given that, we propose below test cases:

- Inter-frequency handover from FR2 to FR2; unknown target cell for UE configured with [FBS signaling]

- Inter-frequency RRC Re-establishment in FR2 for UE configured with [FBS signaling]

- Redirection from NR in FR2 to NR in FR2 for UE configured with [FBS signaling]

- Intra-frequency Measurements

  - [FBS] triggered measurement and reporting test without gap under non-DRX

- Inter-frequency Measurements

  - [FBS] triggered measurement and reporting tests for FR2 without SSB time index detection when DRX is not used

Proposal 1: RAN4 to consider at least the below test cases for verifying performance requirements for FBS L3 measurement:

- Inter-frequency handover from FR2 to FR2; unknown target cell for UE configured with [FBS signaling]

- Inter-frequency RRC Re-establishment in FR2 for UE configured with [FBS signaling]

- Redirection from NR in FR2 to NR in FR2 for UE configured with [FBS signaling]

- Intra-frequency Measurements

  - [FBS] triggered measurement and reporting test without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements

  - [FBS] triggered measurement and reporting tests for FR2 without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Another issue, which hasn’t been fixed in core requirements, is FBS quit condition. Depending on the outcome, quitting FBS may need to be verified by performance requirements.

Proposal 2: RAN4 to define tests cases verifying the process of quitting FBS.

## Topic #2: Performance requirements for CSSF enhancement

The below agreements on core requirements are reached in previous meetings.

Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. …

Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

Although no final agreements have been reached on Solution 3, the CSSF enhancement primarily affects L3 measurements. Therefore, it is reasonable to initiate the study of performance requirements at this stage.

To validate CSSF Solution 1 based on performance requirements, multiple serving cells should be configured within a single frequency band.

Proposal 3: CSSF solution 1 shall be verified in performance requirements, the test case may consider below aspects:

- Intra-frequency Measurements, including

  - Event triggered reporting test in CA configuration without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements, including

  - Event triggered reporting tests in CA without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Where the CA carrier configuration includes [3] serving cells (one PCC and 2 SCCs) in a single FR2 band.

To validate CSSF Solution 3 based on performance requirements, multiple serving cells should be configured across multiple single bands.

Proposal 4: CSSF solution 3 shall be verified in performance requirements, the test case may the test case may consider below aspects:

- Intra-frequency Measurements, including

  - Event triggered reporting test in CA configuration without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements, including

  - Event triggered reporting tests in CA without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Where the DC carrier configuration includes [3] serving cells, wherein one PCC in a first FR1 band and 1 SCC in a second FR1 band and 1 PSCC in a first FR2 band.

# Conclusion

Proposal 1: RAN4 to consider at least the below test cases for verifying performance requirements for FBS L3 measurement:

- Inter-frequency handover from FR2 to FR2; unknown target cell for UE configured with [FBS signaling]

- Inter-frequency RRC Re-establishment in FR2 for UE configured with [FBS signaling]

- Redirection from NR in FR2 to NR in FR2 for UE configured with [FBS signaling]

- Intra-frequency Measurements

  - [FBS] triggered measurement and reporting test without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements

  - [FBS] triggered measurement and reporting tests for FR2 without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Proposal 2: RAN4 to define tests cases verifying the process of quitting FBS.

Proposal 3: CSSF solution 1 shall be verified in performance requirements, the test case may consider below aspects:

- Intra-frequency Measurements, including

  - Event triggered reporting test in CA configuration without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements, including

  - Event triggered reporting tests in CA without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Where the CA carrier configuration includes [3] serving cells (one PCC and 2 SCCs) in a single FR2 band.

Proposal 4: CSSF solution 3 shall be verified in performance requirements, the test case may the test case may consider below aspects:

- Intra-frequency Measurements, including

  - Event triggered reporting test in CA configuration without gap under non-DRX

  - More gap/DRX configurations may be added.

- Inter-frequency Measurements, including

  - Event triggered reporting tests in CA without SSB time index detection when DRX is not used

  - More gap/DRX configurations may be added.

Where the DC carrier configuration includes [3] serving cells, wherein one PCC in a first FR1 band and 1 SCC in a second FR1 band and 1 PSCC in a first FR2 band.

# References