3GPP TSG-RAN WG4 Meeting #114-bis		Draft R4-2504117

Athens, Greece, Feb. 17th – 21st, 2025

Title:	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Response to:	R2-2501572

Release:	Release 19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:

Name:	Antti Immonen

E-mail Address:	aimmonen@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN2 for the reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx.

RAN4 has considered input provided and questions asked by RAN2, acknowledging RAN2 agreement on the necessary scheduling restrictions being network implementation matter. RAN4 would like to provide the following answers into RAN2 questions.

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

- Support of simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction would require partial band RF filter(s) and explicit frequency range definitions in RAN4 specification. RAN4 has agreed that this approach is not pursued in Rel-19, and if pursued in later releases then new bands are considered. Hence it is not possible to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, and it’s not possible to support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

- CA_n5-n8 cannot support simultaneous Rx/Tx because of the frequency overlap (in particular, concurrent n8 Tx and n5 Rx). As explained in Answer 1, frequency gap/guard band is not considered as a solution to enable DL_CA_n5A-n8A with UL CA_n5A-n8A. Hence additional UE capabilities to enable simultaneous Rx/Tx in this case would not help to enable simultaneous Rx/Tx.

In addition, RAN4 would like to inform RAN2 that it has decided to add [35,140us] gap between n8 UL and n5DL, and between n5 DL and n8 UL to allow UE to reconfigure RF Front-End (RFFE). This gap is addressed in the same note with scheduling restriction for n8 UL/n5 UL.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account.

# 3	Dates of next TSG-RAN WG2 meetings

3GPP TSG RAN WG2#129bis	April 7th – 11th, 2025			China

3GPP TSG RAN WG2#130	May 19th – 23rd, 2025			Malta