3GPP TSG-RAN WG4#114-bis	R4-2504119

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.4.1

Source: 	Qualcomm France

Title: 	Intra-band Non-collocated NR CA/EN-DC requirements

Document for:	Approval

# Introduction

Considerations for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- Background

WF was agreed in RAN4#114 [1]:

## Sub-topic 2-1 : UE RF requirements for Type 4 EN-DC/NR-CA

### <Issue 2-1-1:  How to capture the power imbalance and REFSENS requirements for Big CR>

Way forward:

- The CR leader take companies’ inputs of R4#114 and previous meetings into account when drafting actually.

### <Issue 2-1-2:  Whether to introduce term “Type 4” for pre Rel-19 specifications>

Way forward:

- Similar handling as type 2 for Rel-18.

## Sub-topic 2-2 : UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

### <Issue 2-2-3: UE behavior or maxMIMO-layers>4/CC>

Agreement:

  - When maxMIMO-layers > 4, Type 1(collocated). No new BS signaling is needed.

### <Issue 2-2-7: BS signaling framework>

Way forward:

- Continue further discussions on following options in the next RAN4 meeting.

- Option 1 (Samsung/Apple/KDDI)

1）maxMIMO-layers ＞4, it is collocated.

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

- Option 2 (Nokia)

- UE is Type 1 (2/CC), Type 1 (4/CC) or Type 4 depending on present Rel-19 BS signaling element value.

- When Rel-19 BS signaling is absent, UE follows Rel-18 BS signaling, i.e. is Type 4.

- Option 3 (Qualcomm)

1）When maxMIMO-layers ＞4

- UE is Type 1

2）When maxMIMO-layers = 4

- Evaluate following options

  - Type 1 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

  - Type 4 when rel-19 BS signaling is absent, and respective Rel-19 BS signaling

3）When maxMIMO-layers = 2, UE follows Rel-18

- Option 4 (ZTE)

1）If maxMIMO-layer > 4, Rel-19 Type 1 is assumed.

2）If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify Type 4 or Type 1. If maxMIMO-layer = 4 and NW signaling is not provided, Type 1 is assumed.

- Option 5 (Huawei)

1）When maxMIMO-layers ＞4

- UE is Type 1

2) When maxMIMO-layers = 4

- When BS signaling is absent, Type 4 is considered. (in accordance to the way Type 2 was specified in Rel-18) UE is Type 1.

- When BS signalling is present, Type 1 is considered. (in accordance to the way Type 1 was specified in Rel-18).

3) When maxMIMO-layers = 2, UE follows Rel-18

## Sub-topic 2-3 : Other aspects (incl. clarification of contiguous LTE CCs)

### <Issue 2-3-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Firstly, conclude up to Rel-18 CRs on improving specification readability.

- Then, descriptions of Rel-19 UE RF requirements should be aligned to above conclusion.

- Discussion

### To recap, the behavior of the Rel-18 UE indicating support of Rel-18 Non-collocated scenario is as follows.

### Rel-18 UE+Rel-18NW

- maxMIMO-layers=2

  - BS signaling present => Collocated with 2L

  - BS signaling absent => Non-collocated with max 2L

- maxMIMO-layers=4

  - BS signaling field is absent and UE is Collocated

- maxMIMO-layers>4

  - BS signaling field is absent and UE is Collocated

### We believe the main open issue is if Rel-19 UE supporting Rel-19 Non-collocated scenario is collocated or non- collocated when maxMIMO-layers=4 and BS signaling field is absent. Below we show UE behavior with both alternatives for both Rel-19UE+Rel-19NW, and R19 UE+R18 NW. Blue text is when UE is collocated and Red text is when UE is non-collocated. Basically either of the schemes could work for Rel-19 UE in Rel-19 NW.

### Rel-19 UE+Rel-19 NW

- maxMIMO-layers=2

  - BS signaling present => Collocated with 2L

  - BS signaling absent => Non-collocated with 2L

- maxMIMO-layers=4

  - BS signaling present => Non-collocated with 4L

  - BS signaling absent => Collocated with 4L

  - BS signaling present => Collocated with 4L

  - BS signaling absent => Non-collocated with 4L

- maxMIMO-layers>4

  - BS signaling field is absent and UE is Collocated

### Rel-19 UE+Rel-18 NW

- maxMIMO-layers=2

  - BS signaling present => Collocated with 2L

  - BS signaling absent => Non-collocated with 2L

- maxMIMO-layers=4

  - BS signaling present => Non-collocated with 4L

  - BS signaling absent => Collocated with 4L

  - BS signaling absent => Non-collocated with 4L

- maxMIMO-layers>4

  - BS signaling field is absent and UE is Collocated

### In Red text case, Rel-18 NW tries to tell it’s collocated, but UE understands absent BS field as it being non-collocated so there is discrepancy. Please note that in Red text case, BS signaling field cannot be present with maxMIMO-layers=4 as per Rel-18 specifications.

### The easiest way to avoid any discrepancies would be to choose approach with blue text, i.e assume UE is collocated when maxMIMO-layers=4 and BS signaling field is absent. This way, Rel-18 and Rel-19 UE’s would also behave same way with maxMIMO-layers=4 and BS signaling field being absent. If behavior according to red text is chosen, then the discrepancy with Rel-19 UE in Rel-18 NW would need to be ignored. UE which is supporting non-collocated mode and obviously support also collocated so from that perspective nothing would go broken even in that case.

### We understand that in Rel-18, the targeted non-collocated case with 2L was achieved with BS signaling field being absent. It can be arguably said that same principle should be followed in Rel-19, meaning targeted non-collocated case with 4L to be achieved with BS signaling field being absent. However there is one big difference, that is these NR bands are required to support 4L so it’s a different thing to play with 2L for non-collocated/collocated for bands which support 4L, than to play with 4L for non-collocated/collocated for bands which support 4L.

Our preference would be to choose the following UE behavior. With this, there would be no need to have earlier discussed Present with “0”/Present with “1” approach.

Proposal 1: RAN4 to agree the following UE behavior for Rel-19 Non-collocated EN-DC/NR-CA and indicate this to RAN2. (Note that maxMIMO-layers=2 and maxMIMO-layers>4 are already agreed and are here just for completeness)

- maxMIMO-layers=2

  - BS signaling present => Collocated with 2L

  - BS signaling absent => Non-collocated with 2L

- maxMIMO-layers=4

  - BS signaling present => Non-collocated with 4L

  - BS signaling absent => Collocated with 4L

- maxMIMO-layers>4

  - BS signaling field is absent and UE is Collocated

- Conclusion

Considerations for Non-collocated NR CA/EN-DC UE were provided with the following proposals.

Proposal 1: RAN4 to agree the following UE behavior for Rel-19 Non-collocated EN-DC/NR-CA and indicate this to RAN2. (Note that maxMIMO-layers=2 and maxMIMO-layers>4 are already agreed and are here just for completeness).

- maxMIMO-layers=2

  - BS signaling present => Collocated with 2L

  - BS signaling absent => Non-collocated with 2L

- maxMIMO-layers=4

  - BS signaling present => Non-collocated with 4L

  - BS signaling absent => Collocated with 4L

- maxMIMO-layers>4

  - BS signaling field is absent and UE is collocated

- Reference

[1] R4-2502924, “WF on UE requirements for non-collocated scenario”, KDDI