3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504126

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	On further improvements for simultaneous Rx-Tx notes

Agenda item:	6.11.2

Document for:	Discussion

Introduction

Simultaneous Rx/Tx UE Capability

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory, and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

At RAN4#108bis a discussion was raised in [1] on the ambiguity of the conditional mandatory simultaneous Rx/Tx UE capability. This led to a WF being agreed in [2] which was further discussed in [3] at RAN4#109. Further discussion led to a WF [4] at RAN4#109. During RAN4#110 another WF [5] was agreed based on the discussion captured in [6]. The WF resulted in a draftCR [7] being endorsed. This led to an improvement of the notes by removing redundant notes with a CR [8] in RAN4#111.

Discussion

Missing implementation of endorsed draftCRs

At RAN4#112 it was possible to remove yet another redundant note with the draftCR in [9] as well as clarification on the applicability related to FDD combinations in [10] and [11]. However, the endorsed CRs was not implemented to the TS after the meeting.

Endorsed improvements for simultaneous Rx-Tx in R4-2412541, R4-2414266 and R4-2414267 was not implemented to the TSs after RAN4#112.

We believe that these draftCRs improve the understanding of the applicability of simultaneous Rx-Tx why these should be implemented to the TS as already endorsed by RAN4

At RAN4#112bis it was in fact noted by some companies that the above mentioned CRs were not implemented but no action seems to have been taken. The missing implementation meant that a CR in [12] was postponed.

The previously not implemented endorsed draftCRs meant the CR in R4-2416359 were postponed.

At RAN4#113 a draftCR [13] were endorsed seemingly combining the content of [10] and [12]. This, [13], was implemented into the BigCR for 38.101-1 (Note the Tdoc title says 38.101-3 but the CR is for 38.101-1) together with the content of [9] in [14] which were endorsed. Similarly, a bigCR for 38.101-3 were endorsed with the content of [11] was endorsed in [15]. However, none of the two bigCRs were implemented meaning none of the additional endorsed improvements for simultaneous Rx-Tx is visible in the TSs.

BigCRs R4-2417985 and R4-2417986 was not implemented to the specifications after RAN4#113 as MCC focused on other CRs and wanted to postpone the implementation of these.

At RAN4#114 BigCR for 38.101-1 was agreed in [16] and for 38.101-3 in [17]. These two bigCRs should include all the endorsed changes in [9-15]

All previous endorsed changes are included in the agreed BigCRs R4-2500714 and R4-2500715 from RAN4#114.

RAN4 shall ensure R4-2500714 and R4-2500715 are implemented to the TSs including all already endorsed agreements related to simultaneous Rx-Tx.

Agreements at previous meetings

At RAN4#114 a WF for simultaneous Rx-Tx [18] includes the following:

Way forward on Simultaneous Rx/Tx feature with inter-band CA/DC:

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode

  - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD.

  - For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling.

    - Deviations to this due to UL CA_n5-n8 is TBD

  - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling.

    - Deviations to this above due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

- Consider to specify the requirements of simultaneous Rx-Tx for inter-band CA/DC using a new column in the two-band BC table(s), capturing the handling of each type of BC with different duplex modes.

  - Handling properly every Note for simultaneous Rx-Tx.

Handling of simultaneous Rx-Tx interactions

Together with the agreements made for FDD-FDD combinations expected to enter the specification and the in the WF captured additional cases of TDD-SDL and FDD-SDL all CA configuration types should be covered. This as TDD-TDD and TDD-FDD cases are already indicated via the conditional mandatory capability described in the introduction.

With agreements from the WF at RAN4#114 and the existing framework for conditional mandatory support of simultaneous Rx-Tx all types of CA configurations seem covered.

The discussion is now focused on the exceptions related to e.g. CA_n5-n8 which is further discussed in [19]. Our understanding is that RAN2 is currently working on enabling the needed capability with indication of these exceptions to mandatory support of simultaneous Rx-Tx FDD-FDD combinations.

RAN2 is working on needed capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

RAN4 shall ensure RAN2 understands the need for a capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

Further improvements to the indication of conditional mandatory support of simultaneous Rx/Tx

As captured in the WF and discussed in [20] RAN4 can consider abandoning the use of notes to indicate the mandatory support of simultaneous Rx/Tx with text in a new column in the BC tables. Here an important point to first address is whether there is a need for more than the two-band BC table. This as there currently are also notes for simultaneous Rx/Tx in the 3, 4 and 5-band BC tables in TS 38.101-1.

RAN4 shall discuss whether indication of mandatory support of simultaneous Rx/Tx are needed for other than two-band BCs.

Whether or not adopting a new column in the BC table(s) can be further discussed after the clarification on above.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

Observation 2: Endorsed improvements for simultaneous Rx-Tx in R4-2412541, R4-2414266 and R4-2414267 was not implemented to the TSs after RAN4#112.

Observation 3: The previously not implemented endorsed draftCRs meant the CR in R4-2416359 were postponed.

Observation 4: BigCRs R4-2417985 and R4-2417986 was not implemented to the specifications after RAN4#113 as MCC focused on other CRs and wanted to postpone the implementation of these.

Observation 5: All previous endorsed changes are included in the agreed BigCRs R4-2500714 and R4-2500715 from RAN4#114.

Proposal 1: RAN4 shall ensure R4-2500714 and R4-2500715 are implemented to the TSs including all already endorsed agreements related to simultaneous Rx-Tx.

Observation 6: With agreements from the WF at RAN4#114 and the existing framework for conditional mandatory support of simultaneous Rx-Tx all types of CA configurations seem covered.

Observation 7: RAN2 is working on needed capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

Proposal 2: RAN4 shall ensure RAN2 understands the need for a capability with indication of exceptions to mandatory support of simultaneous Rx-Tx for FDD-FDD combinations.

Proposal 3: RAN4 shall discuss whether indication of mandatory support of simultaneous Rx/Tx are needed for other than two-band BCs.

- References

- R4-2316097, Discussion on Simultaneous Rx/Tx Notes, Nokia

- R4-2317576, WF on simultaneous Rx-Tx requirements, Apple

- R4-2320020, Discussion on Simultaneous Rx/Tx, Nokia

- R4-2321992, WF on simultanesou Rx-Tx requirements, Apple

- R4-2403606, WF on simultaneous Rx-Tx issues, Huawei.

- R4-2402087, Discussion on Simultaneous Rx/Tx, Nokia

- R4-2406577, draftCR to 38.101-1 Update to Simultaneous Rx/Tx Notes, Nokia

- R4-2409193, CR to 38.101-1 Update to Simultaneous Rx/Tx Notes, Nokia

- R4-2412541, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2414266, Draft CR to 38.101-1 to clarify simultaneous Rx/Tx applicability for FDD-TDD combinations, Apple

- R4-2414267, Draft CR to 38.101-3 to clarify simultaneous Rx/Tx applicability for band combinations, Apple

- R4-2416359, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2420505, drafCR to 38.101-1: On Rel-19 simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2417985, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2417986, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2500714, BigCR to 38.101-1: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2500715, BigCR to 38.101-3: Simultaneous Rx-Tx, Huawei, HiSilicon

- R4-2502840, WF on simultaneous Rx-Tx, Huawei

- R4-2504124, On LS response to RAN1 on UE capability for non-simultaneous RxTx FDD-FDD inter-band CA, Nokia

- R4-2501993, On simultaneous RxTx requirements, Skyworks Solutions, Inc., Nokia.