3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504128

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	Discussion on RF requirements for Fragmented CA

Agenda item:	7.5.3

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below:

#### Issue 3-3: Discussion on in-gap interference requirements

- For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC.

- For image of in-gap interference (ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC.

# Open issues

### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

### Discussion on semi-statically switch hardware resources procedure and fallback considerations

### Triggering condition to enable ”One RF Rx chain”

FFS:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

### Discussion on applicability for PCell and Scell

### Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

Discussion

Receiver requirements

REFSENS considerations

In case the fragmented carrier configuration includes PCC operation, the DL operation is expected to at least experience the same receive implementation complications as found in non-contiguous intra-band carrier aggregation. It has been determined that placing a DL intra-band carrier (SCC) closer to the uplink carrier, than the normal duplex distance between the uplink and downlink carrier, will lead to degradation in receiver sensitivity of the DL intra-band carrier (SCC), as the uplink emission leaks into the receiver band, despite isolation in the duplex filter between RX and TX ports and between Rx and Tx components of the front-end.

Support of fragmented carrier configurations imply similar receive implementation complications as for non-contiguous intra-band carrier aggregation.

When it comes to the analogue filter bandwidth, if it fits the carrier bandwidth, or if it covers two (or more) downlink carriers of separated location, the filter is not providing any protection towards the uplink interference. This is because the uplink interference is already added within the duplex filter and between the front-end components, so filtering at the baseband signal has no impact to the uplink interference. The uplink interference is present inside the downlink carrier bandwidth.

Creating rules for PCC placement may be limiting for operators, and the discussion and agreements should already have concluded at NC IB CA study that such configuration imposes issues for the UE reception. Therefore, we see no reason to pursue the consideration to discuss PCC placements any further, but also stress that the requirements for ΔRIBNC remains as they are in NC IB CA.

RAN4 should not consider specific relaxation of ΔRIBNC for fragmented carrier operation but apply NC IB CA requirements.

When it comes to aspects of semi-statically switching the hardware resources, we see no need to discuss such procedures in relation to the impact of the UE uplink interference, since any new procedure for improving the receiver conditions are applicable as well for the non-contiguous intra-band DL operation. Currently the focus on the study is on DL operation. However, it would be recommended also to consider the UE uplink interference. Hence, we encourage to make a separate study that more generally looks at aspects of UE uplink interference, like MSD.

RAN4 shall consider UE uplink interference when applying fragmented carrier operation impact to MSD

Wanted and unwanted CC ratio

In Fig. 1 we show how the Rx images surrounding the LO has 25dB suppression (in blue) for the fragmented carriers, 25dB suppression for the in-gap interference (in grey) and how the 25dB in red will not be achievable unless the in-gap interference strength subsides the fragmented carriers.

25dB

25dB

25dB

Fig.  1: Image interference dependent on LO placement and scheduling

The network is not aware of the in-gap interference, nor is the UE without a supporting scheme to measure the in-gap interference. This is discussed further in [2].

The network is not aware of the in-gap interference, nor is the UE without a supporting scheme to measure the in-gap interference.

In Fig. 1 two options are shown where scheduling restriction and channel reconfiguration is applied for avoiding image interference of the in-gap interferer. Both examples concern the carrier of the largest bandwidth, but it may be dependent whether the reconfiguration is applied, or restrictions is done for the carrier of larger bandwidth. If the scheduling restriction is applied, the larger analogue filter bandwidth remains, while applying the channel reconfiguration allows an analogue filter bandwidth lesser than in the previous case. Lesser analogue filter bandwidth increases adjacent channel selectivity outside the total configured carrier bandwidth.

Scheduling restrictions may be considered to avoid LO shifts when operating with fragmented carrier CA

From Fig. 1 we also see that any increased power of the in-gap interferer beyond the power of the wanted carriers increases the image interference level at the same ratio. So as the in-gap interference becomes stronger than the wanted carriers the signal-to-interference and noise ratio (SINR) also decreases for the sub-carriers the image interference frequency range overlap inside the larger carrier of the wanted signal. An agreement and performance of a UE to tolerate 25dB image interference (of wanted signal) therefore indicates that levels exceeding the SINR will also impact the receiver quality. This impact happens exactly as the in-gap interference power becomes stronger than the wanted signal.

The image interference will become an issue for the UE reception but is only present when the wanted fragmented carriers are of unequal bandwidth.

The image interference will become an issue for the UE reception but is only present when the wanted fragmented carriers are of unequal bandwidth.

When it comes to aspects of semi-statically switching the hardware resources, we see potential to switch states of the UE configuration based on two factors:

- The UE has been configured to receive fragmented carriers of unequal carrier bandwidth.

- The UE monitors the strength of the in-gap interference.

Fallback options include:

- Scheduling restrictions that require no LO shift (fragmented carrier bandwidth remains).

- Channel reconfiguration that may change the UE center frequency and reduce the fragmented carrier bandwidth.

- Single carrier fallback.

Agreements of semi-statically switching the configurations may reduce the need to define receiver sensitivity requirements for a UE operation of having image-interference, where both image interference suppression ratio will have to be agreed, and in-gap-interference levels needs to be agreed.

RAN4 shall further discuss the fallback behaviour of the UE such the network efficiently can utilize the fragmented carrier capability.

ACS and IBB relaxations

As presented in [3], the adjacent channel and in-band blocking interference is of a much higher power level than the wanted carrier in the RF requirements. In the previous section we established that the image interference of the in-gap interference exceeds image interference from the wanted carriers, when the in-gap interference becomes stronger than the wanted carrier power.

Therefore, if we should consider requirements for the UE receiver in a state of fragmented carriers of un-equal carrier bandwidth the test becomes dominated by the image interference of the in-gap interference and not really tests the UE for ACS or IBB as such. Therefore, it does not make sense to discuss ACS and IBB in conditions of un-equal carrier bandwidths of the fragmented carriers, because it becomes a test for image interference of the in-gap interference rather than a test of the tolerated level of the in-gap interference.

RAN4 shall not consider ACS and IBB in conditions of un-equal carrier bandwidths of the fragmented carriers, because it becomes a test for image interference of the in-gap interference rather than a test of the tolerated level of the in-gap interference.

Number of RX in fragmented CA

Since the UE may support the bands n2/n25, n3, n26 and n66 using more than the mandated number of antennas in the downlink, we repeat that a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers. The following Fig.  5 shows a UE using four Rx RF chains for receiving fragmented carriers using the wider analogue filter.

Fig.  5: Single filter fragmented carrier scenario

Considering the UE is for some bands only mandated to receive using a minimum of 2Rx antennas, a UE may divide the use of the Rx RF chains among the antennas in a way possible to support the fragmented carrier operation using the same number of Rx RF chains. This allows the UE to improve the Rx performance by applying the analogue channel filter matching the respective channel bandwidth. An example of this is shown in Fig. 6, where 2Rx is configured for each fragmented carrier with a channel filter matching the carrier bandwidth.

Fig.  6: Selective filter fragmented carrier scenario

The study can consider the aspect whether a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers.

There are several UE implementation aspects involved in the configurations within this study. In this section we have brought attention to these. Since there is a variety of ways to support fewer Rx RF chains, we want to encourage RAN4 to identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported. These mechanisms can be thresholds, measurements, network managed transitions or UE events that alter the UE configuration to comply with the non-contiguous intra-band CA receiver requirements, when entering, operating or exiting the fragmented carriers operational state.

RAN4 shall identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: Support of fragmented carrier configurations imply similar receive implementation complications as for non-contiguous intra-band carrier aggregation.

Proposal 1: RAN4 should not consider specific relaxation of ΔRIBNC for fragmented carrier operation but apply NC IB CA requirements.

Proposal 2: RAN4 shall consider UE uplink interference when applying fragmented carrier operation impact to MSD

Observation 2: The network is not aware of the in-gap interference, nor is the UE without a supporting scheme to measure the in-gap interference.

Observation 3: Scheduling restrictions may be considered to avoid LO shifts when operating with fragmented carrier CA

Observation 4: The image interference will become an issue for the UE reception but is only present when the wanted fragmented carriers are of unequal bandwidth.

Proposal 3: RAN4 shall further discuss the fallback behaviour of the UE such the network efficiently can utilize the fragmented carrier capability.

Proposal 4: RAN4 shall not consider ACS and IBB in conditions of un-equal carrier bandwidths of the fragmented carriers, because it becomes a test for image interference of the in-gap interference rather than a test of the tolerated level of the in-gap interference.

Proposal 5: The study can consider the aspect whether a UE may trade off the number of Rx antennas as a means for using fewer Rx RF chains when receiving fragmented carriers.

Proposal 6: RAN4 shall identify the mechanisms that surround the UE implementation, i.e. when fragmented carrier operation can be supported.

- References

- R4-2502873, WF on NR FR1 DL fragmented carriers study, RAN4#114

- R4-2504173, Discussion on methods for reducing the number of UE Rx chains, Nokia

- R4-2501662, Discussion on requirements for Fragmented CA, Nokia