3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504134

Wuhan Meeting, Apr. 7th – Apr. 11th, 2025

Title:	TP to TR 38.719-02-01 CA_n2-n48

Source:	Nokia, Verizon

Agenda item:	6.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n2A-n48B with CA_n2A-n48B in uplink BCS 4&5. Note that this TP updates the existing entry in the TR since UL configuration CA_n2A-n48B is added.

************************************* Start of TP*****************************************

## 5.4		CA_n2-n48

### 5.4.1	Common for 1 band UL and 2 bands UL CA

#### 5.4.1.1	Operating bands for CA

Table 5.4.1.1-1:  CA band combination of band n2+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

#### 5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n2+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band |  | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n48A | CA_n2A-n48A | n2 |  | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 |  | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n2A-n48B | CA_n48B CA_n2A-n48A |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS2 |  |
|  | CA_n48B CA_n2A-n48A CA_n2A-n48B |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS 4 and 5 |  |
| CA_n2(2A)-n48B | CA_n48B CA_n2A-n48A CA_n2A-n48B |  | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS 4 and 5 |  |

#### 5.4.1.3	UE co-existence studies for 1 band UL

Table 5.4.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2-n48.

Table 5.4.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n2 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n48 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 3550 | 3700 | N/A | N |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a near miss collision between the band n2 2nd harmonic to the band n48 DL. |  |  |  |  |  |

Based on Table 5.4.1.3-1:

For the n2 uplink 2nd harmonic falling to the band n48, the MSD defined in Table 7.3A.4-1 of 38.101-1 can be applied. There is no newer requirement needed.

Table 5.4.1.3-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.4.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 3550 | 3700 |
| fDL (MHz) | 1930 | 1990 | 3550 | 3700 |
| CBW (MHz) | 5 | 40 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 3050 | 4200 |
| Analysis | There is no overlap of the band n2 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n2 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n2 so no cross-band MSD is needed. |  |

Based on Table 5.4.1.3-2:

#### There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

#### 5.4.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n48, the ∆TIB,c and ∆RIB,c values have already been specified.

#### 5.4.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.4.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.4.2	Specific for 2 bands UL CA

#### 5.4.2.1	Maximum output power for inter-band CA

Table 5.4.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n48B | 23 | +2/-3 |

#### 5.4.2.2	UE co-existence studies for 2 bands UL

Table 5.4.2.2-2 lists Band n2 + Band n48 2 bands UL CA(3CC) triple beat IMD analysis for Band n2- Band n48

Table 5.4.2.2-2: Band n2 and Band n48 for 3CC UL IMD products

| Band / CA1 | n2A |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1850-1910 |  | 3550-3700 |  |
| fDL | 1930-1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1750-1850 |  | 1910-2010 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### Based on Table 5.4.2.2-2:

The bands are not of the same or adjacent band groups, so Note 1 applies.

#### 5.4.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

************************************* End of TP******************************************