3GPP TSG-RAN WG4 Meeting #114bis									R4-2504150

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.14.3.1

Source:	CAICT

Title:	Discussion on TRP TRS requirements

Document for:	Approval

1	Introduction

In this contribution, we provide our views on Rel-19 TRP TRS requirements.

2	Discussion

2.1 2Tx lab alignment activity

According to Work Plan for R19 OTA WI [1], the 2Tx lab alignment activity for 2Tx non-coherent UE was originally targeted for completion in this meeting, with subsequent measurement campaign and requirements specifications to commence in the May meeting.

While RAN4 has planned two LADs for this activity [2], notable progress divergence exists between n41 and n78 bands.

- N41: LAD5 supporting n41 band achieved test readiness several meetings before, CAICT test lab has completed testing of LAD5 two meetings ago. The current progress for other test labs for this LAD requires rapporteur-level confirmation and coordination.

- N78: Due to various issues such as changes in device providers, device connection and testing configurations, the alignment activity for the n78 frequency band has not been performed in volunteer labs, and the overall progress is behind schedule.

| Rel-19 2Tx Lab Alignment Devices (LADs) for non-coherent UL MIMO |  |  |  |  |
| --- | --- | --- | --- | --- |
| DUT | UL-MIMO Configuration | Provider/Owner | Contact | issue |
| LAD5 (new device for Rel-19 2Tx) | TPMI index 2 | vivo | Ruixin Wang ruixin. wang@vivo.com | Only support n41. |
| LAD6 (new device for Rel-19 2Tx) | TPMI index 2 | Xiaomi |  | For n78 testing. Checking configuration |

If the alignment across labs cannot be confirmed, volunteer labs cannot proceed with subsequent measurement campaigns.

Observation 1: The overall 2Tx lab alignment activity progress is behind schedule of the Work Plan for R19 OTA WI due to multiple reasons.

Proposal 1: Focus on completing 2Tx lab alignment on n41 first to control schedule risk. Rapporteur should coordinate with all volunteer labs to report progress on this activity.

Proposal 2: Further decide whether to remove the 2Tx lab alignment for n78 in this release based on progress in May meeting.

TRP and TRS testing for 2Tx non-coherent UEs is a relatively new test case for OEMs, TE vendors, and test labs, and may encounter various issues during practical implementation. According to the Working Procedure for Rel-19 2Tx Lab Alignment and UE Performance Measurement Campaign [3], this alignment and measurement campaign activity only requires testing and reporting TRP/TRS performance under TPMI=2 configuration, without needing results from other TPMI indexes. However, vendors and test labs should check whether the DUT is operating correctly in the intended state.

During previous debugging efforts, we observed that when the base station emulator was correctly configured with TPMI index 2 and the UE successfully camped on the network to complete testing, the UE might not actually operate in the corresponding "1-layer 2-port" simultaneous transmission state. Specifically, the measured radiation patterns under this configuration were nearly the same to those of single-port transmission (TPMI=0/1). Further log analysis confirmed that despite the TPMI=2 configuration, the UE did not activate the expected dual-port transmission mode. Notably, this phenomenon has been observed in more than one device model during debugging. The root cause remains unclear, but one potential explanation is that low data volume triggers network down-scheduling to allocate only single-stream data transmission.

If the above situation occurs during the lab alignment and measurement campaigns, it will result in test data not meeting the expected performance of 2Tx UEs, thereby affecting the performance requirement definition. Therefore, it is kindly recommended that DUT providers/test labs pay attention to troubleshooting related issues in subsequent activities to ensure that the tested devices operates in the expected working state.

Observation 2: Test experience has shown that when TPMI=2 is configured, more than one UE model fails to activate dual port transmission (1-layer 2-ports) as required, resulting in test results deviating from the expected performance of 2Tx UE with dual-port transmission.

- Proposal 3：It is kindly recommended that DUT providers/test labs pay attention to troubleshooting related issues in subsequent activities to ensure that the tested devices operates in the expected working state.

2.2 1Tx Measurement campaign and target bands

Per working procedure for Rel-19 Measurement Campaign to define requirements [1], commercial device selection criteria is illustrated as below:

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign: - DUT size: Size 1(width >72mm and ≤92mm) - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands - The following selection criteria can also be considered: - Year of production: [2022-2025] - Brand variety - Price range (to capture different price segment, including High/Mid/Low-end products) - Popularity - Number of bands supported - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

The current commercial device selection criteria for the test campaign allow a four-year span (2022-2025) in production years. While device diversity benefits data pool representativeness, “old devices” may not accurately represent the performance of mainstream commercial devices currently available in the market, given advancements in RF front-end architectures and antenna designs.

- Proposal 4: Revise the year of production criterion for commercial device selection to [2023-2025].

At the last meeting, it was agreed to further discuss how to down select 4 bands from n3, n5, n7, n8 and n20, and encouraged companies to provide feedback on available number of devices [2]:

Issue 4-2-3: band number for 1Tx requirements in Rel-19 Agreements: - RAN4 further discuss how to select 4 bands from n3, n5, n7, n8, and n20, encourage companies to provide feedback by RAN4 #114bis on available number of devices each band and provide devices to test labs for data collecting.

According to the GSA 5G Device Ecosystem Report (September 2024), the candidate frequency bands with the top four device support are n3, n5, n8, and n7 (ranked in descending order). Based on this data, it is recommended to prioritize these four bands for defining the 1Tx requirements in Release 19.

Fig. 1.  Announced 5G spectrum support, number of models by band, for bands supported by 500 devices or more (data not available for all devices). Quoted from GSA 5G Device Ecosystem Report (Sep. 2024)

Proposal 5: It is recommended to select n3, n5, n7, n8 as four target bands for 1Tx requirements in Rel-19.

3	Conclusion

This contribution makes the following observations and proposals.

- Observation 1: The overall 2Tx lab alignment activity progress is behind schedule of the Work Plan for R19 OTA WI due to multiple reasons.

- Proposal 1: Focus on completing 2Tx lab alignment on n41 first to control schedule risk. Rapporteur should coordinate with all volunteer labs to report progress on this activity.

- Proposal 2: Further decide whether to remove the 2Tx lab alignment for n78 in this release based on progress in May meeting.

- Observation 2: Test experience has shown that when TPMI=2 is configured, more than one UE model fails to activate dual port transmission (1-layer 2-ports) as required, resulting in test results deviating from the expected performance of 2Tx UE with dual-port transmission.

- Proposal 3：It is kindly recommended that DUT providers/test labs pay attention to troubleshooting related issues in subsequent activities to ensure that the tested devices operates in the expected working state.

- Proposal 4: Revise the year of production criterion for commercial device selection to [2023-2025].vities to ensure that the tested devices operates in the expected working state.

Proposal 5: It is recommended to select n3, n5, n7, n8 as four target bands for 1Tx requirements in Rel-19.

References

- R4-2412056, Updated Workplan of Rel-19 FR1 OTA WI, vivo, CAICT

- R4-2502370, WF for [112][323] TRP_TRS_ph3, vivo

- R4-2415777, Working procedure for Rel-19 2Tx lab alignment and UE performance measurement campaign, vivo