3GPP TSG-RAN WG4 Meeting #114bis									R4-2504151

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.4

Source:	CAICT

Title:	Discussion on OTA test method for A-IoT device

Document for:	Approval

1	Introduction

In RAN4#114 meeting, following agreement were captured in the WF regarding OTA test method for A-IoT device1 [1]. In this contribution, we provide our further views on OTA aspects for A-IoT devices.

- Topic 4-1: General Issue 4-1-1: Scope of OTA test methodology Agreement: - For OTA test, the starting point of test methodology for Ambient IoT includes - Chamber setup - Position of Device - Test procedures - Minimum range/ QZ (Quiet Zone) size/QZ calibration - Preliminary MU assessment - Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop test methodology. Issue 4-1-2: Measurement campaign Agreement: - Measurement campaign is not possible in this work item. Refinement in future work items based on measurement campaign is not precluded. Issue 4-1-3: chamber Agreement: - Use anechoic chamber as baseline for device1 test in Rel-19 Issue 4-1-4: Energy source Agreement： - Further discuss the energy source issue for device 1. - Topic 4-2: Test method and configurations Issue 4-2-1: Test configuration Agreement： - Test configuration is for a single A-IOT device under free space condition Issue 4-2-2: Device positioning guidance Agreement： Take following as a starting point and further discuss the details. - The traditional test system of AC for handheld UE radiated testing can be considered as the starting point for A-IoT device testing. - The following test setup parameters shall be defined for A-IoT device OTA testing: - DCW2D: distance between CW node and DUT - DMA2D: distance between measurement antenna and DUT - IsoCW2M: Isolation between CW node and measurement antenna

2	General views on OTA test method for A-IoT

- Test setup

It was agreed in last meeting to use Anechoic chamber as baseline for device1 test in Rel-19 and consider traditional test system for handheld UE radiated testing as the starting point for further discuss. To reduce testing complexity, it is recommended to reuse existing OTA testing systems and methods as much as possible.

Proposal 1: Reuse 30cm QZ size, minimum range length and Ripple test procedure defined in TR38.870. FFS Calibration procedure for A-IoT OTA test system.

It is also agreed that the following test setup parameters shall be defined for A-IoT device OTA testing:

  - DCW2D: distance between CW node and DUT

  - DMA2D: distance between measurement antenna and DUT

  - IsoCW2M: Isolation between CW node and measurement antenna

Proposal 2: Use minimum range length for 30cm QZ as the requirement for DCW2D and DMA2D.

- Measurement grid

In OTA measurement, the traditional fine grid is defined as 15º for TRP and 30º for TRS. However, 3GPP also allowed a coarser measurement grid for some test cases which is 30º for TRP and 45º for TRS to optimize testing efficiency while maintaining acceptable accuracy.

Given that A-IoT devices are likely to utilize omnidirectional antennas, the resolution requirements for the measurement grid can be relaxed. Fine grids are typically adopted to capture the rapid gain variations of directional antennas, while omnidirectional antennas have smaller changes, which means that there is no need for fine measurement grids like high directional antennas during testing. To reduce testing complexity and time, it is recommended to adopt coarser grid configurations for A-IoT testing, such as a 30° grid for power measurements and 30°/45° grids for sensitivity measurements.

Proposal 3: It is recommended to adopt coarser grid configurations for A-IoT testing, such as a 30° grid for power measurements and 30°/45° grids for sensitivity measurements.

Note: This proposal assumes A-IoT OTA testing requires spatial scanning (either full-sphere or hemisphere scanning ). If the test only requires directional metrics at manufacturer-declared orientations (e.g., peak gain direction) without scanning, grid resolution becomes irrelevant.

- Energy source

The test system need to support device charging functionality due to limited energy storage and address the unique energy supply mechanism of A-IoT Device.

For A-IoT testing, it is important to understand how to charging the DUT, such as the required charging power, charging time slot and duration, when the device is awakened, and the duration of continuous operation. Considering that energetic behavior are out of the scope of the 3GPP discussion, and that the details of energy supply may be up to UE implementation, it is necessary to request the device vendor to provide the testing lab with specific methods and procedure for supplying energy to the device to ensure the smooth execution of testing and the accuracy of results.

Proposal 4: It should be stated in the spec that device vendor should provide the testing lab with detailed method and procedure for supplying energy to the device to ensure the smooth execution of testing and accuracy of the results.

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: Reuse 30cm QZ size, minimum range length and Ripple test procedure defined in TR38.870. FFS Calibration procedure for A-IoT OTA test system.

- Proposal 2: Use minimum range length for 30cm QZ as the requirement for DCW2D and DMA2D.

Proposal 3: It is recommended to adopt coarser grid configurations for A-IoT testing, such as a 30° grid for power measurements and 30°/45° grids for sensitivity measurements.

Proposal 4: It should be stated in the spec that device vendor should provide the testing lab with detailed method and procedure for supplying energy to the device to ensure the smooth execution of testing and accuracy of the results.

References

- R4-2502858, WF on requirements for system parameters and A-IOT device，CMCC, 3GPP RAN4 #114.