# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504152
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.1.1.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1].

Issue 1-1-1: Boundary to apply SE - Options - Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW - Option 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz - WF - option 1, to meet the ITU recommendation Issue 1-1-2: Integral region basis for OOBE - Options - Option 1: The OOBE requirements are based on UE CBW - Option 2: The OOBE requirements are based on extended UE CBW OP 1. OP 2. - WF - option 1 Issue 1-1-3: A-MPR handling - Proposals - Option 1: - If A-MPR is specified for a given RB allocation X, then the UE performs - max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs - max(MPR(X), A-MPR(X)) - Option 2: - For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which A-MPR(X)≥minX∈SMPR(X) where S is the set of all possible RB allocations for the given carrier, then the UE performs max(MPRenh(X), A-MPR(X)) - Option 3: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation. - Others - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW Issue 1-1-4: How to derive the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified with ΔRB =1 or ΔRB =0. FFS the specific offset value. - IBE is not used in the GB - Option 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions (Moderator: NRB,V and LCRB,V could be replaced by other terms, but the concept is similar) max-25-10⋅log10(NRB,V/LCRB,V), 20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V, -57dBm+10log10SCS/15kHz-PRB - Option 3: Others - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW Issue 1-1-5: Modulation order for the extended IBE - Proposals - For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. Table. Comparison of emission level @ the first 1MHz adjacent to the UE CBW UE CBW (MHz). 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. SEM (dBm/30kHz); -18.2; -20.0; -21.2; -22.2; -23.0; -23.6; -24.2; -24.8; -24.0. IBE QPSK 26dBm (dBm/30kHz); -16.7; -18.7; -20.0; -21.0; -21.8; -22.5; -23.2; -23.7; -24.1; -25.0; -25.7; -26.3; -26.8; -27.2. IBE 16QAM 26dBm (dBm/30kHz); -19.6; -21.6; -22.9; -24.0; -24.8; -25.5; -26.1; -26.6; -27.1; -28.0; -28.6; -29.2; -29.7; -30.2. IBE BPSK 26dBm (dBm/30kHz); -12.0; -14.0; -15.3; -16.4; -17.2; -17.8; -18.5; -19.0; -19.5; -20.3; -21.0; -21.6; -22.1; -22.6. - WF - FFS whether the extended IBE with high order EVM could have impact on MPR for new inner RB allocation Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible Issue 1-2-2: How to get the width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed value, i.e., 0.5* UE CBW is stipulated in the spec - Option 2: Flexible value signalled by the NW, the extended CBW at each side ≤ 0.5* UE CBW but not necessarily the one in Table 5.3.2-1 in TS 38.101-1 - Option 2a: Flexible value signalled by the NW, the extended CBW at each side should be the CBW specified in Table 5.3.2-1 in TS 38.101-1 - Option 3: Based on SIB1 cell specific CBW - WF - FFS in next meeting Issue 1-2-3: Guard band for extended CBW - Options - Option 1: if extended CBW at each side is from CBW in Table 5.3.2-1 in TS 38.101-1, the RB number is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, GB is determined accordingly - Option 2: guard band for extended CBW is derived from the total RBs including the extended ones, which is larger than the larger guard band between: - The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB). - The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB) - Option 3: For fixed 0.5* UE CBW case, The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW. - ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW. - Others - WF - FFS in next meeting Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. - If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) - If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_high - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting Issue 1-3-1: NW signaling - Proposals - Proposal 1: (NTT DOCOMO) - BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature. - Proposal 2: (MediaTek) - Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, in order to allow mobile operators the choice of whether to allow it or not. - Proposal 3: (vivo) - Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered: - It is up to the NW to judge the gap width on both sides of the UE. - When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side. - When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides. - Proposal 4: (Skyworks) - Proposal on RB extension: - Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH - Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW. - Inner and outer allocation can be redefined by the UE, using the existing inner/outer allocation equations where: - NRB is replaced by NRB+RBextL+RBextH - RBstart is replaced by RBstart+RBextL - With 273RB for 100MHz CBW, the maximum RB extension value is 137, thus can be signalled with 8 bits, corresponding to 16bit total for the lower and higher side RB extensions. For 15kHz SCS cases, 50MHz has NRB/2=135, thus the same number of bits. - Note that for the 100MHz case in 30kHz SCS and 50MHz in 15kHz SCS, the RB extension would correspond to BS bandwidth cases with DL CA or multiple channels (for example band n41 100+90MHz case in the US, or 100+60MHz case in China) - several unused codes can be used as “short-cut” signalling: for example, all 1s would mean that all allocations can be turned to inner. - Others - WF - FFS whether NW signalling need to consider different requirements or just a function enabler without considering specific requirements Issue 1-3-2: UE capability - Proposals - Proposal 1: - If the UE reports the capability supporting MPR reduction with extended CBW, e.g. [MPR improvement with BW extension-r19], the NW decides whether to allow such feature with specific signaling. - Proposal 2: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability (Sony) - Proposal 3: Define separate UE capabilities for scenario 1 and scenario 2 (Nokia) - Proposal 4: (Skyworks) - A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered: - One where the UE is able to reassign outer/edge to inner, based on 1RB granularity of signalled RB extension. - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs). - Others - WF - MPR reduction with extended CBW is an optional feature with UE capability - FFS details of UE capability

This contribution further discusses power domain enhancement for single carrier.

- Discussion

In previous meeting, it was discussed how to extend IBE for usage between edges of UE CBW and extended UE CBW, and in last meeting the issue focused on whether the extended IBE can be used in the original GB of UE CBW and/or the GB of extended CBW.

Actually, the IBE for the original UE CBW, the unwanted emission falling into non-allocated RB(s) is calculated after the FFT as described below:

Based on the IBE test procedure, no matter what extend IBE is used between edges of UE CBW and extended UE CBW, the unwanted emission falling into non-allocated RB(s) which is located in the region between edges of UE CBW and extended UE CBW should also be calculated after FFT.

Proposal 1: The extend IBE for usage between edges of UE CBW and extended UE CBW should be calculated after FFT.

There will have one issue that the GBs between edges of UE CBW and extended UE CBW may not be the integer multiple of RB.

If the test equipment uses one FFT to do the time-frequency conversion for the signalling and emissions in extended UE CBW, the FFT sampling points may not align with the assigned RBs in UE CBW due to the inner GBs, it will cause the equipment can’t sample the wanted signalling correctly as shown in figure 2-1:

Figure 2-1 One FFT for extended UE CBW to test extended IBE

So, the test equipment needs three FFT to do the time-frequency conversion in extended UE CBW due to the presence of the inner GBs as shown in Figure 2-2:

Figure 2-2 Three FFT for extended UE CBW to test extended IBE

In principle, the GBs of UE CBW is to prevent interference of adjacent channels and ensure sufficient isolation between channels.

Now we research the MPR reduction is under the conditions where no co-existence issue is caused especially for the deployment of two adjacent frequency blocks.

Therefore, the inner GBs is unnecessary to reserve for extended UE CBW.

Since how to extended RBs between edges of UE CBW and extended UE CBW only impacts the extended IBE, to ensure the flexibility of the test equipment implementation, it’s better to keep the extended RBs and transmission bandwidth configuration NRB of UE CBW are continuous.  That is, the extended RBs on each side start from the edge of the transmission bandwidth configuration NRB of UE CBW as shown in figure 2-3:

Figure 2-3 The GBs and extended RBs of extended UE CBW

Proposal 2: The extended RBs on each side start from each edge of the transmission bandwidth configuration NRB of UE CBW.

And to keep the original UE CBW unchanged, the original GB of UE CBW can be moved to outside of the extended RBs on each side of UE CBW, the extended IBE don’t apply to the outside GBs.

Proposal 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW.

Proposal 4: The GBs of extended UE CBW reuse the size of original GB of UE CBW.

Proposal 5: The extended IBE requirements don’t apply to the GBs of extended UE CBW.

About how to derive the extended IBE, we verify the impact of extended IBE to MPR through MPR simulations based on below case1 and case2. To facilitate compare the difference, we set the extended UE CBW is less than 1.5 * UE CBW.

Case1: There is no extended IBE in extended RBs as shown in Figure 2-4:

Figure 2-4 Case 1 without extended IBE in extended RBs

Case 2: There is the extended IBE calculated by the general function of IBE in extended RBs as shown in Figure 2-5:

Figure 2-5 Case 2 with extended IBE in extended RBs

- MPR without extended IBE in extended RBs                   b. MPR with extended IBE in extended RBs

Figure 2-6 the MPR without extended IBE in extended RBs

Compared the MPR results of Case 1 and Case 2 as shown in figure 2-6, we can see there is no any difference due to the extended IBE is added. That is, the extended IBE is not the limited factor for the outer MPR converting into inner MPR. Therefore, it is unnecessary to consider how to relax the extended IBE requirements. Considering the brevity and readability of the Spec, it is better to extend the general IBE requirement of the UE CBW by replacing NRB with NRB,E.

Proposal 6: The extended IBE requirements should be an extension of the IBE requirement of the UE CBW by replacing NRB with NRB,E as below functions:

max-25-10⋅log10(NRB,E/LCRB),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,
-57dBm+10log10SCS/15kHz-PRB

Where NRB,E=NRB,E,L +NRB,E,R+ NRB, NRB,E,L is the extended RBs on left side of UE CBW, NRB,E,R is the extended RBs on right side of UE CBW.

In previous meeting, we provided the related simulation results in [2] research the relationship of the MPR reduction and the extended UE CBW. And we got that:

- When extended UE CBW is 2 times of the UE CBW, all outer RB allocation converts to inner RB allocation and the MPR can be improved up to 2.5dB.

- When extended UE CBW is 1.5 times of the UE CBW, the MPR can still be improved up to 2dB although not all outer RB allocation are converted to inner RB allocation.

- When extended UE CBW is 1.25 times of the UE CBW, only small part of regions for outer RB allocation can get improvement.

That is, the MPR won’t reduce further when the gap between the edge of virtual CBW and the edge of configured UE CBW goes beyond 0.5*UE CBW on each side.

To further verify that whether it can convert all outer MPR into inner MPR under the conditions of the GB of extended UE CBW reuse the GB of UE CBW and the extended RBs on each side of UE are ceil (0.5*NRB), we further do the simulation using 10MHz UE CBW with 30kHz SCS and extended RBs on each side of UE CBW are 12RBs for PC2, the result are shown in figure 2-7 that all outer MPR were converted into inner MPR.

Figure 2-7 the MPR for 12 extended RBs on each side of UE CBW with GB of UE CBW

Proposal 6: Limited the largest extended RBs on each side of UE CBW are ceil (0.5*NRB) and the largest extended UE CBW will be UE CBW+2* ceil (0.5*NRB), and NRB is the maximum number of RBs for UE CBW.

In fact, as long as the extended UE CBW is larger than the UE CBW, some outer MPR will be converted to inter MPR, regardless of whether the extension is symmetric or asymmetric. The flexible extended should be allowed, and it is unnecessary to extended the RBs by 1RB step, the least extended RBs and the extended step can be limited, like the least extended RBs are larger than 10RBs and the step is 10RBs.

Proposal 7: Flexible extended RBs on each side are allowed and the extended RBs number are signalled by the NW, the extended RBs at each side can be X RBs ≤ NRB,E,L/NRB,E,R ≤ 0.5* ceil (0.5*NRB) but not necessarily the one in Table 5.3.2-1 in TS 38.101-1, the least extended RBs X can further discuss and the extended step can be limited.

Based on above analysis, the extended UE CBW will equal to UE CBW + NRB,E,L +NRB,E,R and the GB of the extended UE CBW reuse the GB of UE CB, how to deriving the new inner RB region with RBs in extended CBW is a question. Below simulation cases is to verify whether it can reuse the existing inner RB allocation conditions of UE CBW.

Case 3: The OOBE requirements are based on UE CBW

Case 4: The OOBE requirements are based on extended UE CBW

To facilitate compare the difference, we set the extended UE CBW is less than 1.5 * UE CBW.

- The OOBE requirements based on UE CBW           b. The OOBE requirements based on extended UE CBW

Figure 2-8 The MPR with different OOB requirements

Compared the MPR results of Case 3 and Case 4 as shown in figure 2-8, we can see the inner RB allocation of the OOBE requirements based on UE CBW are slightly larger than the inner RB allocation of the OOBE requirements based on extended UE CBW. Although deriving the inner RB allocation of the extended CBW based on the existing inner RB allocation conditions of UE CBW will slightly reduce the inner RB allocation. This will greatly simplify the definition of inner RB allocation, from this point, the small effects can be ignored.

Proposal 8: The conditions of inner RB allocation for extended UE CBW should be consistent with the conditions of inner RB allocation of UE CBW, and re-define based on the parameters of the extended UE CBW as below:

RBStart,Low,E  ≤  RBStart,E  ≤  RBStart,High,E, and

LCRB,E  ≤  ceil(NRB,E/2)

NRB,E is the maximum number of RBs for the extended UE CBW, and

RBStart,Low,E = max(1, floor(LCRB,E/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High,E = NRB,E – RBStart,Low,E – LCRB,E

If the flexible extended RBs is allowed, how to test the Tx requirements for extended UE CBW also need discuss. Since The new inner RB region with RBs in extended UE CBW are derived based on the same conditions as the original UE CBW.

The new inner RB region and new outer RB region due to flexible extended RBs should reuse the original MPR values of original inner RB region and original outer RB region, that is, there is no new MPR definition for the new inner RB region and new outer RB region based on the extended UE CBW.

The UE just need to judge the MPR enhancement region (the extended Inner RB region compared to the original inner RB region), and update the outer MPR values to inner MPR values for this enhanced region according to the NW configuration. The MPR enhancement region can be derived according to the original inner RB regions based on UE CBW and the new inner RB regions based on extended UE CBW.

Therefore, the test only needs verify the case that all outer RB allocations convert into inner RB allocations, that is, it just needs test the largest symmetrical extended case on each side of UE CBW.

Proposal 9: Only test the Tx requirements for the extended UE CBW that the extended RBs on each side of UE CBW are ceil (0.5*NRB).

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band, and proposed:

Proposal 1: The extend IBE for usage between edges of UE CBW and extended UE CBW should be calculated after FFT.

Proposal 2: The extended RBs on each side start from each edge of the transmission bandwidth configuration NRB of UE CBW.

Proposal 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW.

Proposal 4: The GBs of extended UE CBW reuse the size of original GB of UE CBW.

Proposal 5: The extended IBE requirements don’t apply to the GBs of extended UE CBW.

Proposal 6: Limited the largest extended RBs on each side of UE CBW are ceil (0.5*NRB) and the largest extended UE CBW will be UE CBW+2* ceil (0.5*NRB), and NRB is the maximum number of RBs for UE CBW.

Proposal 7: Flexible extended RBs on each side are allowed and the extended RBs number are signalled by the NW, the extended RBs at each side can be X RBs ≤ NRB,E,L/NRB,E,R ≤ 0.5* ceil (0.5*NRB) but not necessarily the one in Table 5.3.2-1 in TS 38.101-1, the least extended RBs X can further discuss and the extended step can be limited.

Proposal 8: The conditions of inner RB allocation for extended UE CBW should be consistent with the conditions of inner RB allocation of UE CBW, and re-define based on the parameters of the extended UE CBW as below:

RBStart,Low,E  ≤  RBStart,E  ≤  RBStart,High,E, and

LCRB,E  ≤  ceil(NRB,E/2)

NRB,E is the maximum number of RBs for the extended UE CBW, and

RBStart,Low,E = max(1, floor(LCRB,E/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High,E = NRB,E – RBStart,Low,E – LCRB,E

Proposal 9: Only test the Tx requirements for the extended UE CBW that the extended RBs on each side of UE CBW are ceil (0.5*NRB).

- Reference

[1] R4-2503013 WF on power domain enhancement, Huawei, RAN4 #114

[2] R4-2412351 R19 MPR reduction for single carrier, OPPO, RAN4 #112