# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504153
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.5.2

Source: 	OPPO

Title: 	Discussion on reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, the general issues on reducing the number of UE RC chains were discussed in WF [1], and some open issue need further discuss:

Open issues How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. - Other options are not precluded Discussion on semi-statically switch hardware resources procedure and fallback considerations Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition. Discussion on applicability for PCell and SCell Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

In this contribution, we further discuss the general issues on reducing the UE Rx chain for fragmented carriers.

- Discussion

In the SID, reducing the UE Rx chain for fragmented carriers only consider two non-contiguous CCs sharing one Rx chain within a CA combination, therefore, it’s enough to specify the capability as per-band per-BC.

First, we should first discuss what is the default capability for the UE supporting one Rx RF chain for fragmented carrier. For example, the UE supporting highest CA order is 3 NC CCs in some band with the new capability, does it mean the UE has 2 Rx chains or 3Rx chains, these Rx chains number don’t include the diversity Rx chain. In my understanding, it means the UE only has 2Rx chains, the network can only configure 2 NC CCs for the UE if the network didn’t enable the new capability. That is, the network need indicate the UE enable the new capability and indicate UE which existing CC will share the Rx chain with the new CC that the network will add for the UE. From this point, it’s enough to specify the capability as per-band per-BC.

Proposal 1: The UE supporting “one Rx RF chain” for DL fragmented carriers can use legacy band combination reporting scheme + new capability supporting “one Rx RF chain” indication.

Proposal 2: Specify the new capability per-band per-BC.

As above analysis, whether enabling one Rx RF chain for the UE supporting the new capability should be under the control of network. The UE only need switch hardware resource semi-statically once it received the enable signal. No triggering condition for UE working in one Rx RF chain is needed, the UE just treats this CA as a general CA and do the measurement on each wanted CC, if PCC is interfered, the network can remove SCC and make this CA fallback to single CC state; if PCC is interfered, the PCC can switch to other cell. Therefore, the triggering condition for UE working in legacy Rx RF chains is not needed. In additional, there is no RS in the gap, the UE can’t test the interference from RRM point.

Proposal 3: It only allows the UE supporting “one Rx RF chain” for DL fragmented carriers to switch from legacy Rx RF chains to one Rx RF chain semi-statically.

Proposal 4: It doesn’t allow the UE supporting “one Rx RF chain” for DL fragmented carriers to switch from one Rx RF chain to legacy Rx RF chains semi-statically.

Proposal 5: Triggering condition for UE working in one Rx RF chain and triggering condition for UE working in legacy Rx RF chains are not needed.

- Conclusion

This paper discussed the general issues for fragmented carriers and proposed:

Proposal 1: The UE supporting “one Rx RF chain” for DL fragmented carriers can use legacy band combination reporting scheme + new capability supporting “one Rx RF chain” indication.

Proposal 2: Specify the new capability per-band per-BC.

Proposal 3: It only allows the UE supporting “one Rx RF chain” for DL fragmented carriers to switch from legacy Rx RF chains to one Rx RF chain semi-statically.

Proposal 4: It doesn’t allow the UE supporting “one Rx RF chain” for DL fragmented carriers to switch from one Rx RF chain to legacy Rx RF chains semi-statically.

Proposal 5: Triggering condition for UE working in one Rx RF chain and triggering condition for UE working in legacy Rx RF chains are not needed.

Reference

[1] R4-2502873, WF on fragmented DL, MediaTek Inc., RAN4#114.