# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504154
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.5.3

Source: 	OPPO

Title: 	Discussion on impacts on UE RF requirements

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, the impacts on UE RF requirements and DL performance were discussed in WF [1]:

Agreements: Issue 3-1: Discussion on maximum input level for fragmented carriers - Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies evaluation TPs Issue 2-6: Baseband assumption - We do not need new agreements. Refer to WF R4-24020519 Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC. Issue 3-4: Discussion on DL fragmented carriers in TDD bands - Option 2: Separate LOs are assumed for UL and DL for TDD band

In this contribution, we further discuss the impact on UE DL RF requirements reducing the UE Rx chain for fragmented carriers.

- Discussion

In previous discussion, the common understanding is the impact of in-gap interferer and its image and IMDs associated with the interferer to the Rx requirements for PCC and SCC should be considered.

- The impact of Tx leakage in REFSENS test.

The Tx leakage will become obvious due to the suppression of the LPF with wider bandwidth become worse compared to two LPFs with narrower bandwidths. Maybe larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

Proposal 1: Larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

- The impact of in-gap blocker in ACS and IBB tests

In last meeting, it was agreed the effects of in-gap interference on ACS/IBB should be evaluated based on ΔRIBNC on PCC and SCC.

To further analyze the effects of in-gap interference on ACS/IBB, here given the assumption that [0-25dB] ΔRIBNC for both of PCC and SCC in band n25.

In current Spec, for DL intra-band NC CA with 2CC, each CC need meet the ACS and IBB requirements of the non-CA and keeping 2 DL CC activated.

Here excerpt the ACS and IBB requirements of the non-CA as below tables for frequency < 2700 MHz:

Table 2-1 ACS requirements of the non-CA for frequency < 2700 MHz

|  | Case 1 |  |  |  |  | Case 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BWwanted(MHz) | 5,10 |  | 15 |  | 20 | 5,10 | 15 | 20 |
| Pwanted(dBm) | REFSENS + 14 dB |  |  |  |  | -56.5 | -53.5 | -50.5 |
| Pinterferer(dBm) | REFSENS + 45.5 dB | REFSENS + 42.5 dB |  | REFSENS + 39.5 dB |  | -25 |  |  |
| BWinterferer | 5MHz |  |  |  |  | 5MHz |  |  |
| ACS (dB) | 33 | 30 |  | 27 |  | 33 | 30 | 27 |

Table 2-2 IBB requirements of the non-CA for frequency < 2700 MHz

|  | Case 1 |  |  | Case 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BWwanted(MHz) | 5,10 | 15 | 20 | 5,10 | 15 | 20 |
| Pwanted(dBm) | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| Pinterferer(dBm) | -56 |  |  | -44 |  |  |
| BWinterferer | 5MHz |  |  | 5MHz |  |  |

The REFSENs in above tables will be increased by ΔRIBNC for general DL intra-band NC CA, here using CA_n25A-n25A as an example, the channel bandwidth is 20MHz+20MHz, assume ΔRIBNC is 0/25dB for the SCC.

Table 2-3 The REFSENs for band n25

Here we summarized the parameters for ACS and IBBs test cases for each CC with 20MHz channel bandwidth considering ΔRIBNC as shown in Table 2-4:

Table 2-4 The existing parameters for ACS and IBBs test cases for n25 with 20MHz channel bandwidth

|  |  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) |  | -90.3+ΔRIBNC +14= -76.3 +ΔRIBNC | -50.5 | -90.3+ΔRIBNC +9= -81.3+ΔRIBNC | -90.3+ΔRIBNC +9= -81.3+ΔRIBNC |
| Pinterferer(dBm) |  | -90.3+ΔRIBNC +39.5= -50.8 +ΔRIBNC | -25 | -56 | -44 |
| BWinterferer |  | 5MHz | 5MHz | 5MHz | 5MHz |
| Pinterferer -Pwanted (dB) |  | 25.5 | 25.5 | 25.3-ΔRIBNC | 37.3-ΔRIBNC |
| Pinterferer -Pwanted (dB) | ΔRIBNC=0 | 25.5 | 25.5 | 25.3 | 37.3 |
|  | ΔRIBNC=25 | 25.5 | 25.5 | 0.3 | 12.3 |

Based on Table 2-4, it can get for general 2CC DL intra-band NC CA:

- For ACS, the power ratio of interference to wanted signal power is 25.5 dB, and independent of ΔRIBNC.

- For IBB, the power ratio of interference to wanted signal power is in range of 0~37.3dB, the lowest bound depends on ΔRIBNC.

Observation 1: For general 2CC DL intra-band NC CA without share RF chain:

- For ACS, the power ratio of interference to wanted signal power is 25.5 dB, and independent of ΔRIBNC.

- For IBB, the power ratio of interference to wanted signal power is in range of 0~37.3dB, the lowest bound depends on ΔRIBNC.

b-1: For image of in-gap interference not overlap with wanted CC

The in-gap interference not overlap with wanted CC will not impact the wanted signal, it will not impact the ACS requirements and IBB requirements.

Therefore, ACS requirements and IBB requirements won’t be change compared to general DL intra-band NC CA for the image of in-gap interference not overlap with wanted CC.

Proposal 2: In-gap interference won’t impact the ACS requirements and IBB requirements for the image of in-gap interference not overlap with wanted CC.

b-2: For image of in-gap interference (ACS/IBB) overlap with wanted CC

As the analysis in [2], there are two cases for ACS and IBB testing:

Case 1: when testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap blocker may also fall into co-channel of CC0 as illustrated in Figure 3.

Case 2: On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of blocker may fall into the co-channel range of CC1 as illustrated in Figure 4.

Here we excerpt the figure from [2] to further analyse the image impact.

Since the conformance test does not measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, here we only analyse case 1 and use band n25 as an example. And Table 2-3 is the REFSENs for band n25

We assumed CC0 is 20MHz, when testing in-gap ACS on CC0, the image of interferer will fall into CC0 as Figure 3, based on Table 2-4 assuming 25dB receiver image rejection, we can get the related image power of the interferer and the worsen power in transmission bandwidth configuration as shown in table 2-5:

Table 2-5 The parameters with the image impact for ACS and IBBs test cases for n25 with 20MHz channel bandwidth

|  |  |  | ACS |  |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Case 1 |  | Case 2 |  |  |
| Pwanted(dBm) |  |  | -76.3 +ΔRIBNC |  | -50.5 | -81.3+ΔRIBNC | -81.3+ΔRIBNC |
| Pimage |  |  | -75.8 +ΔRIBNC |  | -50 | -81 | -69 |
| Pwanted+Pimage (dBm) |  | ΔRIBNC=0 | -73 |  | -47.2 | -78 | -68.8 |
|  |  | ΔRIBNC=25 | -48 |  |  | -56.3 | -43.8 |
| Pinterferer(dBm) |  |  | -50.8 +ΔRIBNC | -50.8 | -25 | -56 | -44 |
|  |  |  |  | -25.8 |  |  |  |
| Pinterferer -(Pwanted+Pimage) (dB) | ΔRIBNC=0 |  | 22.2 |  | 22.2 | 22 | 24.8 |
|  | ΔRIBNC=25 |  | 22.2 |  |  | 0.3 | -0.2 |

Based on the calculation of Table 2-5, it can get:

- For ACS, the power ratio of interference to wanted signal power is independent of ΔRIBNC, it only depends on the receiver image rejection.

- For IBB, the power ratio of interference to wanted signal power are related to both od receiver image rejection and ΔRIBNC.

- For IBB, the worst case is Pwanted+Pimage equals to Pinterferer, the power ratio of interference to wanted signal power is 0dB.

Observation 2: For image of in-gap interference (ACS/IBB) overlap with wanted CC

- For ACS, ΔRIBNC doesn’t impact of the power ratio of interference to wanted signal power, it only depends on the receiver image rejection.

- For IBB, the power ratio of interference to wanted signal power are related to both of receiver image rejection and ΔRIBNC, can only be in the range of 0~ 25dB.

- For IBB, the worst case is Pwanted+Pimage equals to Pinterferer, the power ratio of interference to wanted signal power is 0dB, the lowest bound depends on ΔRIBNC.

Proposal 3: The power ratio of interference to wanted signal power should be evaluated separately for image of in-gap interference (ACS/IBB) overlap with wanted CC.

Proposal 4: For ACS, the power ratio of interference to wanted signal power is almost 22.2 dB for image of in-gap interference (ACS/IBB) overlap with wanted CC.

Proposal 5: For IBB, the power ratio of interference to wanted signal power can only be in the range of 0~ 25dB, for image of in-gap interference (ACS/IBB) overlap with wanted CC.

- Conclusion

This paper discussed the RF requirements for fragmented carriers and proposed:

Proposal 1: Larger REFSENS reduction is needed for fragmented carrier operation compared to the intra-band non-contiguous DL CA.

Observation 1: For general 2CC DL intra-band NC CA without share RF chain:

- For ACS, the power ratio of interference to wanted signal power is 25.5 dB, and independent of ΔRIBNC.

- For IBB, the power ratio of interference to wanted signal power is in range of 0~37.3dB, the lowest bound depends on ΔRIBNC.

Proposal 2: In-gap interference won’t impact the ACS requirements and IBB requirements for the image of in-gap interference not overlap with wanted CC.

Observation 2: For image of in-gap interference (ACS/IBB) overlap with wanted CC

- For ACS, ΔRIBNC doesn’t impact of the power ratio of interference to wanted signal power, it only depends on the receiver image rejection.

- For IBB, the power ratio of interference to wanted signal power are related to both of receiver image rejection and ΔRIBNC, can only be in the range of 0~ 25dB.

- For IBB, the worst case is Pwanted+Pimage equals to Pinterferer, the power ratio of interference to wanted signal power is 0dB, the lowest bound depends on ΔRIBNC.

Proposal 3: The power ratio of interference to wanted signal power should be evaluated separately for image of in-gap interference (ACS/IBB) overlap with wanted CC.

Proposal 4: For ACS, the power ratio of interference to wanted signal power can be 22.2 dB for image of in-gap interference (ACS/IBB) overlap with wanted CC.

Proposal 5: For IBB, the power ratio of interference to wanted signal power can only be in the range of 0~ 25dB, for image of in-gap interference (ACS/IBB) overlap with wanted CC.

Reference

[1] R4-2502873, WF on fragmented DL, MediaTek Inc., RAN4#114.

[2] R4-2416164, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc., RAN4#112bis 2024-10