# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504156
Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.10.2.2.2

Source: 	OPPO

Title: 	Discussion on Rx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Rx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Rx requirements Issue 3-1-0: General Agreement: - For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5. Issue 3-1-1: RSD requirements for PC2 on n256 Agreement: - For PC2 on n256: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx. Issue 3-1-2: RSD requirements for PC2 on n254 Agreement: For PC2 on n254: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx Open issues: ========================================= Issue 3-1-3: RSD requirements for PC2 on n255 - Proposals - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; [0.4]; [0.4]; [0.5]; [0.5]. 2Tx; [0.8]; [0.8]; [0.9]; [0.9]. - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei) - PA noise as -117 dBm/Hz. - Option 3: Assuming IIP2 54dBm (Qualcomm) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; 1.3; 1.3; 1.3; 1.3. 2Tx; 2.4; 2.4; 2.4; 2.4. Way forward: - Continue align the assumptions and further discuss above options. Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo) - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek) Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [1.8]; [1.8]; [1.8]; [1.8]. n255; [2.4]; [2.4]; [2.5]; [2.5]. n254; [1.8]; [1.8]; [1.8]. Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [2.2]; [2.2]; [2.2]; [2.2]. n255; [3.2]; [3.2]; [3.3]; [3.3]. n254; [2.2]; [2.2]; [2.2]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung) - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256. Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3.7]; [3.7]; [3.7]; [3.7]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options.

This contribution further discussed the UE Rx RF requirements for NR NTN HPUE.

- Discussion

In last meeting, it was agreed 0dB RSD for PC2 NR-NTN in band n256 and n254, but for n255 the duplexer isolation and PA interference need further study.

Figure 2-1 the default Tx-Rx frequency separation for NR NTN bands

From Figure 2-1, we can see the default Tx-Rx frequency separation of n255 is 101.5MHz, and n266 is 70MHz. And the REFSENs of n255 with PC3 UL is better than the REFSENs of n256 and n254 with PC3 UL in current spec as shown Table 2-1:

Table 2-1 REFSENs of NR NTN bands with PC3 UL

At least, it proved that the duplexer isolation and PC3 PA performance of n255 is better than the duplexer isolation and PC3 PA performance of n256.

In order to reduce the implementation difficulty of  NR-NTN PC2 UE in band n255, we can keep the same REFSENs of NR-NTN PC2 UE in band n256, n255, n254.

Proposal 1: 0.5dB RSD can be specified for PC2 NR NTN UE in n255.

In last meeting, it was agreed that PC1 and PC1.5 only for non-handheld NR-NTN UE. And in current NR TN UE spec, some FDD bands support PC1 and only applying to for non-handheld as shown in table 2-2, there is no defining the RSD for non-handheld HPUE .

Table 2-2 NR TN power class

So, it’s unnecessary to define the RSD for non-handheld UE.

Proposal 2: 0 RSD for non-handheld UE including PC 1 and PC 1.5.

- Conclusion

In this paper we give further discussion on the RX requirement for HPUE NTN. The proposals are as following:

Proposal 1: 0.5dB RSD can be specified for PC2 NR NTN UE in n255.

Proposal 2: 0 RSD for non-handheld UE including PC 1 and PC 1.5.

- Reference

[1] R4-2502594, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114.