3GPP TSG-RAN WG4 Meeting #114bis	R4-2504165

# Wuhan, China, 7– 11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_MC_enh) R19 correction of switchig time between NUL and SUL carriers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current spec, 0us switch time is mandated in 38.101-1 spec for NUL and SUL band combination in serval tables like below: Table 5.2C-1: Operating band combination for SUL in FR1 NR Band combination for SUL; NR Band (Table 5.2-1). SUL_n1-n802; n1, n80. SUL_n1-n812; n1, n81. …; …. SUL_n79-n982; n79, n98. NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory.. However, non-zero switching time for the SUL-NUL band combination has already been allowed in Tx switching as below table from Rel-18 38101-1. #### 6.3C.3.5 Time mask for switching across up to four uplink bands The switching time mask requirements specified in this sub-clause are applicable for an NR SUL band configuration with inter-band CA when the capability supportedBandPairListNR-r18 is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [10]. In the NR SUL band configuration with inter-band CA, the number of uplink bands with different carrier frequencies is up to four. NR UL carrier(s) in each of the up to four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability FeatureSetUplinkPerCC. The switching time masks in Figure 6.3C.3.5-1 and Figure 6.3C.3.5-2 are applicable to each of the uplink band pairs in the SUL band configuration with inter-band CA, and are applicable to uplink transmissions when configured with switchedUL or dualUL by the parameter switchingOptionConfigForBandPair. To simplify the figures, the two bands in different band pairs are denoted as NR band X and band Y. The uplink transmission on either band X or band Y is with one or two transmit antenna connector(s), – if NR UL carriers in both bands in one band pair are capable of one transmit antenna connector, 1Tx-1Tx switching is supported for the band pair; – if NR UL carrier(s) in one band of one band pair is capable of one transmit antenna connector, and NR UL carrier(s) in the other band of the band pair is capable of two transmit antenna connectors, 1Tx-2Tx switching is supported for the band pair; – if NR UL carriers in both bands of one band pair are capable of two transmit antenna connectors, 2Tx-2Tx switching is supported for the band pair.. And also UE capability as below: ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840 Indicates UE supports R18 dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL as defined in TS 38.214 [12] and TS 38.101-1 [2]. The capability signalling comprises of the following parameters: - bandIndexUL1-r18 and bandIndexUL2-r18 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth UL band entry in the band combination. UE shall indicate support of 2-layer UL MIMO in FeatureSet on both bands for 2Tx-2Tx switching, or indicate support of 2-layer UL MIMO on one band and 1-layer MIMO on the other band for 1Tx-2Tx switching, or indicate support of 1-layer UL MIMO on both bands for 1Tx-1Tx switching. … - switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period. switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.101-1 [2]. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2]. … - configured1T1T-OnTwoBands-r18 indicates the support of 2-band configuration of 1T-1T UL Tx switching using Rel-18 UL Tx switching configurations. This capability is applicable for a band pair where the UE reports no UL-MIMO on both bands and indicate support of switchedUL.; BC; FD; N/A; FR1 only. Therefore, there are inconsistency between the notes of mandating 0us switching time and the non-zero switching period in time mask requirements. |
| Summary of change: |  | Modify the SUL-NUL band combination switch time notes to allow non-zero switching time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistency still exists and UE mandatory to support 0us switching time for SUL-NUL band combination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n1-n802 | n1, n80 |
| SUL_n1-n812 | n1, n81 |
| SUL_n1-n892 | n1, n89 |
| SUL_n3-n842 | n3, n84 |
| SUL_n5-n842 | n5, n84 |
| SUL_n8-n842 | n8, n84 |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n802 | n41, n80 |
| SUL_n41-n812 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n78-n892 | n78, n89 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n842 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840 if reported, otherwise 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Table 5.2C-2: Operating SUL band combination with intra-band non-contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41(*)-n992 | n41, n99 |
| CA_n48(*)-n992 | n48, n99 |
| CA_n77(*)-n992 | n77, n99 |
| CA_n78(*)-n862 | n78, n86 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840 if reported, otherwise 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2. |  |

Table 5.2C-3: Operating SUL band combination with intra-band contiguous CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41-n80 | n41, n80 |
| CA_n41-n83 | n41, n83 |
| CA_n41-n95 | n41, n95 |
| CA_n41-n98 | n41, n98 |
| CA_n78-n80 | n78, n80 |
| CA_n78-n81 | n78, n81 |
| CA_n78-n84 | n78, n84 |
| CA_n78-n89 | n78, n89 |
| CA_n79-n80 | n79, n80 |
| CA_n79-n83 | n79, n83 |
| CA_n79-n95 | n79, n95 |
| CA_n79-n98 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840 if reported, otherwise 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |  |

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_n78-n80 | n1, n78, n80 |
| CA_n1_n78-n81 | n1, n78, n81 |
| CA_n1_n78-n84 | n1, n78, n84 |
| CA_n1_n78-n89 | n1, n78, n89 |
| CA_n3_n41-n80 | n3, n41, n80 |
| CA_n3_n78-n80 | n3, n78, n80 |
| CA_n3_n78-n84 | n3, n78, n84 |
| CA_n1-n3_n78-n80 | n1, n3, n78, n80 |
| CA_n1-n3_n78-n84 | n1, n3, n78, n84 |
| CA_n3_n79-n80 | n3, n79, n80 |
| CA_n5_n78-n84 | n5, n78, n84 |
| CA_n8_n78-n81 | n8, n78, n81 |
| CA_n8_n78-n84 | n8, n78, n84 |
| CA_n28_n41-n83 | n28, n41, n83 |
| CA_n28_n79-n83 | n28, n79, n83 |
| CA_n41_n79-n80 | n41, n79, n80 |
| CA_n41_n79-n83 | n41, n79, n83 |
| CA_n41_n79-n95 | n41, n79, n95 |
| CA_n41_n79-n97 | n41, n79, n97 |
| CA_n41_n79-n98 | n41, n79, n98 |
| CA_n78_n1-n80 | n1, n78, n80 |
| CA_n78_n1-n81 | n1, n78, n81 |
| CA_n78_n1-n89 | n1, n78, n89 |
| CA_n78_n3-n84 | n3, n78, n84 |
| CA_n78_n5-n84 | n5, n78, n84 |
| CA_n78_n8-n84 | n8, n78, n84 |
| CA_n79_n41-n80 | n41, n79, n80 |
| CA_n79_n41-n83 | n41, n79, n83 |
| CA_n79_n41-n95 | n41, n79, n95 |
| CA_n79_n41-n97 | n41, n79, n97 |
| CA_n79_n41-n98 | n41, n79, n98 |
| CA_n28-n79_n41-n83 | n28, n41, n79, n83 |
| CA_n28-n41_n79-n83 | n28, n41, n79, n83 |
| CA_n41A-n95A_n79A-n98A | n41, n95, n79, n98 |
| CA_n41A-n98A_n79A-n95A | n41, n98, n79, n95 |
| CA_n41A-n83A_n79A-n98A | n41, n83, n79, n98 |
| CA_n41A-n83A_n79A-n95A | n41, n83, n79, n95 |
| CA_n41C_n95A-n98A | n41, n95, n98 |
| CA_n78C_n80A-n84A | n78, n80, n84 |
| CA_n78C_n81A-n84A | n78, n81, n84 |
| CA_n78C_n84A-n89A | n78, n84, n89 |
| NOTE 1: If a UE is configured with a single cell consisting of a NR UL carrier and a corresponding NR SUL carrier, the switching time between NR UL carrier and the corresponding NR SUL carrier is according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840 if reported, otherwise 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

<End of change>