3GPP TSG-RAN WG4 Meeting #114bis													R4-2504170
Wuhan, China, 7– 11 April, 2025

Title:	R19 MPR for PC1.5 contig CA

Source:	OPPO

Agenda item:	7.1.1.1.2

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA MPR based on WF [1] and requirement clarifications in [2][3].

- Contiguous RB allocation MRP

2.1 PSD imbalance impacts to MPR

The PSD imbalance issue has been raised for several meetings, and to better understand how it will impact the MPR, some simulations have been done with following configurations.

- CA contiguous class C

- Cc1 = 100MHz

- Cc2 = 90MHz

- scs = 30khz

- CP-OFDM

- LO = 1 or 2

- Outer RB allocations

- PSD cc2 / PSD cc1 = 0dB, 6dB

Table 1 PSD imbalance impacts to Contiguous CA MPR

| cc1 RB length | cc1 RB start | cc2 RB length | cc2 RB start | Delta MPR with 1LO: psd 6dB – psd 0dB | Delta MPR with 2LO: psd 6dB – psd 0dB |
| --- | --- | --- | --- | --- | --- |
| 135 | 138 | 161 | 1 | 0.00 | 1.60 |
| 135 | 138 | 179 | 1 | -0.01 | 1.89 |
| 135 | 138 | 193 | 1 | 0.00 | 2.21 |
| 135 | 138 | 201 | 1 | 0.22 | 2.38 |
| 135 | 138 | 215 | 1 | 0.00 | 2.36 |
| 135 | 138 | 218 | 1 | 0.23 | 2.18 |
| 135 | 138 | 224 | 1 | 0.00 | 2.25 |
| 135 | 138 | 242 | 1 | 0.24 | 2.11 |
| 15 | 258 | 179 | 1 | 0.00 | 0.48 |
| 15 | 258 | 193 | 1 | 0.00 | 0.31 |
| 15 | 258 | 201 | 1 | -0.19 | 0.31 |
| 15 | 258 | 215 | 1 | 0.00 | 0.32 |
| 15 | 258 | 218 | 1 | 0.00 | 0.32 |
| 15 | 258 | 224 | 1 | 0.00 | 0.33 |
| 15 | 258 | 242 | 1 | 0.00 | 0.34 |
| 273 | 1 | 245 | 1 | 0.50 | 2.55 |
| 270 | 3 | 242 | 1 | 0.24 | 2.30 |
| 270 | 3 | 1 | 1 | 0.00 | -0.07 |
| 270 | 3 | 2 | 1 | 0.00 | -0.06 |
| 270 | 3 | 3 | 1 | 0.00 | -0.05 |
| 270 | 3 | 4 | 1 | 0.00 | -0.34 |
| 144 | 129 | 201 | 1 | 0.22 | 2.53 |
| 144 | 129 | 215 | 1 | 0.22 | 2.35 |
| 144 | 129 | 218 | 1 | 0.23 | 2.36 |
| 144 | 129 | 224 | 1 | -0.01 | 2.42 |
| 144 | 129 | 242 | 1 | 0.24 | 2.38 |

From above it can be noticed that the PSD imbalance has more impact to 2LO cases than the 1LO cases. And For 1LO cases, PSD imbalance of 6dB has MPR impact within 0.5dB.

Observation 1:         PSD imbalance has more impact to 2LO UE implementation than the 1LO implementation. And For 1LO cases, 0.5dB MPR impact can be observed when PSD imbalance reach 6dB.

2.2 MPR values for contiguous RB allocation

In last meeting, the MPR requirements were discussed based on the assumption that “The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2”. And the relative values comparing to PC2 means the PA assumed is just meet the PC2 MPR requirements which is worse than normal PA. The tentative requirements as table 2 from last meeting WF.

Table 2 MPR for PC1.5 intra-band contiguous CA with contig RB allocations

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | [4.5] | [9] |
|  | QPSK | [4.5] | [9] |
|  | 16QAM | [4.5] | [9] |
|  | 64QAM | [7] | [9] |
|  | 256QAM | [9] | [9.5] |
| CP-OFDM | QPSK | [5-5.5] | [9.5] |
|  | 16QAM | [5-5.5] | [9.5] |
|  | 64QAM | [6-6.5] | [9.5] |
|  | 256QAM | [8.5] | [9.5] |

For the above 0.5dB additional MPR caused by PSD imbalance, it is no necessarily has to be added to the tentative requirements, however, the PSD imbalance restriction to the tests needs to be clear to avoid large PSD imbalance scenarios, especially for UEs with 2LO implementations.

Proposal 1:         0.5dB additional MPR can be considered for up to 6dB PSD imbalance in contiguous UL CA with contiguous RB allocation.

Proposal 2:         Inform RAN5 about the up to 6dB PSD difference assumption in the MPR requirement definition. And below is a content example.

# Overall description In Rel-19, the MPR for PC1.5 intra-band contiguous UL CA and intra-band non-contiguous UL CA have been defined with the assumption that PSD difference between two CCs in UL is within 6dB. RAN4 would like to inform RAN5 about this assumption, and respectfully ask RAN5 to take them into account when developing test specifications for PC1.5 intra-band UL CA. # Actions To RAN5 ACTION: RAN4 respectfully ask RAN5 to take them into account when developing test specifications for PC1.5 intra-band UL CA.

- Non-Contig RB allocation MRP

For non-contiguous RB allocations the previous proposed results in [4] are reproduced below for discussion.

The configurations are:

- CA class C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

The MPR results as shown in Annex A can be summarized with necessary margins as below table 3. It can be found that for some regions the values are smaller than current non-contiguous CA PC2 MPR with 2Tx. And if same principle “The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2” is intended to be applied also to non-contiguous RB allocation, then our proposal as shown in table 4 can be 0.5dB additional values comparing to PC2 if simulation results are smaller than PC2 requirements to make it looks more rational.

Table 3 MPR based on simulation results for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 6.5 | 8.5 |
|  | QPSK | 5 | 7 | 8.5 |
|  | 16QAM | 5 | 7.5 | 8.5 |
|  | 64QAM | 5 | 7.5 | 8.5 |
|  | 256QAM | 5 | 8 | 9 |
| CP-OFDM | QPSK | 5 | 8.5 | 9 |
|  | 16QAM | 5 | 8.5 | 9 |
|  | 64QAM | 5 | 8.5 | 9 |
|  | 256QAM | 5 | 8.5 | 9 |

Table 4 MPR based on PC2 requirements for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 9 | 15 |
|  | QPSK | 5 | 9 |  |
|  | 16QAM | 5 | 9 |  |
|  | 64QAM | 6.5 | 9 |  |
|  | 256QAM | 8 | 9.5 |  |
| CP-OFDM | QPSK | 5 | 9.5 | 16 |
|  | 16QAM | 5 | 9.5 |  |
|  | 64QAM | 6.5 | 9.5 |  |
|  | 256QAM | 9 | 9.5 |  |

Proposal 3:         If use the principle of “The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2”, then the proposed MPR for non-contiguous RB allocation can be found in table 4, though our simulation results doesn’t support that big values in some region.

- Conclusion

This paper shared more MPR simulation results for PC1.5 intra-band contiguous CA, got below proposals.

Observation 1:         PSD imbalance has more impact to 2LO UE implementation than the 1LO implementation. And For 1LO cases, 0.5dB MPR impact can be observed when PSD imbalance reach 6dB.

Proposal 1:         0.5dB additional MPR can be considered for up to 6dB PSD imbalance in contiguous UL CA with contiguous RB allocation.

Proposal 2:         Inform RAN5 about the up to 6dB PSD difference assumption in the MPR requirement definition. And below is a content example.

# Overall description In Rel-19, the MPR for PC1.5 intra-band contiguous UL CA and intra-band non-contiguous UL CA have been defined with the assumption that PSD difference between two CCs in UL is within 6dB. RAN4 would like to inform RAN5 about this assumption, and respectfully ask RAN5 to take them into account when developing test specifications for PC1.5 intra-band UL CA. # Actions To RAN5 ACTION: RAN4 respectfully ask RAN5 to take them into account when developing test specifications for PC1.5 intra-band UL CA.

Proposal 3:         If use the principle of “The MPR values of intra-band contiguous ULCA PC1.5 are supposed to be higher than single CC PC1.5, as well as intra-band contiguous ULCA PC2”, then the proposed MPR for non-contiguous RB allocation can be found in table 4, though our simulation results doesn’t support that big values in some region.

Table 4 MPR based on PC2 requirements for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 5 | 9 | 15 |
|  | QPSK | 5 | 9 |  |
|  | 16QAM | 5 | 9 |  |
|  | 64QAM | 6.5 | 9 |  |
|  | 256QAM | 8 | 9.5 |  |
| CP-OFDM | QPSK | 5 | 9.5 | 16 |
|  | 16QAM | 5 | 9.5 |  |
|  | 64QAM | 6.5 | 9.5 |  |
|  | 256QAM | 9 | 9.5 |  |

- Reference

[1] R4-2502863 WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, #114 2025-02

[2] R4-2412349 R19 MPR for PC1.5 contiguous CA, OPPO, #112 2024-08

[3] R4-2412350 R19 MPR for PC1.5 NC CA, OPPO, RAN4#112 2024-08

[4] R4-2417914 R19 MPR for PC1.5 contiguous CA, OPPO, RAN4#113 2024-11

- Annex A: MPR for non-contig RB allocation

Table 1 CP-OFDM non-contig RB allocation: CA Class C

| CP-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk |  | qpsk |  | 16qam |  | 64qam |  | 256qam |  |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 1.3 |  | 1.4 |  | 1.5 |  | 1.3 |  | 1.2 |  |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 2.0 |  | 1.8 |  | 2.1 |  | 1.6 |  | 1.6 |  |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 1.4 |  | 1.5 |  | 1.5 |  | 1.4 |  | 1.4 |  |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 1.3 |  | 1.4 |  | 1.4 |  | 1.4 |  | 1.3 |  |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 1.1 |  | 1.2 |  | 1.3 |  | 1.1 |  | 1.1 |  |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 1.6 |  | 1.7 |  | 1.7 |  | 1.5 |  | 1.5 |  |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 2.3 |  | 2.3 |  | 2.3 |  | 2.2 |  | 2.2 |  |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 2.2 |  | 2.3 |  | 2.3 |  | 2.0 |  | 1.9 |  |
| Inner Max MPR |  |  |  |  |  |  |  |  | 2.3 |  | 2.3 |  | 2.3 |  | 2.2 |  | 2.2 |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 4.7 |  | 5.1 |  | 5.2 |  | 5.0 |  | 4.9 |  |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 5.0 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 5.0 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 5.0 |  | 5.1 |  | 5.2 |  | 5.0 |  | 4.9 |  |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.5 |  | 5.5 |  | 5.7 |  | 5.4 |  | 5.4 |  |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 5.0 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 5.0 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 5.0 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 5.2 |  | 5.3 |  | 5.5 |  | 5.2 |  | 5.2 |  |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.6 |  | 5.7 |  | 5.9 |  | 5.6 |  | 5.6 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  |  | 5.6 |  | 5.7 |  | 5.9 |  | 5.6 |  | 5.6 |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 6.1 |  | 6.1 |  | 6.0 |  | 6.1 |  | 6.0 |  |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 5.0 |  | 5.3 |  | 5.5 |  | 5.0 |  | 5.0 |  |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 5.3 |  | 5.3 |  | 5.5 |  | 5.3 |  | 5.2 |  |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 4.7 |  | 5.1 |  | 5.2 |  | 5.0 |  | 5.0 |  |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 5.2 |  | 5.5 |  | 5.7 |  | 5.2 |  | 5.1 |  |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 5.2 |  | 5.3 |  | 5.4 |  | 5.2 |  | 5.1 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  |  | 6.1 |  | 6.1 |  | 6.0 |  | 6.1 |  | 6.0 |

Table 2 DFT-S-OFDM non-contig RB allocation: CA Class C

| DFT-S-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam |  | 64qam |  | 256qam |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 1.3 | 1.4 | 1.6 | 1.6 |  | 1.5 |  |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 1.8 | 2.0 | 2.1 | 2.2 |  | 2.2 |  |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 1.4 | 1.4 | 1.4 | 1.5 |  | 1.5 |  |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 1.5 | 1.5 | 1.5 | 1.5 |  | 1.6 |  |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 1.2 | 1.3 | 1.4 | 1.3 |  | 1.3 |  |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 1.8 | 1.8 | 2.0 | 2.0 |  | 2.1 |  |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 2.1 | 2.3 | 2.4 | 2.5 |  | 2.5 |  |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 2.1 | 2.2 | 2.4 | 2.5 |  | 2.4 |  |
| Inner Max MPR |  |  |  |  |  |  |  | 2.1 | 2.3 | 2.4 | 2.5 |  | 2.5 |  |  |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 3.1 | 3.5 | 3.9 | 4.2 |  | 4.2 |  |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.1 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.4 |  | 4.5 |  |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 3.1 | 3.5 | 4.1 | 4.2 |  | 4.5 |  |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.5 | 4.0 | 4.6 | 4.6 |  | 4.9 |  |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.5 |  | 4.5 |  |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.5 |  | 4.5 |  |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.5 |  | 4.5 |  |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 3.3 | 3.8 | 4.4 | 4.5 |  | 4.7 |  |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.7 | 4.1 | 4.7 | 4.8 |  | 5.1 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  | 3.7 | 4.1 | 4.7 | 4.8 |  | 5.1 |  |  |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 6.0 | 5.8 | 6.0 | 5.7 |  | 6.1 |  |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 3.1 | 3.6 | 4.2 | 4.5 |  | 4.5 |  |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 3.3 | 3.8 | 4.4 | 4.5 |  | 4.5 |  |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 3.1 | 3.5 | 3.9 | 4.2 |  | 4.2 |  |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 3.5 | 3.7 | 4.3 | 4.6 |  | 4.7 |  |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 3.3 | 3.7 | 4.3 | 4.4 |  | 4.4 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  | 6.0 | 5.8 | 6.0 | 5.7 |  | 6.1 |  |  |