3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504173

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	Discussion on methods for reducing the number of UE Rx chains

Agenda item:	7.5.2

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents some open issues as listed below:

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

Discussion on semi-statically switch hardware resources procedure and fallback considerations

Triggering condition to enable ”One RF Rx chain”

FFS:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

Discussion on applicability for PCell and SCell

Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

Next, we discuss some of these open aspects.

Discussion

In this document we further discuss the methods for reducing the number of UE Rx chains used in relation to the operation of fragmented carriers.

General on fragmented carriers

The issue of UE indicating support of operating a certain band combination as fragmented carriers, has been discussed extensively. As we have mentioned earlier, we find it necessary to clearly distinguish the radio configuration of the non-contiguous intra-band carrier aggregation between the modes of operation related to using the partially shared RF architecture and the fully shared RF chain at the UE side.

In [2], submitted under agenda 7.5.1, we have suggested that the currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation. This would make it possible to clearly indicate both when a UE is capable of supporting fragmented carrier operation and the network to explicitly configure the UE in this type of CA configuration.

Using an explicit CA configuration indication, like bandwidth class (BCS) ‘F’, would allow separating fragmented carrier operation from other types of CA configurations.

In general, we assume that if the UE can change its HW configuration between using 1Rx chain or 2Rx chains between a given band combo while still fulfilling the UE minimum requirements, and without causing negative impact on the scheduling, this is already allowed within the current requirements. This would be possible if UE has spare Rx chain to perform such HW reconfiguration.

Figure 1 Illustration where UE changes its HW configuration between 1 Rx chain and 2Rx chain usage for receiving 2 CCs

To facilitate the continued discussions, we would like to see if it is possible to reach a common understanding on a couple of aspects.

Assuming we have the wanted signal (CCs) and one or more unwanted signal(s) (CCs) in between. It is assumed that the unwanted signal occurs due to another operator operating the CC(s) in between the wanted signals. In this scenario under discussion the wanted and unwanted signals originate from collocated transmitters.

From deployment point of view, there will be power restrictions in place regulating the maximum transmit power of CCs operated by one operator. It is assumed that the maximum transmit power difference cannot exceed 6dB between CCs of the same operator.

The maximum transmit power difference between wanted CCs cannot exceed 6dB.

However, there are in principal no transmit power requirements related to the maximum allowed transmit power difference between the CC between the two operators. The power balancing is done by mutual agreements and alignments between operators. Even if some efforts are put into balancing power between operators the effective received power difference at the UE may be significantly different. There may be differences between collocated scenario and non-collocated scenario, but especially when the operators are not co-located, the received power difference may be significant. Additionally, the basic physical factors, such as the distance from the UE to the gNB or sectorization of the different operators, is effectively determining the field strength at any given location of the UE. As a result it is not practically possible to limit the maximum transmit power difference between CCs operated by different operators and thereby the received power difference at UE side. Hence, we assume that the power difference between wanted and unwanted signals may vary significantly.

Figure 2 Illustration of maximum allowed power difference between wanted CCs and unwanted interferer

Discuss and agree if there is a maximum allowed power difference between wanted CCs used in Fragmented CA and unwanted interferer (i.e. CCs operated by the different operators).

We assume, that in order facilitate proper operations of CA as a fragmented aggregation, with an unknown/unwanted interferer in between the wanted, the UE must be made aware that a certain CA configuration can be operated with either single Rx chain or separate Rx chains.

General on SCell control for fragmented CA

If a change in HW configuration between UE using 1 Rx chain or 2 Rx chains for reception in a fragmented carrier aggregation deployment does impact the UE performance, system performance or the network, there is a need to ensure that whatever ‘mode’ the UE is operating in, is aligned between the UE and the network. Hence, it needs to be well aligned between UE and network whether the UE is applying 1Rx chain or 2 Rx chain.

1Rx chain operation shall be controlled by network if using 1Rx chain for reception in a Fragmented carrier CA has negative impact on performance.

In many scenarios, if the UE can change freely between using 1Rx or 2Rx chains without negative performance impact, this could be feasible. However, if the UE cannot change between using 1Rx or 2Rx in a Fragmented CA setup, without system impact, it needs to be clear how and when such HW reconfiguration is allowed and can be performed.

One such example discussed in last meeting was the control of the SCell. Hence, what shall happen if the UE has PCell and Scell configured in Fragmented CA setup and the interference from the unwanted signal suddenly increases beyond what the UE can manage?

Figure 3 Illustration of changing from receiving CC1 and CC2 with single Rx chain to receiving only CC1 with single Rx chain. In this case CC2 cannot be received anymore after the change.

If the UE does not have a spare Rx chain to use, would the SCell (e.g. CC2 in figure 3) remain ‘active’ on the UE side or not? Would the UE just ‘drop’ the SCell? Would some UEs drop the PCell? Having different and unspecified UE behaviour in the field is of course not a good solution and will surely lead to a feature which is not very attractive in the field. Hence, UE behaviour and operation cannot be left up to each different UE implementation.

Currently SCell operation is under network control. Hence, the network configures and de-configures the SCell. The network activates and deactivates the SCell. This has been the principle since Rel-10, and this principle should not be broken. In fact, SCell control cannot be changed without involvement of at least RAN2.

Figure 4 Simple illustration of network SCell control

We suggest that RAN4 does not discuss solutions which breaks the current network control of SCell operations.

RAN4 does not discuss solutions which breaks the current network control of SCell operations.

If RAN4 explores solutions which operates differently than current network control of SCell operations, RAN4 must involve other working groups and at least RAN2 during the discussions. Such UE (guided) autonomous SCell operations could be for example (as discussed) include UE fallback rules etc.

Any changes to existing SCell operation must include at least RAN2 (during the WI phase).

General RRM aspects

Based on the principle that any changes related to PCell and/or SCell in Fragmented CA operations need to be well coordinated between UE and network, a number of RRM related questions have been raised.

One question raised was:

Question 1: Can it be assumed that the PCell can be part of a Fragmented Carrier configuration, or can a Fragmented CA configuration only include SCells?

Currently we do not see it is possible to give a clear answer. Such a restriction may limit the use of feature. Additionally, if proper solutions are developed to protect the PCell, there may not be a reason to not allowing PCell being part of a fragmented CA configuration.

Continue assuming PCell is not excluded from being part of a fragmented CA configuration.

When the network configures the UE with fragmented CA configuration, the basic approach is to avoid a design relying on any try-and-error method. One proposal set forward is to request the UE to perform in-gap measurements with reference to the CA configuration, so that any connection feasibility, based on the in-gap interferer, can be evaluated in advance. One important point with this, is to avoid any radio link failure once the fragmented CA is configured.

This leads to the next question to be raised:

Question 2: Enabling network-controlled actions (such as a fragmented CA configuration) would require some UE assistance. Which measurements and/or UE feedback would provide suitable network assistance when needed?

Multiple options have already been discussed. However, the basic assumptions still seem unclear. Additionally, it is open which UE assistance information would be needed on network side, what information would be enough on network side and how could the UE potentially provide such information to the network. Would it be in form of measurement reporting or something else?

Different companies have diverse views, but at least RAN4 would need to discuss and agree how to configure and control fragmented CA in an orderly manner which is not based on a ‘try-and-error’ approach or other non-robust solutions.

RAN4 need to discuss and agree which (if any) measurements and/or UE feedback would provide suitable network assistance when needed, to enable the network configuring fragmented CA in a robust way.

Measurements for fragmented CA

As mentioned, developing a framework for fragmented CA which is not functioning in a robust manner does not seem attractive and can cause very negative impact on the network side and UE side in addition to the user experience.

As discussed, in many situations the HW reconfiguration between using 1Rx chain or 2Rx chain for CA can done on UE side without network involvement. This can be done provided there are no negative impact on the system performance and UE fulfilling the minimum requirements.

For those cases where there is clear network impact there is a need to ensure UE and network alignment. For example, for the UE configured with fragmented CA, if the UE has exhausted the number of Rx chains but cannot anymore receive the CCs involved in the fragmented CA configuration, a HW reconfiguration from 1Rx chain reception to 2Rx chain reception cannot be done without network impact.

For these examples where there is network impact, any action would need to be aligned with the network. For example, either any action is network initiated or at least based on clearly defined rules.

One aspect to be discussed specifically for fragmented CA would be the assumptions related to the nature of the unwanted signal located in between the wanted signal from the two wanted CCs. This is also related to our discussion above, and which restrictions (if any) that can be assumed in power difference between the wanted and unwanted signals. When discussing potential UE measurement assistance, RAN4 would also need to discuss the assumption concerning how much the fluctuations between the power differences could be assumed and what could be the impact from any such fluctuations.

Discuss the assumption concerning the power fluctuations on the wanted and unwanted signals.

We assume that for the wanted signal the signals will always be within 6dB. We also assume that within the unwanted signals (if from same operator) the signals will be within 6dB.

Discuss the assumption concerning power difference fluctuations between the wanted and unwanted signals.

The assumption here will need to account the outcome of the proposal 1 in addition to outcome of proposal 7.

From a measurement assistance point of view the outcome of this discussion and proposals 1, 7 and 8 would give insights to the type of any UE assistance information to the network.

To exemplify some of the possible assistance information that could be provided by the UE in order to minimize the effect of in-gap interference, three types of measurements could be used for evaluation and control of fragmented carrier pairs.

- The average power of the in-gap interference.

- Peak power of the in-gap interference, so that the ADC does not saturate.

- CQI measurement on the fragmented carrier.

However, CQI measurement are likely not able to precisely identify the cause of signal deterioration, which may result from factors like image interference or out-of-gap interference due to relaxed bandwidth of combined carriers. Hence, CQI measurements may not alone be enough to determine whether use of fragmented CA is feasible – especially before actually having fragmented CA configured.

Hence, there will likely be benefits in knowing up front (before entering into fragmented CA configuration and operation) whether use of single Rx for reception will cause possible negative impact on network, UE and system level.

Here we see that in-gap measurements could provide important measurement assistance information to the network. Based on such measurements the network would be aware whether the UE can be configured with fragmented CA.

RAN4 to discuss if in-gap measurements could provide important measurement assistance information to network for assisting Scel configuration.

Contribution to TR

Based on the progress of the discussions we would propose to capture the open issues and if conclusions are reached also capture those in the TR. We suggest at least start capturing TP related to the following open issues:

- Any common understanding concerning the questions related to assumed power differences between wanted signals, unwanted signals and also between wanted and unwanted signals.

- Some of the important aspects related to the assumptions related to SCell control.

- RAN4 does not exclude PCell being part of a fragmented CA configuration. However, the actual configuration is entirely up to the network.

- RAN4 need to discuss and agree which (if any) measurements and/or UE feedback would provide suitable network assistance when needed, to enable the network configuring fragmented CA in a robust way.

  - RAN4 to discuss if in-gap measurements could provide important measurement assistance information to network for assisting Scel configuration.

Work on addressing and concluding the above open aspects and capture the open issues including possible conclusions in the TR.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

General on fragmented carriers:

- Using an explicit CA configuration indication, like bandwidth class (BCS) ‘F’, would allow separating fragmented carrier operation from other types of CA configurations.

The maximum transmit power difference between wanted CCs cannot exceed 6dB.

- Discuss and agree if there is a maximum allowed power difference between wanted CCs used in Fragmented CA and unwanted interferer (i.e. CCs operated by the different operators).

General on SCell control for fragmented CA:

1Rx chain operation shall be controlled by network if using 1Rx chain for reception in a Fragmented carrier CA has negative impact on performance.

RAN4 does not discuss solutions which breaks the current network control of SCell operations.

Any changes to existing SCell operation must include at least RAN2 (during the WI phase).

General RRM aspects:

Continue assuming PCell is not excluded from being part of a fragmented CA configuration.

RAN4 need to discuss and agree which (if any) measurements and/or UE feedback would provide suitable network assistance when needed, to enable the network configuring fragmented CA in a robust way.

Measurements for fragmented CA:

Discuss the assumption concerning the power fluctuations on the wanted and unwanted signals.

Discuss the assumption concerning power difference fluctuations between the wanted and unwanted signals.

From a measurement assistance point of view the outcome of this discussion and proposals 1, 7 and 8 would give insights to the type of any UE assistance information to the network.

RAN4 to discuss if in-gap measurements could provide important measurement assistance information to network for assisting Scel configuration.

Contribution to TR:

Work on addressing and concluding the above open aspects and capture the open issues including possible conclusions in the TR.

- References

- R4-2502873, WF on NR FR1 DL fragmented carriers study, RAN4#114

- R4-2504129, General aspects of Fragmented CA, Nokia

- R4-2504128, Discussion on RF requirements for Fragmented CA, Nokia