3GPP TSG RAN WG4 #114bis	R4-2504193

Wuhan, China, April 7 – 11, 2025

Agenda item:		7.23.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114 [1] regarding evolution of NR duplex operation. There were not so many agreements made regarding SBFD general aspects and system parameters. Also, the open items are listed in the WF. The most important topic is related to subband configurations and guardbands. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## CLI handling impact on RF requirement

The following was agreed in RAN4#112 and remained open for RAN4#112bis and RAN4#113:

- Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Regarding the FFS, our preference is to define the BS RF requirements without considering the potential impact of gNB-to-gNB CLI handling mechanisms, i.e., the FFS should not be considered. First, for the sake of progress we should not risk slowing down the work in RAN4 with revisiting some requirements pending on RAN1 progress. Secondly, the CLI handling schemes discussed in RAN1 are mainly applicable to co-channel CLI and their applicability to adjacent channel CLI is questionable. In fact, RAN1 discussions on gNB-to-gNB CLI handling currently focuses on enabling time/frequency coordination and spatial coordination (beam nulling and beam pairing). All the discussed mechanisms require, at least,

i) exchange of CLI-RS configurations across gNBs,

ii) identification of aggressor gNBs via measurements,

and iii) exchange across gNBs of measurement results and additional information to enable the aggressor-victim coordination.

All the above rely on signaling between gNBs over the Xn interface (intra-operator signaling). However, there is no standardized inter-operator interface and therefore exchange of information between operators is not possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling on the RF requirements is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc. The arguments above support the following proposal:

RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## DL/UL subband configurations support for SBFD

### FR2-1 CBW and SCS support for SBFD

The following was agreed in RAN4#114:

Agreement: - In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz. - RAN4 to further study on 100MHz for DUD case to identify if any issue.

For FR2-1, considering SSB bandwidth and sync raster step size of 28.8 MHz and 17.28 MHz. respectively, 100 MHz seem sufficient for DU/UD case, but not for DUD case. Note that RAN1 is still discussing whether SBFD slots containing SSB transmissions should be still considered as ‘SBFD slots’ or should be converted to ‘DL only slots’ instead. Under the assumption that SSBs are transmitted on SBFD slots, 200 MHz would be needed for DUD case. If the SSBs are not transmitted in SBFD slots but in DL-only or non-SBFD slots, then 100MHz is also sufficient for FR2-1.

If SSBs are transmitted in SBFD slots, RAN4 to consider 200MHz channel bandwidth for FR2-1 DUD case. A note can be added in the specification to indicate that FR2-1 SBFD DUD operation with 100 MHz channel bandwidth requires the SSB to be transmitted in DL-only or non-SBFD slots.

### Declaration and flexibility for SBFD operation

The following was agreed in RAN4#114:

Agreement: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS

The agreement mentioned above is valid for FR1 WA BS and in our view, this is also applicable to FR1 MR BS. As FR1 MR BS is very similar to FR1 WA BS and has high output power in the top of the range, the same restriction should also be applied for FR1 MR BS. We agree that the restriction shall not be applied to FR1 LA BS.

The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS.

### DL/UL subband configurations support for SBFD

The following was agreed in RAN4#113 and no new agreement was reached in RAN4#114:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In RAN4#113 meeting, there was no consensus on whether the number of UL subband sizes to be declared by the gNB is 2 or 3. In the same meeting, it has also been agreed that the gNB shall not operate any declared configuration that has not been tested. Our view is that we should limit the complexity and testing at the gNB, therefore, we support that maximum 2 specific UL subband sizes are allowed to be declared.

Regarding the first FFS in this agreement, we should discuss if we support different sizes for specific UL subband for DUD and DU/UD. In our view, one of the 2 specific UL subband sizes can be applicable to the DUD configuration, and the other to the DU/UD configuration.

Regarding both the DL and the UL subband sizes, we support that the restriction is also applicable to the FR1 MR BS.

For a specific channel BW supported for SBFD, maximum of 2 specific UL subbands sizes are allowed to be declared.

One of the 2 specific UL subband sizes can be applicable to the DUD configuration, and the other to the DU/UD configuration.

The restriction on maximum number of specific UL subband sizes to be declared is also applicable to the FR1 MR BS.

The restriction on maximum number of specific DL subband sizes to be declared is also applicable to the FR1 MR BS.

## Requirement set applicability table for SBFD-capable BS

The following way forward was agreed in RAN4#114:

Way Forward: - RAN4 further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on. NOTE: The table does not imply that the BS can be declared 1-H for non-SBFD and 1-O for SBFD, or vice versa.

We see that the requirement table is a very important table and needs careful consideration when defining the applicability. The proposal from R4-2501967 can be used as example to be further studied. The reflected requirement sets for SBFD-capable BS can be referenced to the new clause 12 that will include the requirements for SBFD-capable BS.

If requirement set applicability table for SBFD-capable BS will be implemented in the specification, the reflected requirement sets for SBFD-capable BS can be referenced to the new clause 12 that will include the requirements for SBFD-capable BS.

## BS RF Specification structure for SBFD requirements

The following was agreed in RAN4#114:

Agreement: - RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline, while details can be FFS by taking below structure as the example. Foreword 1 Scope 2 References 3 Definitions, symbols and abbreviations 4 General 5 Operating bands and channel arrangement 6 Conducted transmitter characteristics 7 Conducted receiver characteristics 8 Conducted performance requirements 9 Radiated transmitter characteristics 10 Radiated receiver characteristics 11 Radiated performance requirements 12 Requirements for SBFD-capable BS

#### There was an agreement in RAN4#114 to capture RF requirements for SBFD-capable BS in TS 38.104 and using a separate chapter as the baseline. In our view, using new clause 12 is a good compromise between having a new specification for SBFD-capable BS and using sub-clauses. The discussion of selecting one or the other method had been ongoing for several meetings, and we would not like to reverse the agreement done in the latest meeting. Therefore, we propose to agree to introduce new clause 12 for the requirements for SBFD-capable BS. The applicability of requirements in clause 4 can have references to clause 12 for SBFD-capable BS. The existing agreements for SBFD-capable BS regarding operating bands and channel arrangement can still be applied in clause 5. The agreements on requirements and endorsed TP’s can be applied in the new clause 12.

- To introduce requirements for SBFD-capable BS in TS 38.104 in a separate clause 12.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed:

- RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

If SSBs are transmitted in SBFD slots, RAN4 to consider 200MHz channel bandwidth for FR2-1 DUD case. A note can be added in the specification to indicate that FR2-1 SBFD DUD operation with 100 MHz channel bandwidth requires the SSB to be transmitted in DL-only or non-SBFD slots.

The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS.

For a specific channel BW supported for SBFD, maximum of 2 specific UL subbands sizes are allowed to be declared.

One of the 2 specific UL subband sizes can be applicable to the DUD configuration, and the other to the DU/UD configuration.

The restriction on maximum number of specific UL subband sizes to be declared is also applicable to the FR1 MR BS.

The restriction on maximum number of specific DL subband sizes to be declared is also applicable to the FR1 MR BS.

If requirement set applicability table for SBFD-capable BS will be implemented in the specification, the reflected requirement sets for SBFD-capable BS can be referenced to the new clause 12 that will include the requirements for SBFD-capable BS.

To introduce requirements for SBFD-capable BS in TS 38.104 in a separate clause 12.

# References

- R4-2502310 “Way Forward for [114][306] NR_duplex_evo_General”, RAN4#114, Athens, Greece, 17th-21st February, 2025