3GPP TSG-RAN WG4 Meeting #114bis	R4-2504202

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On CLTM RRM requirements

Agenda item:	7.27.2.3

Document for:	Discussion

Introduction

In this contribution, we continue the discussion about RRM requirements for CLTM. In the discussion, we take into account the agreements RAN2 has made in the RAN2#129 meeting:

Agreements on C-LTM: - From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. - The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. - For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). - When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. - For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. - For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfils the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. - Do not support DG-based first UL transmission for RACH-less conditional LTM. - For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. - For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. - Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

Discussion

Scope

Issue 1-1-1: The applicable scenarios for conditional LTM RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.. < Agreement > - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

Issue 1-2-1: UE based TA measurement RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK). < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

In the last RAN4 meeting, the chairman guidance was to select either to define requirements for deactivated SCell measurements, or UE based TA estimation, or none of these in Rel-19.

Considering the necessity of the requirements being discussed, we think that defining new L1 measurement requirements for deactivated SCells is not necessary in the scope of the Rel-19 WI.

RAN4 not to define CLTM measurement requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based. Consequently, conditional cell switch to a deactivated SCell based on L1 measurements is not to be supported.

Regarding UE-based TA estimation, we are neutral and are fine either to define or not define requirements. If one of the Options 1 and 2 is selected, we prefer Option2. However, we are equally fine with Option 3.

If RAN4 agrees to define requirements for UE-based TA estimation in Rel-19 (Option 2), we believe that there are multiple critical issues from the network point of view that need to be addressed. Some of these issues were discussed in detail in our earlier contribution [1]. We list them here together with some additional considerations.

- Applicability of the UE-based TA estimation requirements: Since Rel-18 LTM also supports UE-based TA estimation (in RAN2 spec), but RAN4 requirements do not exist, we would prefer the requirements for UE-based TA estimation to be defined in a way that they can apply for both Rel-19 CLTM and Rel-18 LTM. This is especially important as a candidate cell for CLTM cell switch is always also a candidate cell for LTM cell switch.

- Reference signals: It is not visible to the network when, and based on which reference signals, the UE performs UE-based TA measurements. This needs to be clarified.

- TA acquisition time: Without the information on how long TA acquisition takes, the network will not know when it can send the cell switch command with the assumption that the UE has the TA. Knowledge of the RS to be used for TA estimation is needed for defining this delay.

- Interruptions due to UE-based TA measurements: SSB-based L1-RSRP measurements specified for LTM candidate cells do not cause interruptions. Therefore, UE-based TA measurements shall not cause interruptions either.

- UL timing accuracy requirements: If UE is using UE-based TA estimation, it shall not deviate from the existing timing accuracy requirements for any uplink transmission.

- Accuracy of the UE-estimated TA: RAN4 should discuss accuracy requirements for the UE-estimated TA and how to verify that the UE-estimated TA is accurate enough.

If RAN4 agrees to define requirements for UE based TA estimation in Rel-19 (Option 2), at least the issues listed below need to be taken into account.

  - Requirements to cover not only Rel-19 CLTM but ideally also Rel-18 LTM.

  - Which reference signals are expected to be used when performing UE-based TA measurements?

  - How long does the acquisition of the UE-based TA take after candidate cell configuration?

  - UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

  - Existing transmit timing accuracy requirements shall apply.

  - How to ensure the accuracy of the UE-estimated TA?

If the issues in Proposal 2 cannot be solved within Rel-19, then Option 3 is preferred.

Early TCI state activation

Issue 1-2-3: Early TCI state activation RAN4#113 < Agreement > - Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.. < Agreement > - No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

In the last meeting, RAN4 agreed that there is no need to change the TCI state activation delay requirements in section 8.25 for CLTM purposes. While this still applies, RAN4 needs to discuss the conditions under which DL sync is not part of the cell switch delay i.e. when Tfirst-RS and TRS-proc are zero. As the baseline, it can be expected that the conditions defined for Tfirst-RS and TRS-proc for Rel-18 cell switch command-based cell switch delay are reused.

According to TS 38.133 (Section 6.3.1.3), unless the periodicity of the SSB of the activated TCI state is up to 160 ms, the TCI state activation command sent as part of the early DL synchronization step is considered to be useful for cell switch interruption reduction (Tfirst-RS and TRS-proc are zero) only if the cell switch command is sent within TCI state activation delay + 160 ms (for UE configured with L1 measurements) or within TCI state activation delay + 480 ms or 160 ms (intra- or inter-frequency TCI state activation for UE configured with L3 measurements, respectively) from the LTM candidate cell TCI state activation MAC-CE.

Reusing these requirements for Rel-19 CLTM would mean that early TCI state activation is considered to be useful only if CLTM decision, and potentially UL synchronization steps are performed within TCI state activation delay + 160 ms (for UE configured with L1 measurements) or within TCI state activation delay + 480 ms or 160 ms (for UE configured with L3 measurements) from the LTM candidate cell TCI state activation MAC-CE.

Hence, assuming that all UEs have the limitation of keeping track of the fine DL timing synchronization of an activated TCI state only for 160/480 ms in the case without L1 measurements or with L1 measurements with periodicity larger than 160 ms would reduce the usefulness of the early TCI state activation feature especially for Rel-19 CLTM, where the moment in time the cell switch condition is fulfilled and cell switch is triggered by the UE cannot be controlled. If some UEs would be able to keep track of the DL sync for a longer time than 160/480 ms, this is in no way visible to the network and hence the network may not consider it beneficial to activate TCI states for CLTM cell switch interruption reduction purposes.

Early TCI state activation may in most cases benefit interruption reduction for CLTM cell switch only if condition becomes met and cell switch is triggered within 160 ms/480 ms after TCI state activation is completed.

RAN4 to discuss how to ensure usefulness of early TCI state activation for CLTM cell switch interruption reduction in the definition of Tfirst-RS and TRS-proc.

Taking the above into account, we would like to check with the group whether it is still reasonable to assume that all UEs supporting CLTM have the same limitation for early DL synchronization. If the limitation cannot be removed/relaxed for all UEs, an option is to define a UE capability that allows the network to know if the UE is able to keep track of the fine DL timing synchronization of an activated TCI state until cell switch or at least for a longer period than 160/480 ms as the minimum requirements state.

Consider either removing the limitation of 160/480 ms for DL sync after TCI state activation or defining a UE capability to indicate UE is able to keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160 ms with L1 measurements, or for longer than 480 ms/160 ms with L3 measurements.

If the limitation cannot be removed, we think at minimum the conditions for Tfirst-RS and TRS-proc being zero need to be adjusted to fit the CLTM procedure better. At minimum, when the network sends PDCCH order and the UE sends the preamble to a candidate cell based on this, similar to L3-based Rel-18 LTM, the DL sync should be considered applicable based on a successful preamble transmission. Furthermore, for UE being able to use the TA it received in TA MAC-CE for RACH-less cell switch, UE will need to have DL sync. Hence, while the TAT is running, Tfirst-RS and TRS-proc should be zero.

If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS and TRS-proc are zero based on successful preamble transmission and while the TAT is running.

Conditional LTM cell switch delay

Issue 1-3-2: The starting point of conditional LTM delay < Agreement > ad hoc - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM]. - Revisit if RAN2 has more progress.

RAN2 agreed that UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. As a result, it is clear now that the starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-3-3: The components of conditional LTM delay < Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Issue 1-3-9: The components of Tinterrupt < Way forward > FFS the following proposals: - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. - Proposal 2: Excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU. - Proposal 3: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

The components that are included in the interruption time depend obviously on the components that will be included in the CLTM cell switch delay. We think it is reasonable to assume that TLTM_RRC-processing, Tfirst-RS and TRS-proc do not need to be considered as part of the interruption. When the steps related to these components –ASN.1 decoding and fine DL synchronization – are done before the cell switch in LTM, the procedures do not cause an interruption. We proposed in Rel-18 that the components would not cause an interruption during the cell switch either, but at that time it was argued by some companies that since the network sends the cell switch command, the UE will not be scheduled after receiving the cell switch command anymore. This is not the case in CLTM, where the cell switch decision happens at the UE side. Hence, we think ASN.1 decoding and DL synchronization components, if included in the cell switch delay, should not be part of the interruption time.

Using the latest RAN2 agreements and the above discussion about the interruption time components, we propose the following definition for the CLTM cell switch delay.

The CLTM cell switch delay DCLTM is the delay from the end of the last TTI containing the RRC command for CLTM until the time the UE transmits the first UL message on the target cell:

- DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt,
Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU

Issue 1-3-4: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay. - Proposal 2: Too early to agree whether TRRC is needed.

According to the latest RAN2 agreements, the UE starts execution condition evaluation once it receives CLTM configuration including CLTM execution condition. Hence, for a non-subsequent CLTM cell switch, TRRC component is needed in the CLTM delay and it is the RRC procedure delay defined in clause 12 in TS 38.331.

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

Issue 1-3-5: TEvent_DU < Way forward > FFS the following proposals: - Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Proposal 1a: It can cover both cases: - UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation - Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met. - Proposal 2: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. - Proposal 3: Wait for more RAN2 agreements.

RAN2 agreed that UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. Hence, we can reuse the definition of TEvent_DU in CHO for CLTM.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM command/configuration until the execution condition becomes met over the air.

Issue 1-3-6: Tmeasure < Way forward > FFS the following proposals: - Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - Proposal 1a: - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on CSI-RS based L3-RSRP measurement. - Proposal 1b: - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure. - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

As part of its latest agreements, RAN2 stated that “For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).”

The conditions supported for CLTM according to the above agreement are defined as follows.

- L1-based conditions:

  - LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell

  - LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2

- L3-based conditions:

  - CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell

  - CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2

As can be seen from these definitions, UE may need to measure both the serving cell/beam of the serving cell and the candidate cell/beam of the candidate cell to be able to realize that the condition is met. However, we do not see a need to distinguish between these cases specifically in the definition of Tmeasure.

When the CLTM execution condition is L3-based, the different measurements are already considered in the L3 measurement delay. Hence, the duration of Tmeasure in CLTM can be defined in the same way as for CHO.

When the execution condition is L3-based, Tmeasure in CLTM to follow the definition of Tmeasure in CHO.

When the CLTM execution condition is L1-based, the existing L1 measurement delay (SSB-based in Rel-18 and CSI-RS-based in Rel-19) can be used as the baseline for the definition of Tmeasure. However, Treport is to be set to zero.

When the execution condition is L1-based, Tmeasure in CLTM to be defined using the existing L1 measurement period for serving cell and candidate cells, with Treport = 0.

Issue 1-3-7: TCLTM_execution / TLTM_execution < Way forward > FFS the following proposals: - Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Proposal 1a: TLTM_execution can be up to 10ms. - Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay.

In Proposal 1 in the above issue, TCLTM_execution is defined as the equivalent of TCHO_execution, which is the UE execution preparation time for conditional handover. TCHO_execution starts after UE realizes the condition of CHO is met and identity of the target cell is determined.

In Rel-18 LTM cell switch delay, TLTM_RRC-processing represents the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. Considering both TCLTM_execution and TLTM_RRC-processing in the CLTM delay leads to duplicating the UE execution preparation time. Hence, we propose the UE execution preparation time to be covered by TCLTM_RRC-processing in the CLTM cell switch delay.

TCLTM_RRC-processing covers the delay related to the processing of the CLTM candidate cell configuration. There is no need to add an additional UE execution preparation time to the CLTM cell switch delay.

Subsequent Conditional LTM

Issue 1-4-2: The starting point of Subsequent conditional LTM delay < Agreement > ad hoc - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM]. - Revisit if RAN2 has more progress.

RAN2 agreed that UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. At least based on the current RAN2 status, this agreement also applies for subsequent cell switch. Hence, we here revisit the RAN4 agreement made in last meeting regarding the starting point of subsequent conditional LTM delay.

The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous cell switch.

Issue 1-4-3: The components of subsequent conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

We propose the following definition for the subsequent CLTM cell switch delay.

The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell:

- DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, 
Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU

Regarding how to capture the subsequent conditional cell switch delay, we think this can be merged to the same section with the initial CLTM cell switch delay. The only difference is the lack of TRRC parameter and the starting point, so there is no need to define a separate section to indicate this difference.

Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3.

Other aspects of CLTM condition evaluation and measurements

For condition evaluation, CLTM is expected to follow the measurement requirements defined for Rel-18 LTM. This means that at early TCI state activation for one (or more) candidate cell(s), L1 measurement requirements for other candidate cells stop applying, i.e., the UE is not required to perform L1 measurements on the candidate cells without active TCI states.

Hence, if the UE is configured with L1-based CLTM condition for a candidate cell without an active TCI state, the related measurement requirements are undefined, and it is unclear whether the UE will continue condition evaluation for this cell.

In Rel-18 L1 measurement requirements, after TCI state activation the UE is allowed to prioritize measurements on candidate cells with active TCI states and hence there are no requirements for L1 measurements on candidate cells without active TCI states. Therefore, it is unclear whether the UE continues L1-based condition evaluation (if configured) on the cells without active TCI states.

To avoid any uncertainty in condition evaluation and the whole CLTM procedure, RAN4 needs to discuss how to take CLTM condition configuration into account in the L1 measurement requirements. If UE continues to evaluate a L1-based condition on a cell without active TCI states, measurement requirements are needed for these cells so that the evaluation delay (Tmeasure) can be defined and tested. If measurement requirements are not defined, RAN4 needs to confirm whether this means that at the time of TCI state activation, the UE stops also the evaluation of the corresponding L1-based CLTM conditions for candidate cells without active TCI states.

RAN4 to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: RAN4 not to define CLTM measurement requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based. Consequently, conditional cell switch to a deactivated SCell based on L1 measurements is not to be supported.

Proposal 2: If RAN4 agrees to define requirements for UE based TA estimation in Rel-19 (Option 2), at least the issues listed below need to be taken into account.

	Requirements to cover not only Rel-19 CLTM but ideally also Rel-18 LTM.

	Which reference signals are expected to be used when performing UE-based TA measurements?

	How long does the acquisition of the UE-based TA take after candidate cell configuration?

	UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

	Existing transmit timing accuracy requirements shall apply.

	How to ensure the accuracy of the UE-estimated TA?

Proposal 3: If the issues in Proposal 2 cannot be solved within Rel-19, then Option 3 is preferred.

Observation 1: Early TCI state activation may in most cases benefit interruption reduction for CLTM cell switch only if condition becomes met and cell switch is triggered within 160 ms/480 ms after TCI state activation is completed.

Proposal 4: RAN4 to discuss how to ensure usefulness of early TCI state activation for CLTM cell switch interruption reduction in the definition of Tfirst-RS and TRS-proc.

Proposal 5: Consider either removing the limitation of 160/480 ms for DL sync after TCI state activation or defining a UE capability to indicate UE is able to keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160 ms with L1 measurements, or for longer than 480 ms/160 ms with L3 measurements.

Proposal 6: If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS and TRS-proc are zero based on successful preamble transmission and while the TAT is running.

Proposal 7: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 8: The CLTM cell switch delay DCLTM is the delay from the end of the last TTI containing the RRC command for CLTM until the time the UE transmits the first UL message on the target cell:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU

Proposal 9: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

Proposal 10: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM command/configuration until the execution condition becomes met over the air.

Proposal 11: When the execution condition is L3-based, Tmeasure in CLTM to follow the definition of Tmeasure in CHO.

Proposal 12: When the execution condition is L1-based, Tmeasure in CLTM to be defined using the existing L1 measurement period for serving cell and candidate cells, with Treport = 0.

Proposal 13: TCLTM_RRC-processing covers the delay related to the processing of the CLTM candidate cell configuration. There is no need to add an additional UE execution preparation time to the CLTM cell switch delay.

Proposal 14: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous cell switch.

Proposal 15: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell:

DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt,  Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU

Proposal 16: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3.

Observation 2: In Rel-18 L1 measurement requirements, after TCI state activation the UE is allowed to prioritize measurements on candidate cells with active TCI states and hence there are no requirements for L1 measurements on candidate cells without active TCI states. Therefore, it is unclear whether the UE continues L1-based condition evaluation (if configured) on the cells without active TCI states.

Proposal 17: RAN4 to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell.

- References

[1] R4-2405533, On remaining aspects of LTM core requirements, Nokia, RAN4#110bis, Changsha, China, April 15-19, 2024.