3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504204

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.24.4

Source: 	OPPO

Title: 	On OTA aspect

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the OTA aspect is provided in this paper.

- Discussion

During the RAN4#114 meeting, the agreed WF has been captured as below:

Issue 4-1-1: Scope of OTA test methodology

Agreement:

- For OTA test, the starting point of test methodology for Ambient IoT includes

  - Chamber setup

  - Position of Device

  - Test procedures

  - Minimum range/ QZ (Quiet Zone) size/QZ calibration

  - Preliminary MU assessment

- Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop test methodology.

A test setup for example is as below from regulation for RF ID in figure 1. There are many parts that needs to be settled down as:

-Distance between measurement receiver and device

-Distance between device and CW node

-The CW node antenna maximum gain direction

-The measurement antenna should be max E.R.P

The first two distance can be agreed while the later two might have problem since the regulation defines E.R.P requirement as a max directional requirement while it is still not agreed yet whether a TRP requirement will be used finally.

Proposal 1: To define the distance between measurement receiver and device and the distance between device and CW node.

From pure testing perspective, the max E.R.P and TRP all need to test the full sphere and one is to find the maximum point and the other one is to mark down all the values in each point and do calculation in the end. Hence the test time will be no different. However, as there is more sparse grid for TRP scanning and improving test time, TRP seems a time saving way. But finally this will depend on the agreement of RF requirements.

Observation 1: From pure testing perspective, TRP will save time.

Figure 1 RF ID test setup

Currently the TRP method and corresponding anechoic chamber test setup is quite stable as being discussed fully in RAN4. The test setup for UE OTA testing in anechoic chamber can be a starting point while further to take the CW node into consideration is critical.

Proposal 2: Take the anechoic chamber TRP test method as starting point and further consider the CW node.

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Observation 1: From pure testing perspective, TRP will save time.

Proposal 1: To define the distance between measurement receiver and device and the distance between device and CW node.

Proposal 2: Take the anechoic chamber TRP test method as starting point and further consider the CW node.

4 References

[1] R4-2502858	WF on requirements for system parameters and A-IOT device, CMCC