3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504205

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.24.2.3

Source: 	OPPO

Title: 	On RF requirements for CW

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the RF requirements for CW is provided in this paper.

- Discussion

During the RAN4#114 meeting, the agreed WF has been captured as below:

| CW RF Requirement | Potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD |
| Frequency error | YES |
| Phase noise | TBD for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD |

Issue 3-1: CW Output power (for ahoc discussion)

  - CW output power [33] dBm.

Issue 3-2: ON/OFF power

  - FFS for OFF power

Issue 3-3: Frequency error

  - Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW.

Issue 3-4: Phase noise

  - Further discuss phase noise requirements

Issue 3-5: Unwanted emission

  - Specify emission requirements, values FFS

Issue 3-6: Spurious emissions

  - NR spurious emission requirement can be considered as baseline for R19 AIoT CW node

Issue 3-7: Intermodulation

  - FFS

Firstly, for the CW output power, 33dBm has been proposed during the ad-hoc meeting discussion. This is because the GSM has already used this high power in band 8 and hence no regulation problem will occur. However, we might keep in mind that this should apply only for band n8 and for other example bands, the CW output power needs to be further discussed. Furthermore, during the discussion for device output power, the E.R.P output power requirement has been proposed in our companion paper, for CW node, the 33dBm should be a conducted requirement.

Proposal 1: 33dBm output power is conducted power and only apply for band n8.

For the on/off power, this is to guarantee when the node is turned off that no much power will emit into the air. In our mind, a CW node in the inventory will be an always on status so that different devices can reflect the CW to produce D2R link, In such case, this on/off power requirement seems not necessary. If companies really want to define such requirement, we believe the off-power same as BS is appropriate and the time mask for on/off is not needed.

Proposal 2: For on/off power, same requirement as BS can be used and the on/off time mask is not needed.

For the unwanted emission, as companies agree to introduce this requirement, we give further discussion on such topic. Firstly, the channel bandwidth of a CW signal is not an appropriate requirement to be used and hence the out of band unwanted emission is also not a good choice as it usually depends on the channel bandwidth. As proposed by companies in the last meeting, the phase noise can be another choice. Usually this requirement is defined with frequency offset. A normal CW phase noise requirement is defined as around -110 to -160 dBc/Hz at 10kHz offset. This can be a requirement together consider the phase noise as well as the unwanted emission.

Proposal 3: A -110 to -160 dBc/Hz at 10kHz offset can be a requirement together consider the phase noise as well as the unwanted emission.

3	Conclusions

In this paper, we give discussion on the CW RF requirement, the observations and proposals are as below:

Proposal 1: 33dBm output power is conducted power and only apply for band n8.

Proposal 2: For on/off power, same requirement as BS can be used and the on/off time mask is not needed.

Proposal 3: A -110 to -160 dBc/Hz at 10kHz offset can be a requirement together consider the phase noise as well as the unwanted emission.

4 References

[1] R4-2502859	WF on requirements for A-IoT BS and CW, Moderator (Huawei)