3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504206

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.24.2.2

Source: 	OPPO

Title: 	On RF requirements for device 1

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the system parameter is provided in this paper.

- Discussion

During the RAN4#114 meeting, the agreed WF has been captured as below:

Issue 3-2-1: Transmit output power

Agreement:

- Measurement configuration including input CW power and potential other conditions need to be defined.

Issue 2-1-1: Reference sensitivity level

Agreement:

- Define the reference sensitivity level specifically for 15 kHz D2R channel BW at least,

  - Consider the follow aspects

    - Noise figure

    - SNR

    - Coverage

- FFS on the definition of channel bandwidth depending on RAN1 conclusions.

Firstly, for the transmit output power, during the discussion there are two perspective discussed as one is the reflection efficiency and the other one is the exact reflected power. The reflection efficiency mainly considers the input power and output power ratio of the device and the reflected power mainly considers the absolute power that the device transmitted or reflected. From our point of view, we believe these two requirements can be tested in one OTA test setup and hence a unified requirement can be defined with another one might be implicitly included.

Observation 1: The reflection efficiency and the exact reflected power can be tested in one OTA test setup.

Proposal 1: A unified requirement can be defined.

Looking back to the RF ID test setup as shown below in figure 1, the distance between tag and reader RX is 1m, the CW distance to the tag is 20cm. The requirement is 33dBm E.R.P in 200kHz channel bandwidth in band 865 to 868MHz and 36dBm E.R.P in 400kHz channel bandwidth in band 915MHz to 921MHz. The D2R channel bandwidth is proposed to be 15kHz in our companion paper. The CW node output power is proposed to be 33dBm as agreed in the last meeting. The band n8 UL is set as 925MHz to 960MHz UL. Together consider this, it is proposed to use 36dBm E.R.P as the output power limit while scaling to 15kHz it should be 21.74dBm per 15kHz channel bandwidth.

Proposal 2: To define the output power of device as 22dBm E.R.P over 15kHz channel bandwidth.

Figure 1 RF ID test setup from regulation

For the REFSENS requirement, in our companion contribution for system parameter, we have proposed to define 15kHz channel bandwidth for D2R. Hence the REFSENS requirement will be based on 15kHz CBW. Furthermore, the REFSENS requirement has been considered to use the equation as NR receiver with:

REFSENS = -174 + 10log(CBW) + NF + SNR + IM

The LPWUS NF + IM is agreed in the last RAN4 meeting for OOK based receiver with 17.5dB. For A-IOT with even less power consumption for device 1, a relaxed NF+IM as 20dB might be used. The SNR is derived from the figure 1 as shown below for 10% BLER with FEC with -5.5dB. As illustrated above, 15kHz CBW will be used for D2R. With the above numbers, the final calculated REFSENS is -117.7dBm. Compare to the number we used for co-existence simulation as -36dBm which has quite large gap. During the co-exist simulation study, when calculating the pathloss between the device and reader, if it is larger than -36dBm then it is assumed the device will not be connected to such reader. In such case, to avoid test burden, it is suggested to follow similar as the RF ID case, to let the vender to declare the REFSENS for the device 1.

Observation 2: The calculated REFSENS has large gap comparing to the value used for co-exist simulation.

Proposal 3: Use declaration method for the REFSENS for device 1.

Figure 1 BLER and SNR for device

For other RX requirements, as we proposed in our previous meeting discussion paper, we believe it is not needed especially for device 1.

Proposal 4: For device 1 RX requirement, only define REFSENS requirement.

3	Conclusions

In this paper, we give discussion on the device 1 RF requirement, the observations and proposals are as below:

Observation 1: The reflection efficiency and the exact reflected power can be tested in one OTA test setup.

Observation 2: The calculated REFSENS has large gap comparing to the value used for co-exist simulation.

Proposal 1: A unified requirement can be defined.

Proposal 2: To define the output power of device as 22dBm E.R.P over 15kHz channel bandwidth.

Proposal 3: Use declaration method for the REFSENS for device 1.

Proposal 4: For device 1 RX requirement, only define REFSENS requirement.

4 References

[1] R4-2502858	WF on requirements for A-IoT device, CMCC