3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504208

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.4.1.3

Source: 	OPPO

Title: 	on non-collocated other aspects

Document for:	Approval

- Introduction

During the RAN4#113 meeting, the UE architecture and UE type definition has been fully discussed and an LS has been sent out to RAN2 to clarify the type 2 UE definition [1]. Still there is some ambiguity and we would like to further discuss this issue.

- Discussion

In our previous meeting discussion paper, the UE architecture has been fully discussed and captured again below:

Figure 1 LTE intra-band NC CA baseline RF architecture

Figure 2 NR intra-band NC CA baseline RF architecture

Based on the previous meeting discussion, there is no need to further discuss the UE type in the future RAN4 meeting. Hence couple of CRs for TS 38.101-1 and TS 38.101-3 have been agreed during the last meeting and makes the requirement application clear. In this paper, we try to give further discussion when we try to introduce the intra-band non-collocated feature into handheld UE, which might be a way forward in the future release. From our understanding, currently the type 2 and type 4 UE architecture has no ambiguity that separate RF chain and BB chain without any sharing of components. But for the other types, say type 1 and type 3 might have ambiguity when comparing to the intra-band NC CA default architecture which is shared LNA for different CCs as illustrated in figure 1 and figure 2 above. In such case, we would like to provide the future wording that no to use type 1 and type 3 in the future since it will cause ambiguity. A most simple way to name the UE type is handheld UE supporting intra-band non-collocated feature. If we do really want to investigate and introduce such feature for handheld UE when this can be a name representing the legacy understanding of “type 3”. After that, whether to keep investigate the UE architecture and to see if it is different from figure 2 or not can be left to future WID discussion.

Observation 1: There is no ambiguity for UE architecture for type 2 and type 4 but not for type 1 and type 3.

Proposal 1: Use “handheld UE supporting intra-band non-collocated feature” for future discussion.

3	Conclusions

In this paper, we give discussion on the UE architecture and potential future work on up scoping the WID, the observations and proposals are as below:

Observation 1: There is no ambiguity for UE architecture for type 2 and type 4 but not for type 1 and type 3.

Proposal 1: Use “handheld UE supporting intra-band non-collocated feature” for future discussion.

4 References

[1] R4-2503033	LS on clarification for intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16, Apple