3GPP TSG RAN WG4 Meeting #114bis		R4-2504214

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.5.3

Source: 	MediaTek Inc.

Title: 	Further discussion of impacts on UE RF requirements and DL performance impacts

Document for:	Approval

1. Introduction

The performance degradation when one Rx RF chain is enabled had been discussed in previous RAN4 meeting and concern on PCC degradation were raised. A possible alternative, PCC/SCC swapping, was also considered. In the contribution, we provide RF performance evaluation on the alternative approach.

2. Discussion

Impact on ΔRIBNC for both PCC and SCC when operating under one Rx RF chain mode

2.1 Analysis on Band n25 with PCC SCC Swapping

The existing requirements of n25(2A) is copied below as reference:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

In the last RAN4 meeting, concerns on PCC performance degradation were raised and an alternative approach was also provided. In the network scheduling control, PCC SCC can be swapped to make Tx-Rx frequency gap larger to reduce performance degradation. The spectrum plot is illustrated below Figure 1. With the swapping, it is expected the Rx performance degradation can be much reduced.

Figure 1. Illustration on PCC/SCC swapping for the two cases for DL NCCA test

The evaluation results are as following.

- Case 1, 5 + 5 MHz

Based on the agreed assumptions and reference table format,

Evaluation results on PCC/SCC swapping are in the updated table below:

Table 1. Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -96.5 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -101.5 | -101.5 | / | / |
| LPF attenuation | 22.2 | 1 | 1 | / | / |
| LPF output (dBm) | -45.2 | -102.5 | -102.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 4 | / |
| Noise floor | / | / | / | -97.5 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -101.5 | / | / |
| ADC output (dBm) | / | / | / | / | -112.3 |
| SNR after ADC (dB) | / | / | / | / | -4.0(PCC) |
|  |  |  |  |  | -3.4(SCC) |

- Case 2, 40 + 5 MHz

The REFSENS of 40MHz is not linear scaling from 5MHz in existing specs. Excessive Tx leakage level fall in the DL receiving channel is calculated in the table. The evaluation parameters and results are demonstrated in below table.

Table 2. Link budget of Self-interference for 40+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -79.5 | -89.3 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -84.5 | -94.3 | / | / | / |
| LPF attenuation | 11.1 | 1 | 1 | / | / | / |
| LPF output (dBm) | -34.1 | -85.5 | -95.3 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -92.5 | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 7.2 | 7.2 | / |
| Noise floor | / | / | / | -85.3 | -94.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -94.3 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -92.2 |
| SNR after ADC (dB) | / | / | / | / | / | -2.2(PCC) |
|  |  |  |  |  | / | -8(SCC) |

2.2 Summary results for band n25 with PCC/SCC swapping

Evaluated results are summarized in the table below:

Table 9. Proposed ΔRIBNC for example band n25 on Rx RF chain mode with P/S cell swapping

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.8 4.0 | 14.3 3.4 | FDD |
|  |  |  | Wgap = [10 20] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 2.2 | 24.9 8 | FDD |

Observation1: It can be observed if P/S cell are swapped, the REFSENS degradation can be lower to ~4dB for 5+5MHz case. For 40+5MHz case, the degradation can be lower to 2~8dB.

Proposal 1: Agree on DL fragmented carrier using one Rx RF chain performance evaluation results in the following TP contents for TR38.755

3. Conclusion

Observation1: It can be observed if P/S cell are swapped, the REFSENS degradation can be lower to ~4dB for 5+5MHz case. For 40+5MHz case, the degradation can be lower to 2~8dB.

Proposal 1: Agree on DL fragmented carrier using one Rx RF chain performance evaluation results in the following TP contents for TR38.755

TP for TR38.755

<Start of proposed TP>

7	Study on RF requirements for the inter-operator co-located BS scenario

<Start of proposed TP>

<Unchanged parts are omitted>

7.1.1a Analysis on Band n25 with PCC SCC Swapping

In the last RAN4 meeting, concerns on PCC performance degradation were raised and an alternative approach was also provided. In the network scheduling control, PCC SCC can be swapped to make Tx-Rx frequency gap larger to reduce performance degradation. The spectrum plot is illustrated below Figure 1. With the swapping, it is expected the Rx performance degradation can be much reduced.

Figure 1. Illustration on PCC/SCC swapping for the two cases for DL NCCA test

The evaluation results are as following.

- Case 1, 5 + 5 MHz

Based on the agreed assumptions and reference table format,

Evaluation results on PCC/SCC swapping are in the updated table below:

Table 1. Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -96.5 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -101.5 | -101.5 | / | / |
| LPF attenuation | 22.2 | 1 | 1 | / | / |
| LPF output (dBm) | -45.2 | -102.5 | -102.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 4 | / |
| Noise floor | / | / | / | -97.5 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -101.5 | / | / |
| ADC output (dBm) | / | / | / | / | -112.3 |
| SNR after ADC (dB) | / | / | / | / | -4.0(PCC) |
|  |  |  |  |  | -3.4(SCC) |

- Case 2, 40 + 5 MHz

The REFSENS of 40MHz is not linear scaling from 5MHz in existing specs. Excessive Tx leakage level fall in the DL receiving channel is calculated in the table. The evaluation parameters and results are demonstrated in below table.

Table 2. Link budget of Self-interference for 40+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -79.5 | -89.3 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -84.5 | -94.3 | / | / | / |
| LPF attenuation | 11.1 | 1 | 1 | / | / | / |
| LPF output (dBm) | -34.1 | -85.5 | -95.3 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -92.5 | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 7.2 | 7.2 | / |
| Noise floor | / | / | / | -85.3 | -94.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -94.3 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -92.2 |
| SNR after ADC (dB) | / | / | / | / | / | -2.2(PCC) |
|  |  |  |  |  | / | -8(SCC) |

Summary results for band n25 with PCC/SCC swapping

Evaluated results are summarized in the table below:

Table 9. Proposed ΔRIBNC for example band n25 on Rx RF chain mode with P/S cell swapping

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.8 4.0 | 14.3 3.4 | FDD |
|  |  |  | Wgap = [10 20] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 2.2 | 24.9 8 | FDD |

Observation1: It can be observed if P/S cell are swapped, the REFSENS degradation can be lower to ~4dB for 5+5MHz case. For 40+5MHz case, the degradation can be lower to 2~8dB