3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504216

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:	Further discussion on beam management core requirements

Agenda item:		7.19.3

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2], as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In this contribution, we would like to further discuss on core requirement for AI-based beam management use cases, including the impact on both BM-Case1 and BM-Case2.

# 2. RAN4 Core Requirement Impact for NW-Sided Model

In our previous contribution, we provided more detailed analysis on RAN4 core requirement impact for NW-sided model. In short, we identified the potential RAN4 impact due to the potentially RAN1 introduced procedure of data collection (for inference) and the new performance monitoring mechanism.

However, it should be noted that there is RAN4 agreement on UE reporting for network side models, i.e., “postpone discussion until other WGs define any reporting mechanisms”. Since there is no RAN1 progress on the UE reporting mechanism since RAN1#118 (August 2024), we think it is reasonable to postpone the discussion on RAN4 core requirement impact for NW-sided model.

Observation 1: For NW-sided model, there is no RAN1 progress on the UE reporting mechanism since RAN1#118 (August 2024), and it is reasonable to postpone the discussion on RAN4 core requirement impact for NW-sided model.

# 3. RAN4 Core Requirement for supporting UE-Sided Model

For UE-sided model for AI-BM, we provide our analysis on RAN4 core requirement impact for UE-sided model, for the procedure of data collection, additional assistance information, inference, performance monitoring and LCM.

## 3.1 Data collection for UE-sided model (for model training)

Based on the above contents from TR 38.843, the potential interaction(s) between UE and NW for data collection for UE-sided model could be: (1) UE reporting of the supported/preferred DL RS configuration; (2) trigger/initiating data collection; (3) assistance information from NW to UE for data collection.

As we elaborated in our previous RAN4 contribution [17], we found that RAN4 shall confirm that no RAN4 impact is expected from data collection for UE-sided model training.

Proposal 1: RAN4 shall confirm that no RAN4 impact is expected from data collection for UE-sided model training.

## 3.2 RRM core requirement for model Inference

Based on the existing discussion, seems the group have the common understanding to specify the requirement for model inference for UE-sided model. Specifically, we expect corresponding RRM requirement shall be introduced: to guarantee (1) the accuracy of inference results accordingly to the agreed KPI; (2) the delay of reporting inference results (containing the inference latency).

Observation 2: RRM requirement shall be introduced to guarantee UE-side AI/ML model inference to guarantee (1) the accuracy of inference results accordingly to the agreed KPI and (2) the delay of reporting inference results (containing the inference latency).

### 3.2.1 Predicted absolute RSRP accuracy

In previous RAN4 discussion, the definition of the absolute RSRP accuracy is provided:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In RAN4#114, RSRP accuracy is agreed to one of the KPIs if RSRP is contained in the report, and the agreement shall be applicable to both Case 1 and Case 2:

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Accordingly, the following proposal is given for RSRP accuracy issue.

Proposal 2: For absolute RSRP accuracy requirement, the requirement shall be defined as:

- The absolute RSRP accuracy requirement shall be applied to all beams in the configured top-K predication.

- The existing measurement accuracy requirement is used as the baseline for prediction accuracy.

### 3.2.2 The necessity of predicted relative RSRP accuracy

There is some proposal to have relative RSRP accuracy, however, we found the definition of relative RSRP is misleading: In the existing RAN4 definition of relative RSRP accuracy, the “relative RSRP” (taking SS-RSRP as example) is defined as “the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1”.

In the case of AI-BM (no matter for Case 1 and Case 2), it should be noted that there are two root causes resulting the relative RSRP is not accurate: (1) Root Cause-1: predicted relative RSRP between target beam (i.e., beam a2 in below Figure 1) and optimal beam (i.e., beam a1) is estimated wrongly (larger or smaller), even if the beam index ordering is correct. (2) Root Cause-2: the optimal beam used as reference for relative RSRP is estimated wrongly (as illustrated in below Figure 2)  the beam index with maximum RSRP among Set A could be not be the optimal beam if conducting measurement over Set A, therefore the RSRP level

Figure 1: Illustration of Root Cause-1 for relative RSRP error: Predicted relative RSRP between target beam and optimal beam is estimated wrongly

Figure 2: Illustration of Root Cause-2 for relative RSRP error: the optimal beam used as reference for relative RSRP is estimated wrongly

Observation 3: There are two root causes for relative RSRP error: (1) Predicted relative RSRP between target beam and optimal beam is estimated wrongly, and (2) The optimal beam used as reference for relative RSRP is estimated wrongly.

Accordingly, we can see that the relative RSRP only make sense when the optimal beam used as reference for relative RSRP is estimated wrongly. If not, even the estimated relative RSRP is not far from the groundtruth, as illustrated in Figure 2, we still can claim it is an accurate relative RSRP prediction. Therefore, by realizing the difference between relative RSRP by measurement and by AI-BM, we suggest the relative RSRP accuracy requirement is not applicable.

Proposal 3: In Rel-19 AI-BM, RAN4 shall not introduce any relative RSRP accuracy requirement to any relative RSRP reporting.

### 3.2.3 Beam prediction accuracy

In previous RAN4 discussion, For the metrics/KPI discussion, the following options are given in the Rel-18 study item and captured in TR [1], in which beam prediction accuracy is included:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Considering the dilemma of choosing Option 2 and Option 3, we see the merits from both options. Option 2 provide the beam prediction accuracy by guarantee the correctness of beam ordering, while Option 3 can guarantee the predicted RSRP from top-K predicted beams is not far away from the groundtruth RSRP of strongest beam. It should be noted that RAN1 has agree that for Option 2 (UE-assisted performance monitoring), “at least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring”, as provided below:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

To have the alignment between RAN1 and RAN4, it is reasonable RAN4 to adopt the same metric as RAN1 for UE-assisted performance monitoring.

Proposal 4: If RAN4 still can’t independently conclude on Option 2 and 3 for beam prediction accuracy metrics, RAN4 shall stop the discussion to wait RAN1 conclusion on the KPI for UE-assisted performance monitoring.

### 3.2.4 Prediction delay requirement including measurement period

The legacy L1-RSRP measurement requirement, which is based on one-shot L1 measurement, establishes a relationship between the measurement period and accuracy. Similar to the L1 measurement requirement, the measurement period for AI-BM prediction (for AI-BM Case 1) should first be determined. It is logical to use the legacy measurement period as a baseline and then assess the achievable prediction accuracy accordingly.

Proposal 5: For AI-BM Case-1, the legacy L1 measurement period is used as the starting point for Set-B measurement to evaluate the achievable prediction accuracy.

For AI-BM Case 2, the UE may perform multiple L1-RSRP measurement occasions on Set-B prior to AI-based prediction. The exact number of these measurement periods/occasions could be configurable and will need to be studied and defined in RAN1.

Proposal 6: For AI-BM Case-2, the exact number of the measurement periods/occasions shall be configurable and subject to further study in RAN1.

## 3.3 RRM core requirement for performance monitoring

Regarding the performance monitoring for UE-sided model, key issues discussed in RAN1 include: performance metrics, benchmark/reference for the performance comparison, and monitoring type. Corresponding analysis for potential specification impact for performance metrics and benchmark/reference for the performance comparison is provided below from physical layer perspective:

For the performance monitoring of BM-Case1 and BM-Case2: - Performance metric(s) with the following alternatives: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Benchmark/reference for the performance comparison, including: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

In the normative phase, based on RAN1’s down-selection on performance metric(s) and benchmark/reference for the performance comparison, RAN4 requirement on performance monitoring can be specified accordingly. Specifically, RAN1 has provided two types of performance monitoring, i.e.,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Particularly, Type 1 performance monitoring is performed in the NW-side, for which Option 1 and Option 2 differ in which entity shall perform the calculation of the performance metric(s). On the other hand, Type 2 performance monitoring is performed in the UE-side, while no feedback is required.

In RAN1#117 and #118bis, the following agreement is achieved, particular for Type-1 (Option 2) performance monitoring:

Agreement from RAN1#117 For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement from RAN1#118bis For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

For the configuration for monitoring, based on the two options given in RAN1#118bis, Option 2 (dedicated resource set(s)) are agreed to be supported:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Based on the above understanding on Type 1 and Type 2 performance monitoring, the following proposal is obtained.

Proposal 7: At least for performance monitoring Type 1, Option 2 (UE-assisted performance monitoring), RAN4 prediction accuracy requirement shall be applicable.

## 3.4 RRM core requirement for LCM

Based on the above analysis for different types of performance monitoring for UE-sided model (NW-side or UE-side performance monitoring), the model/functionality control can be performed by the UE or gNB:

- Management: - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the gNB when the monitoring resides within the gNB or UE.

Based on the agreement obtained in RAN2#125bis, the “network decision, network-initiated” AI/ML management is supported as a baseline:

Agreements from RAN2#125bis:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In RAN2#126, the following agreements are achieved on LCM for UE-sided model:

Agreements from RAN2#126

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

In RAN1#119, the signalling procedure on applicable functionality reporting for beam management UE-sided model has been discussed to response RAN2 LS [R2-2407848], in which the following 5 steps signaling procedure (see the below figure) are agreed on applicable functionality reporting for BM in RAN2 while detailed understanding of Step 3-5 are provided by RAN1 [R1-2410898]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

After NW configure CSI-ReportConfig for inference configuration in RRCReconfiguration in Step 5, the applicable functionality may be activated, based on the above RAN1 agreement.

Observation 4: For UE-sided model configuration, after CSI-ReportConfig for inference configuration in RRCReconfiguration signalling in Step 5:

•	aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

•	periodic CSI Report is considered as activated after RRCReconfigurationComplete.

For L1/L2 signaling for activation/deactivation, “RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report” in Step 6.

Observation 5: For UE-sided model activation/deactivation, L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

Based on the above RAN1/2 agreement, we see the similarity between LCM and SCell configuration/activation. Accordingly, RAN4 shall study the necessity of delay requirement for model activation/deactivation, by adopting SCell activation/deactivation as baseline

Proposal 8: For UE-sided model “Network decision, network-initiated” LCM, RAN4 shall study the necessity of delay requirement for model activation/deactivation, by adopting SCell activation/deactivation as baseline.

# 4. Conclusion

In this contribution, we provided our viewpoints on core requirements for beam management in this contribution, accordingly the following observations and proposals are obtained:

RAN4 Core Requirements Impact for NW-sided Model

Observation 1: For NW-sided model, there is no RAN1 progress on the UE reporting mechanism since RAN1#118 (August 2024), and it is reasonable to postpone the discussion on RAN4 core requirement impact for NW-sided model.

RAN4 Core Requirements for Supporting UE-sided Model

Data collection for UE-sided model (for model training)

Proposal 1: RAN4 shall confirm that no RAN4 impact is expected from data collection for UE-sided model training.

RRM core requirement for model Inference

Observation 2: RRM requirement shall be introduced to guarantee UE-side AI/ML model inference to guarantee (1) the accuracy of inference results accordingly to the agreed KPI and (2) the delay of reporting inference results (containing the inference latency).

Proposal 2: For absolute RSRP accuracy requirement, the requirement shall be defined as:

- The absolute RSRP accuracy requirement shall be applied to all beams in the configured top-K predication.

- The existing measurement accuracy requirement is used as the baseline for prediction accuracy.

Observation 3: There are two root causes for relative RSRP error: (1) Predicted relative RSRP between target beam and optimal beam is estimated wrongly, and (2) The optimal beam used as reference for relative RSRP is estimated wrongly.

Proposal 3: In Rel-19 AI-BM, RAN4 shall not introduce any relative RSRP accuracy requirement to any relative RSRP reporting.

Proposal 4: If RAN4 still can’t independently conclude on Option 2 and 3 for beam prediction accuracy metrics, RAN4 shall stop the discussion to wait RAN1 conclusion on the KPI for UE-assisted performance monitoring.

Proposal 5: For AI-BM Case-1, the legacy L1 measurement period is used as the starting point for Set-B measurement to evaluate the achievable prediction accuracy.

Proposal 6: For AI-BM Case-2, the exact number of the measurement periods/occasions shall be configurable and subject to further study in RAN1.

RRM core requirement for performance monitoring

Proposal 7: At least for performance monitoring Type 1, Option 2 (UE-assisted performance monitoring), RAN4 prediction accuracy requirement shall be applicable.

RRM core requirement for LCM

Observation 4: For UE-sided model configuration, after CSI-ReportConfig for inference configuration in RRCReconfiguration signalling in Step 5:

•	aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

•	periodic CSI Report is considered as activated after RRCReconfigurationComplete.

Observation 5: For UE-sided model activation/deactivation, L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

Proposal 8: For UE-sided model “Network decision, network-initiated” LCM, RAN4 shall study the necessity of delay requirement for model activation/deactivation, by adopting SCell activation/deactivation as baseline.

# 5. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[13] R4-2416492, “Further discussion on beam management core requirements”, RAN#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418093, “Further discussion on beam management core requirements”, RAN#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN#113, Nov. 2024.

[17] R4-2501964, “Further discussion on beam management core requirements”, RAN#114, Feb. 2025.

[18] R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm, RAN#114, Feb. 2025.

# 6. Appendix – Existing Agreements on AI-BM

In RAN4#110bis, for the metrics/KPIs for Beam Management requirements/tests, the following agreement is achieved:

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111, for reported measurements and ground truth, the following agreements are achieved:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

And the following agreement is achieved in RAN4#112:

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms

And the following agreement is achieved in RAN4#112bis:

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

The following agreement is achieved in RAN4#113:

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. ### Agreements in ad-hoc session (R4-2420335): Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

The following agreement is achieved in RAN4#114:

- Agreements in ad-hoc session (R4-2503012) Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS ### Agreements in main session Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point