3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504217

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:	Study on testing issues for CSI compression two-sided models

Agenda item:			7.20.2

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

As per the discussions in RAN#105, the exploration of testability and interoperability challenges related to AI-driven CSI compression is set to continue within the remaining timeframe of Release 19, while the relevant objectives are included in a newly set-up study item [RP-243245]. In this contribution, we aim to further the discussion on testing issues for CSI compression two-sided models.

# 2. Discussion

## 2.1 Standardization for Test decoder Option 3

As per the discussions in RAN4#110bis, RAN4 is examining the feasibility of this approach in terms of standardization processes. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed procedure for Option 3 next steps captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Based on RAN4 discussion during/after RAN4#114, based on the reference decoder provided by some companies, most companies provided the trained test encoder based on different assumptions of track-1 and track 2 per requested in RAN4#113.

<Agreement in RAN4#113> Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

### 2.1.1 Track-1 (Private encoder-only training)

Based on the further alignment of quantization-aware training and quantization codebook, as agreed in RAN4#114 as below, we have further updated the reference decoders with and without 2bit quantization, which could be available at:

Directory Listing /ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Models/R4_114

<Agreement in RAN4#114> Agreements on quantization: Companies shall provide decoders with 2 bit quantization, and with no quantization - No quantization aware training when model is unquantized - Quantization aware training when model is quantized For the model with quantization: Agree on a scaling and quantization codebook - Include Sigmoid in model file in the encoder, and inverse Sigmoid in the decoder - Codebook (1/8, 3/8, 5/8, 7/8) in model file - Do not including mapping to 2 bits. Quantizer function of converting to 2 bits should not be in the model file For the model without quantization For no quantization models, do not include sigmoid at the model output and no inverse Sigmoid in the decoder Agreements regarding the input / output dimensionality: define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size define Encoder Output and Decoder Input as data dimension (n, 32) with data type float No additional inputs

By fixing the decoder (i.e., Samsung decoder in ONNX) and performing further training on different datasets provided by different companies, the corresponding encoders are provided with performance evaluated, from which the performance is provided for reference.

| Case | Training Method | Dataset | Encoder trained by | Decoder | SGCS of CNN with quantization | Source for SGCS results |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Joint en/decoder training | Samsung | Joint-trained by Samsung |  | 0.6459 | Reference purpose |
| 1 | Joint en/decoder training | Ericsson | Joint-trained by Ericsson |  | 0.6977 | R4-2416314 |
| 2 | Encoder-only training | Ericsson | Samsung | Samsung (ONNX) | 0.6762(-3.08%) | Evaluated by Samsung |
| 3 | Joint en/decoder training | Nokia | Joint-trained by Nokia |  | 0.705 | R4-2419178 |
| 4 | Encoder-only training | Nokia | Samsung | Samsung (ONNX) | 0.7027(-0.33%) | Evaluated by Samsung |
| 5 | Joint en/decoder training | OPPO | Joint-trained by OPPO |  | 0.717 | R4-2417618 |
| 6 | Encoder-only training | OPPO | Samsung | Samsung (ONNX) | 0.7160(-0.14%) | Evaluated by Samsung |
| 7 | Joint en/decoder training | vivo | Joint-trained by vivo |  | 0.732 | R4-2418748 |
| 8 | Encoder-only training | vivo | Samsung | Samsung (ONNX) | 0.7266(-0.74%) | Evaluated by Samsung |

Specifically, by adopting Samsung decoder to perform training on Ericsson, Nokia, OPPO and vivo provided datasets (accessible in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/
CSI_compression/Datasets/R4_113), we compared the SGCS performance (for CNN model with quantization) by

- Training method-1: Joint encoder/decoder training for Case 1, 3, 5, 7

- Training method-2: Only encoder training for Case 2, 4, 6, 8.

For a given dataset, by comparing the two training methods, the performance degradation percentage values of training method-2 (encoder-only training) from training method-1 (Joint en/decoder training) are provided.

Observation 1: In Option 3 Track-1 (private encoder-only training), over different datasets, the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is minor, i.e., 0.14%, 0.33%, 0.74%, 3.08% depending on different datasets.

Based on our experience of data training for CSI compression, as long as the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a certain dataset, the selected reference decoder shall be regarded as “containing the same feature” as this dataset.

Proposal 1: Based on the experience of AI-CSI compression with CNN-based neural network, the criteria to determine “whether the selected reference decoder contains the same feature as the specific dataset” is:

- The SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a the specific dataset.

Accordingly, with the above Proposal 1 into account, we can identify the selected reference decoder may contain different feature than Ericsson’s dataset.

Proposal 2: The above criteria in Proposal 1, RAN4 can identify the dataset may contain different feature from the one used for reference decoder training.

For the method of reference decoder (Option 3 decoder) to be used for private encoder-only training (i.e., fixing reference decoder, and performing training for encoder in a private company-A dataset), based on the above results, we can reach the following conclusion.

Proposal 3: For Option 3 (specified reference decoder), the feasibility of Track-1 “private encoder-only training” can be confirmed under the following conditions:

- The selected reference decoder shall be generalized by performing the training over the dataset with enough feature contained.

- The dataset used for private training should have the feature distribution aligned with the one used for reference decoder, as much as possible.

- Note: private encoder-only training is referred to: fixing reference decoder, and performing training for encoder in a private company-A dataset.

### 2.1.2 Track-2 (Encoder-only training over mixed dataset)

As agreed in RAN4#113, RAN4 has created a mixed dataset, which consisting of samples from Ericsson, Mediatek, Nokia, Oppo and Vivo’s datasets, in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/
NR_AIML_air/CSI_compression/Datasets/R4_114/MIXEDR4_114dsei00.zip.

Based on companies’ reported reference decoder SGCS performance, as summarized below,

|  | Unquantized | Quantized |
| --- | --- | --- |
| Apple | N/A | 0.7059 |
| CATT | 0.7727 | 0.7172 |
| Ericsson | 0.7593 | 0.6949 |
| MediaTek | 0.739 | 0.695 |
| Samsung | 0.7478 | 0.7016 |
| vivo | 0.7522 | 0.6962 |
| Huawei, Hisilicon | 0.76 | 0.71 |
| OPPO | 0.7742 | 0.7118 |
| Nokia | 0.7581 | 0.6983 |
| Qualcomm | 0.7614 | 0.683 |
| Average | 0.7583 | 0.7009 |
| Standard deviation | 0.0105 | 0.0099 |

Observation 2: In Option 3 Track-2 (encoder-only training over mixed dataset), companies provided decoders have quite similar SGCS performance, with standard deviation less than 0.01 for quantization case.

Based the trained decoders over the above-mentioned mixed dataset provided by companies after RAN4#114, Samsung decoder is selected as the reference decoder, and accordingly companies “companies are invited to train their own encoders using the mixed dataset or/and private dataset and test using the mixed test dataset”.

For Samsung provided decoder, which is available in

- SAM0R4_114mldc_track2_quantized.zip

- SAM0R4_114mldc_track2_unquantized.zip

The SGCS performance of the above trained decoder with and without quantization are summarized here:

| Quantization | Dataset | Encoder | Decoder | SGCS of CNN |
| --- | --- | --- | --- | --- |
| w/o | Mixed | Joint en/decoder training by Samsung |  | 0.7531 |
| w/o | Mixed | Encoder-only training | Samsung ONNX | 0.7478 (-0.7038%) |
| w/ | Mixed | Joint en/decoder training by Samsung |  | 0.7018 |
| w/ | Mixed | Encoder-only training | Samsung ONNX | 0.7016 (-0.0285%) |

In the above table, we demonstrated the SGCS performance by (1) joint encoder/decoder training and (2) encoder-only training by fixing the selected Samsung decoder, for both with and without quantization cases. With the above results taken into account, we reach the following observation especially for the method of (2) encoder-only training by fixing the selected Samsung decoder

Observation 3: In Option 3 Track-2 (encoder-only training over mixed dataset), a minor performance degradation can be expected for (1) encoder-only training by fixing the decoder, from (1) joint encoder/decoder training.

By recalling the above Proposal 1, which consider [1]% as the criteria to determine whether the selected reference decoder contains the same feature as the specific dataset, we can observe that the proposal 1 can be further confirmed by the mixed dataset results here.

In the next step, with the Samsung shared reference decoder (in ONNX format), we further train our own encoders by using the mixed dataset or/and Samsung dataset. In this procedure, the following two methods (retraining and refinement) are used:

| Encoder-only retaining | The encoder is initialized with random weights. |
| --- | --- |
| Encoder-only refinement | The encoder initialized is NOT with random weights but is the encoder trained jointly with the reference decoder using mixed dataset is used. |

For the dataset used for retraining/refinement, we prepared three datasets as follows:

| Samsung dataset | Samsung private dataset |
| --- | --- |
| Combined-dataset-Large (abbv. as Comb-L) | The dataset is constructed by containing all the 500k samples from the mixed dataset and 600k samples from Samsung private dataset. |
| Combined-dataset-Small (abbv. as Comb-S) | The dataset is constructed by containing the 300k samples from Mixed dataset and 300k samples of Samsung dataset. |
| Note: For both Combined-dataset-L and Combined-dataset-S, the samples are proportionally divided into training subset and validation subset. The validation subset contains samples from both Mixed dataset and Samsung dataset. |  |

By adopting the above retraining/refinement methods over three kinds of datasets, the SGCS performances are provided as below. The SGCS performances of joint en/decoder training on Samsung dataset are also provided as the benchmark.

| Quantization | Training dataset | Encoder trained by | Decoder | SGCS on mixed dataset | SGCS on Samsung dataset |
| --- | --- | --- | --- | --- | --- |
| w/o | Samsung | Joint en/decoder training by Samsung |  | N/A | 0.6872 |
|  | Mixed | Encoder-only retaining | Samsung ONNX | 0.7478 | N/A |
|  | Samsung | Encoder-only retaining | Samsung ONNX | N/A | 0.6755 |
|  | Comb-L | Encoder-only retaining | Samsung ONNX | 0.7588 | 0.6781 |
|  | Comb-S | Encoder-only retaining | Samsung ONNX | 0.7766 | 0.6744 |
|  | Samsung | Encoder-only refinement | Samsung ONNX | TBA | 0.6773 |
| w/ | Samsung | Joint en/decoder training by Samsung |  | N/A | 0.6459 |
|  | Mixed | Encoder-only retaining | Samsung ONNX | 0.7016 | N/A |
|  | Samsung | Encoder-only retaining | Samsung ONNX | N/A | 0.6277 |
|  | Comb-L | Encoder-only retaining | Samsung ONNX | 0.7041 | 0.6291 |
|  | Comb-S | Encoder-only retaining | Samsung ONNX | 0.7283 | 0.6265 |
|  | Samsung | Encoder-only refinement | Samsung ONNX | TBA | 0.6294 |

Observation 4: In Option 3 Track-2 (encoder-only training over mixed dataset), by using appropriate dataset in addition to the mixed dataset, the SGCS can be further improved, at least for the following method:

- Encoder-only retaining (The encoder is initialized with random weights).

We are also evaluating the methods of Encoder-only refinement, and we expect performance gain can also be achieved, while detailed results will be updated late.

## 2.2 Discussion on Test decoder Option 4

For test decoder Option 4, the following sub-options are agreed in RAN4#112 [13]:

-	4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

-	4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

-	4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

To make the progress, the following refinement of both Option 4a and 4b in RAN4#114 [18]:

The following update to the procedure description to option 4 is agreed (change marks show delta to WF from RAN4#113) Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3. - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - FFS what is the dataset assumed for the training - Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5 - FFS what is the dataset assumed for the training - Step 7: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

It should be noted that in RAN1 study, the following direction A, B and C are agreed in RAN1#118:

Agreement RAN1#118 Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) … - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues … - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues … Agreement RAN1#118bis For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Considering the above progress in RAN1, we see the similarity of Option 4a and 4b compared with Direction A 4-1 and 3a-1. However, at least the following difference between Option 4 in RAN4 and Direction A in RAN1 should also be noted:

Observation 5: Despite the similarity between Option 4 in RAN4 and Direction A in RAN1, the following difference should be noted:

- In Option 4-1 (Direction A in RAN1) sharing dataset is used to derive practical encoder (to be used by UE), while in Option 4a (in RAN4) sharing dataset is used to derive test decoder (to be used by TE).

- In Option 3a-1 (Direction A in RAN1) reference encoder is used to derive the practical encoder (to be used by UE), while in Option 4b (in RAN4) reference encoder is used to derive test decoder (to be used by TE).

Accordingly, we would like to provide the following proposal to clarify option 4:

Proposal 4: Before Option 3 feasibility is confirmed or disconfirmed, the discussion on Option 4 can be deprioritized, by considering the factors including

- to avoid parallel discussion with RAN1 Direction A down-selection.

- the study on the feasibility of Option 4b can be speed up if Option 3 is confirmed.

# 3. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI compression use case, accordingly the following observations and proposals are obtained:

Standardization for Test decoder Option 3

Observation 1: In Option 3 Track-1 (private encoder-only training), over different datasets, the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is minor, i.e., 0.14%, 0.33%, 0.74%, 3.08% depending on different datasets.

Proposal 1: Based on the experience of AI-CSI compression with CNN-based neural network, the criteria to determine “whether the selected reference decoder contains the same feature as the specific dataset” is:

- The SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a the specific dataset.

Proposal 2: The above criteria in Proposal 1, RAN4 can identify the dataset may contain different feature from the one used for reference decoder training.

Proposal 3: For Option 3 (specified reference decoder), the feasibility of Track-1 “private encoder-only training” can be confirmed under the following conditions:

- The selected reference decoder shall be generalized by performing the training over the dataset with enough feature contained.

- The dataset used for private training should have the feature distribution aligned with the one used for reference decoder, as much as possible.

- Note: private encoder-only training is referred to: fixing reference decoder, and performing training for encoder in a private company-A dataset.

Observation 2: In Option 3 Track-2 (encoder-only training over mixed dataset), companies provided decoders have quite similar SGCS performance, with standard deviation less than 0.01 for quantization case.

Observation 3: In Option 3 Track-2 (encoder-only training over mixed dataset), a minor performance degradation can be expected for (1) encoder-only training by fixing the decoder, from (1) joint encoder/decoder training.

Observation 4: In Option 3 Track-2 (encoder-only training over mixed dataset), by using appropriate dataset in addition to the mixed dataset, the SGCS can be further improved, at least for the following method:

- Encoder-only retaining (The encoder is initialized with random weights).

Discussion on Test decoder Option 4

Observation 5: Despite the similarity between Option 4 in RAN4 and Direction A in RAN1, the following difference should be noted:

- In Option 4-1 (Direction A in RAN1) sharing dataset is used to derive practical encoder (to be used by UE), while in Option 4a (in RAN4) sharing dataset is used to derive test decoder (to be used by TE).

- In Option 3a-1 (Direction A in RAN1) reference encoder is used to derive the practical encoder (to be used by UE), while in Option 4b (in RAN4) reference encoder is used to derive test decoder (to be used by TE).

Proposal 4: Before Option 3 feasibility is confirmed or disconfirmed, the discussion on Option 4 can be deprioritized, by considering the factors including

- to avoid parallel discussion with RAN1 Direction A down-selection.

- the study on the feasibility of Option 4b can be speed up if Option 3 is confirmed.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[12] R4-2416495, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#112bis, Oct. 2024.

[13] R4-2417207, “WF on data sharing for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418095, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN4#113, Nov. 2024.

[17] R4-2501966, “Study on testing issues for CSI compression two-sided models”, Samsung, RAN4#114, Feb. 2025.

[18] R4-2502953, “WF on AI/ML air interface study”, Ericsson, RAN4#114, Feb. 2025.

# 5. Appendix

Following the findings detailed in TR 38.843 [1], RAN4 has progressed its research through RAN4#110 and RAN4#110bis, with consensus and future directions to further the 2-sided model feasibility by executing the standardization procedure as documented in way forwards [4] [6]. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed agreements captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused Issue 4-2: Simulation results and alignment Agreement: - perform a round of alignment until RAN4#112-Bis - refined simulation parameters can be found in R4-2414447 Issue 4-4: Options for Option 4 Agreement: Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Based on another round of alignment and discussion in RAN4#112bis, the following agreements are provided [14]:

Issue 4-1: Simulation results discussion for Option 3 Agreement: Next step: Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format) Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

Based on the approved WF in RAN4#113, the following agreements are provided [16]:

Issue 1-2: Option 3 next step - decoder(s) selection Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Issue 1-3: Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)