3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504218

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:	Further discussion on general aspects for SBFD system

Agenda item:			7.23.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, PUSCH with resource muting for CLI handling, requirement applicability and BS RF specification structure for SBFD-capable BS.

# 2. RAN4 aspects for SBFD system parameters

#### 2.1 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based on the agreement given in RAN4#113, the option 2b is adopted to specify DL/UL subband configurations support for SBFD, with the following restrictions on SBFD configuration provided [11]:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

And the option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Here we would like to further discuss the remaining issues on the restrictions on SBFD configuration.

Restriction-1: For DUD case, the UL subband should be placed in the middle of channel bandwidth

During last meeting, for DUD case, it is agreed that the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD, in which DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB. There is no remaining issue for this restriction except how to implement this agreement into the specification.

Restriction-2: Restriction on the number of UL subband sizes supported

Based on the agreement in RAN4#113, for a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. Furthermore, it is still FFS different sizes for specific UL subband for DUD and DU/UD, and FFS the restriction is also applied to FR1 MR BS.

As the supported UL subband sizes are declaration based, and we see no harm to support maximum 3 specific UL subband sizes. Since the restriction of maximum UL subband sizes comes from the limited number of RF subband filtering implementations, we think the specific UL subband sizes should be applicable for DUD and DU/UD if supported by BS. Furthermore, based on the possible digital filtering design for FR1 MR BS, the restriction shall not be applied to FR1 MR BS.

Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- The specific UL subband sizes supported shall be the same for DUD and DU/UD if BS supports.

- This restriction shall be applied to FR1 WA BS only.

Restriction-3: Restriction on the number of DL subband sizes supported

Based on the agreement in RAN4#113, for a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case. It is still FFS the restriction is also applied to FR1 MR BS. Similar as previous restriction, based on the possible digital filtering design for FR1 MR BS, the restriction shall not be applied to FR1 MR BS.

Proposal 2: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction shall be applied to FR1 WA BS only.

#### 2.2 Restriction on guardband size

Since Option-2b is agreed in which it enable flexible DL subband size(s) and guardband size(s) depending on BS implementation, there is no need to impose restrictions on guardband size, including defining minimum or maximum RB limits.

Proposal 3: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

# 3. PUSCH with resource muting for CLI handling

During RAN1#119 (Nov. 2024), the LS on impact on transmit signal quality/MPR requirement is approved [12]. Based on the information provided by RAN1, RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR.

From RAN4 perspective, one of comparable waveform is almost contiguous allocation, for which extra maximum power reduction is allowed, if NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25 and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. But it should also be noted that the difference exists in (1) the number of muted REs is half of the total REs; (2) the muting is RE-based granularity, rather than RB-based granularity for unallocated resources in almost contiguous allocation. By keeping the above differences in mind, considering PUSCH with resource muting is similar as “almost contiugous allocation”, the detailed MPR could be needed.

Based on the existing analysis [R4-2501840, R4-2501943], the PUSCH with resource muting is comparable to DMRS multiplexing, for which no dedicated minimum RF requirement has been defined due to the common understanding about the limited impact on signal quality. Based on the analysis in [R4-2502250], for the waveform starting at the edge of channel bandwidth, the observed impact on the transmit waveform quality is minimal. Based on these observations, we expect this issue can be concluded in RAN4#114bis, i.e., no new MRP is required for PUSCH with resource muting for CLI handling, if no critical issue identified.

Proposal 4: The RAN4 evaluation on the signal quality impact from the new waveform can be concluded, i.e., no new MRP is required for PUSCH with resource muting for CLI handling, if no critical issue further identified.

# 4. Requirement applicability for SBFD-capable BS

Based on RAN4#114 discussion, RAN4 shall further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on.

As we provided in our previous contribution, the requirement applicable table is provided for SBFD capable BS, in which the requirement sets for non-SBFD symbols and SBFD symbols are specified clearly. On top of that, we further refined the table by considering the new requirements of in-channel adjacent subband leakage and in-channel adjacent subband blocking for both conducted and OTA clauses.

Proposal 5: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline:

Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| In-channel adjacent subband leakage | NA |  |  | TBD |  |  |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| In-channel adjacent subband blocking |  |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | TBD | TBD |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | TBD | TBD |

# 5. BS RF specification structure for SBFD-capable BS

In previous RAN4 discussion, RAN4 agree the compromised solution, i.e.,

<From RAN4#114> Agreement: - RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline, while details can be FFS by taking below structure as the example. Foreword 1 Scope 2 References 3 Definitions, symbols and abbreviations 4 General 5 Operating bands and channel arrangement 6 Conducted transmitter characteristics 7 Conducted receiver characteristics 8 Conducted performance requirements 9 Radiated transmitter characteristics 10 Radiated receiver characteristics 11 Radiated performance requirements 12 Requirements for SBFD-capable BS

Based on this, we provided the further detailed structure for the new chapter for SBFD-capable BS

Proposal 6: The further sub-clauses of Chapter 12 “requirements for SBFD-capable BS” are provided as below:

- Requirements for SBFD-capable BS:

12. 1     General

12.1.1		Applicability of requirements for SBFD-capable BS

12. 2     Operating bands and channel arrangement

12.2.1		General

12.2.2		Operating bands

12.2.3		BS channel bandwidth

12. 3     Conducted transmitter characteristics for SBFD-capable BS

12.3.1		General

12.3.2		Base station output power

12.3.3		Output power dynamics

12.3.4		Transmit ON/OFF power

12.3.5		Transmitted signal quality

12.3.6		Unwanted emissions

12.3.7		Transmitter intermodulation

12.3.8       In-channel adjacent subband leakage

12. 4     Conducted receiver characteristics for SBFD-capable BS

12.4.1		General

12.4.2		Reference sensitivity level

12.4.3		Dynamic range

12.4.4		In-band selectivity and blocking

12.4.5		Out-of-band blocking

12.4.6		Receiver spurious emissions

12.4.7		Receiver intermodulation

12.4.8		In-channel selectivity

12.4.9       In-channel adjacent subband blocking

12. 5     Conducted performance requirements for SBFD-capable BS

12. 6     Radiated transmitter characteristics for SBFD-capable BS

12.6.1		General

12.6.2		Radiated transmit power

12.6.3		OTA base station output power

12.6.4		OTA output power dynamics

12.6.5		OTA transmit ON/OFF power

12.6.6		OTA transmitted signal quality

12.6.7		OTA unwanted emissions

12.6.8		OTA transmitter intermodulation

12.6.9       OTA in-channel adjacent subband leakage

12. 7     Radiated receiver characteristics for SBFD-capable BS

12.7.1		General

12.7.2		OTA sensitivity

12.7.3		OTA reference sensitivity level

12.7.4		OTA dynamic range

12.7.5		OTA in-band selectivity and blocking

12.7.6		OTA out-of-band blocking

12.7.7		OTA receiver spurious emissions

12.7.8		OTA receiver intermodulation

12.7.9		OTA in-channel selectivity

12.7.10     OTA in-channel adjacent subband blocking

12. 8     Radiated performance requirements for SBFD-capable BS

# 6. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

RAN4 aspects for SBFD system parameters

Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- The specific UL subband sizes supported shall be the same for DUD and DU/UD if BS supports.

- This restriction shall be applied to FR1 WA BS only.

Proposal 2: For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case.

- This restriction shall be applied to FR1 WA BS only.

Proposal 3: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

Proposal 4: The RAN4 evaluation on the signal quality impact from the new waveform can be concluded, i.e., no new MRP is required for PUSCH with resource muting for CLI handling, if no critical issue further identified.

Proposal 5: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline:

Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| In-channel adjacent subband leakage | NA |  |  | TBD |  |  |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| In-channel adjacent subband blocking |  |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | TBD | TBD |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | TBD | TBD |

Proposal 6: The further sub-clauses of Chapter 12 “requirements for SBFD-capable BS” are provided as below:

- Requirements for SBFD-capable BS:

12. 1     General

12.1.1		Applicability of requirements for SBFD-capable BS

12. 2     Operating bands and channel arrangement

12.2.1		General

12.2.2		Operating bands

12.2.3		BS channel bandwidth

12. 3     Conducted transmitter characteristics for SBFD-capable BS

12.3.1		General

12.3.2		Base station output power

12.3.3		Output power dynamics

12.3.4		Transmit ON/OFF power

12.3.5		Transmitted signal quality

12.3.6		Unwanted emissions

12.3.7		Transmitter intermodulation

12.3.8       In-channel adjacent subband leakage

12. 4     Conducted receiver characteristics for SBFD-capable BS

12.4.1		General

12.4.2		Reference sensitivity level

12.4.3		Dynamic range

12.4.4		In-band selectivity and blocking

12.4.5		Out-of-band blocking

12.4.6		Receiver spurious emissions

12.4.7		Receiver intermodulation

12.4.8		In-channel selectivity

12.4.9       In-channel adjacent subband blocking

12. 5     Conducted performance requirements for SBFD-capable BS

12. 6     Radiated transmitter characteristics for SBFD-capable BS

12.6.1		General

12.6.2		Radiated transmit power

12.6.3		OTA base station output power

12.6.4		OTA output power dynamics

12.6.5		OTA transmit ON/OFF power

12.6.6		OTA transmitted signal quality

12.6.7		OTA unwanted emissions

12.6.8		OTA transmitter intermodulation

12.6.9       OTA in-channel adjacent subband leakage

12. 7     Radiated receiver characteristics for SBFD-capable BS

12.7.1		General

12.7.2		OTA sensitivity

12.7.3		OTA reference sensitivity level

12.7.4		OTA dynamic range

12.7.5		OTA in-band selectivity and blocking

12.7.6		OTA out-of-band blocking

12.7.7		OTA receiver spurious emissions

12.7.8		OTA receiver intermodulation

12.7.9		OTA in-channel selectivity

12.7.10     OTA in-channel adjacent subband blocking

12. 8     Radiated performance requirements for SBFD-capable BS

# 7. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[8] R4-2416497, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#112bis.

[9] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[10] R4-2418097, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#113.

[11] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[12] R4-2500006 (R1-2410889), “LS on impact on transmit signal quality/MPR requirement”, Huawei, RAN1#119.

[13] R4-2501967, “Further discussion on general aspects for SBFD system”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.