3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504219

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:	On the new requirements for SBFD operation for FR1 and FR2-1

Agenda item:			7.23.2.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111 to #114, further agreements are achieved on the potentially new requirements for SBFD operation [6] [8] [10] [12] [14]. In this contribution, accordingly, we would like to further provide our analysis and viewpoints on the potentially new BS RF requirements for SBFD operation for FR1 and FR2-1.

# 2. Transient period requirement for SBFD

### 2.1 Existing agreements

From Rel-18 study, the requirement of transmitter transient period will be introduced for the transmitter mode switching between SBFD and non-SBFD or SBFD reconfigurations if needed. RAN4 has confirmed that the transition period is needed between the non-SBFD slot and SBFD slot and vice versa. Furthermore, RAN4 has concluded that the transient period shall be introduced to BS in SBFD symbols.

For transmitter transient period between SBFD and non-SBFD or SBFD reconfigurations if needed, the requirement shall be introduced to BS in SBFD symbols/slots, by defining the transient period as the time period which the transmitter is changing from the SBFD operation to non-SBFD operation or vice versa, or during SBFD reconfigurations. Regarding the transition period requirement, RAN4 mainly focus on the transition period related with SBFD. Based on the RAN4 study, between the non-SBFD slot and SBFD slot and vice versa, a transition period is needed. If the SBFD configuration between adjacent SBFD slots is the same, then no transition period is needed.

The following agreements related to the transient period between SBFD and non-SBFD are achieved in RAN4#112 [8]:

#### <Agreement achieved in RAN4#112 [8]> #### Issue 2-1-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. #### Issue 2-1-2: Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. #### Issue 2-1-3: Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. #### Issue 2-2-1: Requirement for transient period between different SBFD configurations - Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations.

And the following way forward is achieved in RAN4#112bis [10] and RAN4#113 [12].

#### <Agreement achieved in RAN4#112bis [10]> #### Issue 2-1-1: Transient period requirement applicability on frequency/antenna Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. #### Issue 2-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol). <Agreement achieved in RAN4#113 [12]> #### Issue 3-1-1: Transient period requirement applicability on frequency/antenna Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases. #### Issue 3-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact

It should be noted that RAN1 agreements as below, for both one or two TDD-UL-DL pattern(s) configured, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. And the SBFD can be flexibly configured from/to any symbol of a certain slot.

<Agreements from RAN1#116> Agreement A slot can consist of SBFD symbols and non-SBFD symbols. Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details <Agreements from RAN1#117> Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. <Agreements from RAN1#118> Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

### 2.2 Requirement for transient period between SBFD and non-SBFD

Next issue to be discussed for transient period between SBFD and non-SBFD is how to capture the agreed transient period requirement into core requirement. Based on the agreement “RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD”, here we would like to identify the necessary changes to clause 6.4.2 and clause 9.5.3.

Based on last meeting discussion, the following agreement has been achieved:

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

Based on our understanding, the transient periods for SBFD UE and legacy TDD UE are illustrated in the below figure:

Figure 1: Detailed illustration for transient periods for SBFD UE and legacy TDD UE for four cases

Based on the agreement from RAN4#114, the different views on Case-B and Case-C.

#### 2.1.1 Location of transient period for Case-B (from SBFD symbol to UL symbol)

For Case-B (from SBFD symbol to non-SBFD UL symbol), for the TDD UE timeline (in the bottom of the above figure), legacy UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset configured by the network (either 25600Tc or 39936Tc). Obviously, the transient period located in guard symbol(s) reserved in SBFD symbol(s) is needed for DL subband(s), while not needed for UL subband, becuase TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all.

Proposal 1: For Case-B, the location of transient period is confirmed to be in guard symbol(s) reserved before UL non-SBFD symbols, and the guard symbol(s) should be reserved in SBFD symbol(s).

#### 2.1.2 Location of transient period for Case-C (from DL symbol to SBFD symbol)

For Case-C (from non-SBFD DL symbol to SBFD symbol), for DL subband(s), no transient period is required because the BS transmission is kept from DL symbol to SBFD symbol. For UL subband, two ways of reserving the transient period have been identified: (1) guard symbols reserved in non-SBFD DL symbol(s) and (2) blanked SBFD symbols in the beginning of SBFD UL transmission. From our understanding, both ways work. Particularly for (1) guard symbols reserved in non-SBFD DL symbol(s), we also think it is also one possible way if NW would configure the TDD time domain configuraiton as this. The overhead on the D/F-slot is reasonable, since all the SBFD configuraiton is provided by NW and the NW can ovoid this if the overehead on the legacy D/F-slot is concerned.

Proposal 2: For the location of transient periods in Case-C, both (1) guard symbols reserved in non-SBFD DL symbol(s) and (2) blanked SBFD symbols in the beginning of SBFD UL transmission shall be allowed, depending on NW configuration.

#### 2.1.3 RAN4 requirement impact

For the RAN4 specification impact from the location of transient period, we found there is no need specify the detailed location of transmitter transient period. Specifically, from RAN4 conformance requirement perspective, the test on the “OFF” power on the OFF period is performed (clause 6.4.2.4 in TS 38.141-1):

3)	Measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the antenna connector or TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

In other words, based on gNB configured TA_offset and gurard symbols, the RAN4 conformance requirement (“OFF” power test) can be performed on the OFF period configured by gNB, which also requires no necessity of specifying the detailed location of transmitter transient period

Proposal 3: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

### 2.3 LS to RAN1 on transient period

After confirming the remaining issues for transient period between SBFD and non-SBFD symbol, it is proposed to send LS to RAN1 to give the information about RAN4 agreement.

Proposal 4: After clarifying the remaining issues for transient period, RAN4 shall send LS to RAN1 to give the information about RAN4 agreements, and the drafted LS is given in the Appendix.

# 3. Discussion on in-channel adjacent subband requirements

### 3.1 In-channel adjacent subband leakage ratio

In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, and in RAN4#114, the following agreement is achieved:

Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

The new requirement is considered to be needed to guarantee the TX leakage between the sub-bands. It should be noted that the impact of self-interference has been guaranteed by OTA sensitivity requirement already, the in-channel adjacent subband leakage ratio requirement is introduced to guarantee the inter-site gNB-to-gNB CLI, especially if no coordination between gNBs. Based on Rel-18 study, for inter-site gNB-gNB co-channel inter-subband CLI modelling, RAN4 suggest the TX leakage can be modelled as gNB ACLR. Based on last meeting discussion, the existing ACLR requirement in absolute basic limit is considered as baseline.

Proposal 5: Define the requirement as “in-channel adjacent subband leakage requirement” by removing “ratio”.

Specially, for TX leakiage, our FR1 3.5 GHz testbed implementation shows that the use of digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands. By taking 80MHz DL subband as example, by considering 50dBm as BS TX power, the power density is around -14.03dBm. However, for 20MHz DL subband, still taking 50dBm as BS TX power, the power density can be as high as -8dBm. From that perspective, we think additional 5dB relation is reasonable to be take into account for in-channel adjacent subband leakage requirement.

Proposal 6: Take the below absolute basic limit values for in-channel adjacent subband leakage requirement:

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -8 dBm/MHz |
| Category B Wide Area BS | -10 dBm/MHz |
| Medium Range BS | -20 dBm/MHz |
| Local Area BS | -27 dBm/MHz |

### 3.2 In-channel adjacent subband blocking

In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity, and no further agreement achieved in RAN4#114:

Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

Based on our evaluation for FR1 UMa-Uma, even without any CLI handling mechanism, co-channel interference from neighboring BS is not significant as adjacent channel interference, for which the existing ACS requirement can be used to mitigate the co-channel gNB-to-gNB interference for more than 95% of the cases. For the remaining high interference cases, RAN1 has introduced gNB-gNB CLI-handling mechanism, by which the co-channel interference from neighboring gNB DL subband transmission could be greatly mitigated.

| Scenario 1: FR1 UMa-UMa |  |
| --- | --- |

Proposal 7: For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement.

By taking into account the analysis in the study item phase, the RF simulation has shown the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI is in the range of 62-93dB, depending on different antenna pair and co/cross-polarization, and 75dB can be regarded as typical value as RAN4 agreement. By taking 50dBm as the assumption for TX BS power, we shall specify -35 dBm as the requirement level for the SBFD in-channel adjacent subband blocking.

Proposal 8: For FR1 WA BS, -35 dBm is adopted as the requirement level for the SBFD in-channel adjacent subband blocking.

# 4. Conclusion

In this contribution, we provided BS RF requirement for SBFD-capable BS, accordingly with the following observations and proposals obtained:

Transient period requirement for SBFD

Proposal 1: For the location of transient periods Case-B, it is in guard symbol(s) reserved before UL non-SBFD symbols, and the guard symbol(s) in this case should be reserved in SBFD symbol(s).

Proposal 2: For the location of transient periods Case-C, both (1) guard symbols reserved in non-SBFD DL symbol(s) and (2) blanked SBFD symbols in the beginning of SBFD UL transmission shall be allowed, depending on NW configuration.

Proposal 3: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

Proposal 4: After clarifying the remaining issues for transient period, RAN4 shall send LS to RAN1 to give the information about RAN4 agreements, and the drafted LS is given in the Appendix.

In-channel adjacent subband requirements

Proposal 5: Define the requirement as “in-channel adjacent subband leakage requirement” by removing “ratio”.

Proposal 6: Take the below absolute basic limit values for in-channel adjacent subband leakage requirement:

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -8 dBm/MHz |
| Category B Wide Area BS | -10 dBm/MHz |
| Medium Range BS | -20 dBm/MHz |
| Local Area BS | -27 dBm/MHz |

Proposal 7: For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement.

Proposal 8: For FR1 WA BS, -35 dBm is adopted as the requirement level for the SBFD in-channel adjacent subband blocking.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis

[5] R4-2409765, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411640, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112.

[8] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[9] R4-2416498, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112bis.

[10] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[11] R4-2418098, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#113.

[12] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[13] R4-2501968, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.

# 6. Appendix – draft LS to RAN1 on SBFD transient period

3GPP TSG-RAN WG4 Meeting #114-bis		R4-25xxxxx

Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	LS on the transient period for SBFD operation

Response to:	N/A

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Wang, He (Jackson)

h0809.wang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

## 1	Overall description

On RAN4 requirements for SBFD transient period at gNB, RAN4 has made the following agreements

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases: Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON.

Particularly for the location of transient period, RAN4 has identified four cases of transitions to/from SBFD symbols as illustrated in Figure 1:

Figure 1: Illustration of four cases of transitions to/from SBFD symbols

And the following agreement is achieved on the location of transient period for the above four cases:

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s).

NOTE: further update the above highlighted content after further agreement is achieved in RAN4#114bis.

RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any.

## 2	Actions

To RAN1

ACTION: RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any.

## 3	Dates of next TSG RAN WG1 meetings

RAN4#115		19th – 23rd May 2025	 	Malta, MT

RAN4#116 		25th – 29th Aug. 2025	 	Bangalore, India