# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504222

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.26.4.2

Source: 	Ericsson

Title:	Discussion on LP-WUR RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1]. In this contribution, we continue the discussion on the RRM impact of LP-WUR operation.

- General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on LP-WUS WID, different scenarios to support the RRM relaxation shall be defined as below:

- MR further relaxation with LR

- MR offloading by LR

In last RAN4 meetings, RAN4 agreed the case #1, #3, but the other MR relaxation case is FFS.

RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

When both NW and UE supports LP-WUS feature, different scenarios are valid by the NW using different thresholds and the UE will need to adapt its measurement behaviour to the respective zones. Adapting herein comprises turning ON/OFF the MR or LR receiver chains.

In legacy release, when neighbour cell measurement relaxation criteria were introduced, UE may perform neighbour cell measurements from normal measurement, relaxed measurement till stop the measurement based on the distance to the cell center. Now in Rel-19 LP-WUS, different serving cell measurement relaxation scenarios will be introduced. As shown in the figure below, different thresholds for neighbour cell measurement and serving cell measurement can be introduced for different serving cell and neighbour cell combinations.

Figure 1 Serving and neighbour cell measurement scenarios

There are still FFS for some scenarios. When UE enters LP-WUS mode, UE will use LR to monitor the serving cell quality without MR to achieve the power saving gain. Due to UE does not to combine the measurement results across MR and LR, it seems scenario 2 is not useful.

RAN4 #112 Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Issue 1-1-2: Threshold for switch between different cases

In last meeting, we noticed there is an open issue to discuss how to define a threshold for switching between scenarios.

Issue 1-1-2: Threshold for switch between different cases - Proposals - P1: Up to RAN2 decision (CMCC CT) - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (Apple ZTE vivo) - P2-1: LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple ZTE) - P2-2: Details on threshold are up to RAN2 (vivo) - P3: Discuss first what has been agreed in RAN2 (Nokia) - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK)

In our understanding, whether and how to introduce the new threshold(s) for switching between different cases should be a RAN2 work. From our information, RAN2 is also discussing the thresholds. Therefore, RAN4 should leave such discussion RAN2.

Proposal 2: Whether and how to introduce new thresholds for switching between different scenarios should be up to RAN2 decision.

Issue 1-1-10: Considerations on higher priority frequency layer

Another remaing issue related to scenario is about higher priority relaxation. In our understanding, legacy RRM relaxation has already relaxed the higher priority measurement. We don’t think any further higher priority relaxation criteria shall be introduced.

Issue 1-1-11: Considerations on higher priority frequency layer - Proposals - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

- Detail LP-WUR requirements

Issue 1-2-1: Accuracy requirements

In RAN4 #111 meeting, RAN4 spent a long time to discuss how to define the accuracy requirement. Two possible way forwards were proposed.

- Define a new LP-SS accuracy directly in performance section

- Follow the legacy IDLE mode requirements to reflect accuracy in reselection margin

Issue 1-2-1: Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG CMCC CT ZTE Apple Ericsson Huawei MTK) - P2: If “using a margin to reflect accuracy performance of LP-WUR in core specs”, is used for LP-WUR, for MR, the following options could be considered (vivo) - Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements. - Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs. - P3: Conclude final accuracy requirement details after simulations have been collected and compared (Nokia)

We prefer the second option to reuse the same approach where the legacy SSB-based measurements in IDLE mode are used indirectly as reselection margin other than creating a new LP-SS performance part for IDLE/INACTIVE mode. The NR SSB based performance requirements were also defined because the SSB performance can be verified in CONNECTED mode, but LP-SS is only used in IDLE mode which cannot be verified due to no measurement reporting.

Proposal 4: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

In last meeting, RAN4 spent a long time to discuss the periodicity of SSB based LP-WUR and made the following agreements. The remaining issue is how to handle the case LO periodicity is smaller than LP-SS periodicity. In our understanding, it’s not reasonable to configure LO periodicity smaller than LP-SS periodicity. Therefore, regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

#### Issue 1-2-2: Periodicity for LP-WUR RRM requirements - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. (LG Apple CMCC oppo) - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. (CATT) - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. (Huawei) - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity.(Ericsson)

Proposal 5: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Some companies also propose to discuss the measurement filtering.

Issue 1-2-4-1: On LR measurement filtering requirement - Proposals - P1: The UE shall filter the LP-RSRP and LP-RSRQ and [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or [LO periodicity /2] for SSB based LR. (Huawei Ericsson vivo)

In legacy, the DRX case, the filtering interval is at least DRX/2. We think it’s reasonable to define the measurement filtering based on the periodicity of LP-SS. In LP-WUR, the typical LP-SS peridocity is 320ms. To achieve the comparable performance, we propose to also use 320/2ms as measurement interval.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

Proposal 6: RAN4 to define the minimum measurement interval in LP-WUR as LO periodicity/2 for both LP-SS and SSB.

Issue 1-2-5-2: On MR evaluation requirements

In last meeting, RAN4 agreed to define a single LR evaluation requirement for both entry and exit threshold evaluation.

Issue 1-2-4-2-2: On single or multiple LR evaluation requirements Agreement: Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement.

Thus, RAN4 needs to define the LP-WUR based serving cell evaluation requirement. In legacy requirement, Nserv is defined in the table below.

Table: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |
| 0.32 | 1 | 8 | 12 | M1*N1*4 |
| 0.64 |  | 5 | 8 | M1*N1*4 |
| 1.28 |  | 4 | 6 | N1*2 |
| 2.56 |  | 3 | 5 | N1*2 |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. |  |  |  |  |

In our understanding, currently, the LR based serving cell evaluation performance shall be better. Thus, we prefer to define the Nserv as 1.

Proposal 7: RAN4 to define Nserv as 1 in LP-WUR based exit/entry evaluation requirement.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

As we mentioned before, it cannot guarantee the quality is always good enough when UE is in the LR mode even if NW configures a very tight condition to stay in purely LR mode case 1. UE may directly trigger the cell selection procedure in LR mode. Thus, RAN4 should define the LP-WUR based cell selection evaluation requirement.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1 - Proposals - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple CMCC vivo Huawei MTK) - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson) - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

Proposal 8: RAN4 to define LP-WUR based serving cell evaluation requirement in case 1.

- RRC-CONNECTED mode

RAN4 shall focus on developing the RRM requirements for IDLE and INACTIVE mode during this meeting since not many agreements have been reached in other WGs for CONNECTED mode. Discussions on CONNECTED mode shall therefore be postponed until any agreements are achived in other WGs regarding, and if so RAN4 can check the RRM requirements for those at later stage.

Proposal 9: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

- Others

EMR

In last meeting, some companies mentioned to discuss UE’s behaviour in EMR. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running. However, RAN4 never discussed the UE’s behaviour when NW configures EMR in MR relaxation and offloading scenarios. Thus, we support RAN4 to discuss UE’s behaviout of MR relaxation with EMR.

Issue 1-5-1: LR based RRM with EMR in IDLE/Inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple) - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo) - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC) - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements. (Nokia) - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode) - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Proposal 10: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

RAN1 LS

In last meeting, RAN1 sent a LS to RAN4 to ask the number of SSBs for wake-up.

In our understanding, when UE wakes up from deep sleep mode, UE can obtain the timing information from LR. In legacy paging interruption requirement, 2 SSBs are enough to perform the fine sync and AGC retuning. Therefore, we prefer to use 2 SSBs instead of 3SSBs. When UE wakes up from ultra-deep sleep mode, we understand UE may not have any prior information. Thus, total 5 SSBs look fine to finish the coarse sync, AGC and fine sync.

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Proposal 11: RAN4 to suggest RAN1 to update the wake-up delay capability to {[50ms], [500ms] and [900ms]}, where [50ms] assumes [2] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

RedCap UE capability

In Rel-19 LP-WUS WID, it clearly explained the use case for LP-WUS as below. Energy efficiency is even more critical for UEs without a continuous energy source. Among vertical use cases, sensors and actuators are deployed extensively. Wearables include smart watches, rings, eHealth related devices, and medical monitoring devices. With typical battery capacity, it is challenging to sustain up to 1-2 weeks as required. In our understanding, RedCap UE is a typical use case for LP-WUS. Therefore, when RAN4 defines the LP-WUS related requirement, RAN4 shall consider both normal UE and RedCap UE. RedCap UE shall also meet LP-WUS related requirement.

Proposal 12: RedCap UE shall meet LP-WUS related requirement based on UE’s capability.

- Summary

In this contribution, we have discussed the LP-WUS operation. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Proposal 2: Whether and how to introduce new thresholds for switching between different scnarios should be up to RAN2 decision.

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

Proposal 4: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Proposal 5: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Proposal 6: RAN4 to define the minimum measurement interval in LP-WUR as LO periodicity/2 for both LP-SS and SSB.

Proposal 7: RAN4 to define Nserv as 1 in LP-WUR based exit/entry evaluation requirement.

Proposal 8: RAN4 to define LP-WUR based serving cell evaluation requirement in case 1.

Proposal 9: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

Proposal 10: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

Proposal 11: RAN4 to suggest RAN1 to update the wake-up delay capability to {[50ms], [500ms] and [900ms]}, where [50ms] assumes [2] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

Proposal 12: RedCap UE shall meet LP-WUS related requirement based on UE’s capability.

- References

[1]. R4-2502600, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. R1-2501624, “LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode”, Apple