# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504225

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.1

Source: 	Ericsson

Title:	Discussion on OD-SSB general part in Rel-19 NES

Document for:	Discussion

- Work scope

Type of OD-SSB based SCell activation

In last meeting, there is an open issue about which type of OD-SSB based SCell activation will be defined.

Issue 1-5-1: The type of OD-SSB based SCell activation FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios. - Option 1: Direct SCell activation at SCell addition - Option 2: Multiple SCells activation - Option 3: Deactivated PUCCH SCell activation - Option 4: Rel-17 Fast SCell Activation - Option 5: Rel-18 SCell Activation with the L3 reporting during activation

In RAN1 #118 meeting, RAN1 made the agreement to support OD-SSB based direct SCell activation. Thus, we propose RAN4 to define OD-SSB based direct SCell activation requirement.

RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

In RAN2 #127 meeting, RAN2 made the agreement to support OD-SSB based multiple SCells activation. At the same time, we understand a typical use case about OD-SSB transmission is multiple SCells activation in FR2. NW can indicate OD-SSB transmission only in one cell. After UE finishing the deactivated SCell measurement, NW can trigger multiple OD-SSB SCells activation based on known cell condition. Thus, we propose RAN4 to define OD-SSB based multiple SCells activation requirement at least for the scenarios all the SCells being activated are OD-SSB SCells.

RAN2 #127bis Agreement NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

However, in RAN4 #112 meeting, RAN4 had the agreement about the scope and discussion priority as follow.

Issue 1-1-1: OD-SSB work plan Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed.

Thus, we proposed to define OD-SSB based direct SCell activation at SCell addition once RAN4 concludes the MAC-CE based single SCell activation. The detail requirement can be discussed in the CR directly. In our understanding, fast SCell activation is mainly based on A-TRS and TRS by aperiodic CSI-RS other than SSB. Thus, RAN4 doesn’t need to define OD-SSB based fast SCell activation, but it’s fine to update the known condition based on OD-SSB.

Proposal 1: RAN4 to define the following OD-SSB based SCell activation requirement once RAN4 concludes the MAC-CE based single SCell activation. The details can be discussed in the CR directly.

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation enahancement with the L3 reporting during activation

Proposal 2: RAN4 not to define the OD-SSB based Rel-17 fast SCell activation requirement, but only update the known cell condition based on OD-SSB.

Considering the workload, we suggest to limit the scope with all SCells being activated are OD-SSBs’ SCells and doesn’t impact the WI completion.

Proposal 3: RAN4 to define the OD-SSB based multiple SCells activation requirements.

- All SCells being activated are OD-SSBs’ SCells

- It doesn’t impact the WI’s completion.

- CR work split

RAN4 needs to start the CR work split in April’s meeting based on the work plan. One of the important issues about the spec. is whether we need to split the OD-SSB related procedures as a new section or keep them in the legacy section.

Proposal 4: RAN4 to discuss whether to split the sections of OD-SSB SCell activation requirement with legacy SSB SCell activation requirement.

The following sections have potential spec. impact.

Proposal 5: The OD-SSB related initial CR work split can be as follow.

| Section | Title | Note | Company |
| --- | --- | --- | --- |
| (New) | Interruptions for OD-SSB transmission | TBD RAN4 to check whether needed |  |
| 8.2.2.2.1 | Interruptions at SCell addition/release | TBD |  |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | TBD |  |
| 8.3.2 | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell |  |  |
| 8.3.3 | OD-SSB based SCell Deactivation Delay Requirement for Activated SCell | TBD Wait RAN1’s progress in OD-SSB timer |  |
| 8.3.4 | OD-SSB based direct SCell Activation at SCell addition |  |  |
| 8.3.7 | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells | TBD Check the scope |  |
| 8.3.8 | OD-SSB based SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells | TBD |  |
| 8.3.12 | SCell Activation Delay Requirement for Deactivated PUCCH SCell | TBD |  |
| 8.3.13 | SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells | TBD |  |
| 8.3.16 | Fast SCell Activation Delay Requirement for Deactivated SCell | TBD |  |
| 8.3.17 | SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation | TBD |  |
| 8.3.18 | SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting | TBD |  |
| 9.2.1 | Introduction |  |  |
| 9.2.5.1, 9.2.5.2 | Intrafrequency cell identification, Measurement period |  |  |
| 9.2.5.3 | Scheduling availability of UE during intra-frequency measurements |  |  |
| 9.5.1, 9.5.4 | L1-RSRP measurements for Reporting - Introduction - L1-RSRP measurement requirements |  |  |
| 9.5.5, 9.5.6 | Measurement restriction for CSI-RS and SSB for L1-RSRP measurement, Scheduling availability of UE during L1-RSRP measurement |  |  |
| 9.8.1, 9.8.4 | L1-SINR measurements for Reporting - Introduction - L1-SINR measurement requirements |  |  |
| 9.8.5, 9.8.6 | Measurement restriction for L1-SINR measurement, Scheduling availability of UE during L1-SINR measurement |  |  |

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals:

Proposal 1: RAN4 to define the following OD-SSB based SCell activation requirement once RAN4 concludes the MAC-CE based single SCell activation. The details can be discussed in the CR directly.

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation enahancement with the L3 reporting during activation

Proposal 2: RAN4 not to define the OD-SSB based Rel-17 fast SCell activation requirement, but only update the known cell condition based on OD-SSB.

Proposal 3: RAN4 to define the OD-SSB based multiple SCells activation requirements.

- All SCells being activated are OD-SSBs’ SCells

- It doesn’t impact the WI’s completion.

Proposal 4: RAN4 to discuss whether to split the sections of OD-SSB SCell activation requirement with legacy SSB SCell activation requirement.

Proposal 5: The OD-SSB related initial CR work split can be as follow.

| Section | Title | Note | Company |
| --- | --- | --- | --- |
| (New) | Interruptions for OD-SSB transmission | TBD RAN4 to check whether needed |  |
| 8.2.2.2.1 | Interruptions at SCell addition/release | TBD |  |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | TBD |  |
| 8.3.2 or New | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell |  |  |
| 8.3.3 or New | OD-SSB based SCell Deactivation Delay Requirement for Activated SCell | TBD Wait RAN1’s progress in OD-SSB timer |  |
| 8.3.4 or New | OD-SSB based direct SCell Activation at SCell addition |  |  |
| 8.3.7 or New | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells | TBD Check the scope |  |
| 8.3.8 or New | OD-SSB based SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells | TBD |  |
| 8.3.12 or New | SCell Activation Delay Requirement for Deactivated PUCCH SCell | TBD |  |
| 8.3.13 or New | SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells | TBD |  |
| 8.3.16 or New | Fast SCell Activation Delay Requirement for Deactivated SCell | TBD |  |
| 8.3.17 or New | SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation | TBD |  |
| 8.3.18 or New | SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting | TBD |  |
| 9.2.1 | Introduction |  |  |
| 9.2.5.1, 9.2.5.2 | Intrafrequency cell identification, Measurement period |  |  |
| 9.2.5.3 | Scheduling availability of UE during intra-frequency measurements |  |  |
| 9.5.1, 9.5.4 | L1-RSRP measurements for Reporting - Introduction - L1-RSRP measurement requirements |  |  |
| 9.5.5, 9.5.6 | Measurement restriction for CSI-RS and SSB for L1-RSRP measurement, Scheduling availability of UE during L1-RSRP measurement |  |  |
| 9.8.1, 9.8.4 | L1-SINR measurements for Reporting - Introduction - L1-SINR measurement requirements |  |  |
| 9.8.5, 9.8.6 | Measurement restriction for L1-SINR measurement, Scheduling availability of UE during L1-SINR measurement |  |  |

- References

- R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson