# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504228

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.2.3

Source: 	Ericsson

Title:	Discussion on OD-SIB1 in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, we achieved an agreement to discuss the potential RRM impact for OD-SIB1 as follow[1].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

In this contribution, we discuss on the Rel-19 NES operation in OD-SIB1 from RAN4 perspective.

# General procedure for OD-SIB1

The valid scenario of OD-SIB1 deployment was discussed in RAN1 and RAN2 with significant progress. We captured some RAN2 agreements as follow.

RAN2#125bis agreement - At least RAN2 starts scenario 1a. Other scenarios are not excluded. - Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts: - Add WUS configuration in SIB of cell A. - Cell reselection from cell A to NES cell, including trigger condition and cell barring changes. - Whether allow camping, paging and SIB update in NES cell. - Cell reselection from NES cell to cell A or normal cell. - On-demand SIB1 acquisition procedure - Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. RAN2#126 agreement - Triggering condition of WUS transmission - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell. RAN2#127 agreement - RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. RAN2#127bis agreement - NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration). RAN2#128 agreement - Triggering condition for OD-SIB1 - Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

RAN1 and RAN2 have already agreed the following terminology to discuss the OD-SIB1 scenario.

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE.

- Cell A: A cell that is periodically transmitting at least its own SIB1.

A typical OD-SIB1 acqusition procedure is as follow.

- Step 1: The UE evaluates cell reselection based on measurements towards SSB of candidate NES cell(s) and UL WUS config (including cell access restriction info to check cell suitability).

- Step 2: After cell reselection criteria towards one NES cell is met, the UE triggers reselection to NES cell and initiates OD-SIB1 procedure during reselection.

  - Step 2-1: UE transmits the UL-WUS

  - Step 2-2: UE receives the RAR

  - Step 2-3: UE monitors OD-SIB1 window

Figure 1. OD-SIB1 valid sceanrio

In last meeting, some companies mentioned UE transmits UL-WUS may result in paging interruption. RAN4 needs to consider the paging interruption requirement for OD-SIB1. Firstly, NW can easily configure to avoid the collision between paging occasion and RACH occasion. Furthermore, paging transmission in Cell A is very sparse(DRX cycle) and important in IDLE mode. However, the RACH transmission occasion for OD-SIB1 is much dense and less important than such paging cycle. In other words, UE can choose the next RACH occasion to request the OD-SIB1 even one of the RACH occasions may collide with paging occasion. Thus, we don’t expect additional paging interruption requirement is needed.

Observation 1: Paging monitoring in cell A is more important than the UL-WUS transmission to acquire the OD-SIB1.

Proposal 1: No RAN4 spec. impact is identified.

- UE shall prioritize the paging reception and delay the UL WUS procedure if collision happens.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we made the following proposals and observations:

Observation 1: Paging monitoring in cell A is more important than the UL-WUS transmission to acquire the OD-SIB1.

Proposal 1: No RAN4 spec. impact is identified.

- UE shall prioritize the paging reception and delay the UL WUS procedure if collision happens.

- References

- R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei