3GPP TSG-RAN WG4 Meeting #114bis	R4-2504229

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 1

Agenda item:	7.19.1

Document for:	Discussion

Introduction

In RAN4#107, WI ‎[1] “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was agreed. In this WI, following objective was agreed for RAN4:

- For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring. Note: the following aspects may be considered: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In this paper, we share our views on the following topics, mainly for UE sided model cases:

- Post deployment testing

- Life-Cycle Management related aspects

- On Generalization, number of tests and non-stationary conditions

Discussion

Post deployment testing

The moderator suggested the following way forward in RAN4#114 for the post deployment testing. We suggested some updated in the Table below as well as our views on different issues addressed in the WF. The detailed view on some of the issues is provided in the text following the table. In the table, the updates are highlighted in green.

The following table presents the description of options 1 and 2 and sub-options as already agreed. A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future. The descriptions of the options are the same as the already agreed description. The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings. The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision. Option 1; Option 2. New name; Post-deployment pre-activation functionality update testing Check Proposal 1 & 2. Post-deployment = the UE is operating in the field, connected to a mobile network. Pre-activation = the functionality/configuration is available in (supported by) the UE device, and it is not RRC/MAC activated. Check Proposal 3. The functionality/configuration requires testing before it can be RRC/MAC activated by the NW. RRC/MAC Activation – used in the field - functionality is already RRC configured in the UE - Functionality is just available in the UE? - Functionality is configured only for test but not for active use Test-active = the ML-enabled functionality will provide ground-truth along with the inference output to the NW as a part of the post deployment testing. If not Option 1 is not feasible.; Post-deployment post-activation functionality testing based on performance monitoring Check Proposal 1 & 4. Post-deployment = the UE is operating in the field, connected to a mobile network. Post-activation = the functionality/ configuration is available in (supported by the) UE, it has been RRC configured by the NW and, has been RRC/MAC activated by the NW. Test-active = the functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing. Check Proposal 5.. Description; Conduct the validation of a change in AI functionality that is already deployed in the UE before its RRC/MAC activation to be used for any inference.. • Validation takes into account the UE capabilities in which the updated functionality is to be activated. Check Proposal 6 & 7.; The UE is configured and provides monitoring reports. The NW takes ML management (“LCM”) actions (RRC re-configuration) based on monitoring reports and the in/applicability indications received from the UE. Check Proposal 8. - Performance monitoring will be designed in other groups (RAN1-RAN2) - Functionality in/applicability configuration procedure is designed in other groups (RAN2) The post-deployment testing can be triggered by the NW when the UE indicates the applicability of a functionality/configurations which was previously indicated as in-applicable to the NW. (RAN2 procedures) Check Proposal 8. - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the active and applicable functionality, sent from the UE, and set requirements as feasible/needed. - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the test-active functionality, sent from the UE, and set requirements as feasible/needed. Check Proposal 8.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.; Possible re-configuration of the performance monitoring to enable monitoring of the test-active functionality/configuration.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. Neither. Check Proposal 11. o Whether models that have been updated post conformance testing should be kept in a device. No. Check Proposal 12. o Whether parallel operating of one model and (in-device) testing of another can be assumed. Yes. Check Proposal 13-14. o Whether signaling from a UE of changes/updates are needed. No for Option 1. Yes for Option 2 only. Check Proposal 15. - Whether there is any relation to GCF procedures. Possibly. Check Proposal 16. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. Not in 3GPP scope. Check Proposal 16. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. Not in Rel 19 scope. Check Proposal 17. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy. Yes. Check Proposal 18.; The following proposals have been made for option 2 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only. Note: Even in case of the Option 2, parallel operation of ML-enabled functionalities is feasible. Explained below. Check Proposal 13-15.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) No. Check Proposal 9. Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. Yes. Check Proposal 10..

Details of Name

In this section we provide our views and proposals related to the new name proposed for Post-Deployment (Row 1) by the Moderator regarding the Option 1 and Option 2 definitions.

Based on the current RAN1-RAN2 agreements and earlier RAN4 agreements on the scope of AI/ML testing requirements, we use the working assumption that in Release 19 there will be no signalling mechanisms introduced which would allow the explicit testing of ML models or ML model updates/changes in RAN4-5. This is also valid for any proposed post-deployment testing solutions which would require access to the inputs/outputs of the ML models in the UE or gNB. In Release 19 RAN4 can only address testing of the different configurations for a ML-enabled feature (aka functionalities).

While we agree that the primary reason to introduce post-deployment testing is to address ML model updates/changes, in Release 19 we support addressing the post-deployment testing requirements only at the level of ML-enabled feature configuration (or functionalities). The post-deployment testing can address the testing of implementation changes/updates of functionalities without the explicit testing of the underlying changes/updates of the ML models.

We further acknowledge as first step the need for clear identification of these two post-deployment Options to be able to select one of them in RAN4 and reach a constructive agreement. Part of this exercise is also to reach consensus on the terminology used while maintaining alignment with the currently used terminology and agreements from RAN1 and RAN2.

The first common aspect, and somehow obvious, for both the Options is the primary reason for post-deployment testing: any functionality/configuration implementation updates on the UE-side which cannot be identified nor handled with the help of the UE capability procedures.

The second common aspect of both the Options is the ‘post-deployment testing’. Therefore, we propose to define and capture the definition clearly in the context of the RAN4 discussions. Future discussions with other RAN WG will greatly benefit from this clarity.

The RAN4 discussions to use “post-deployment testing” defined as “the testing of functionality/configuration in a UE which is operating in the field, connected to a mobile network”.

Thirdly, in our understanding, it should be left to gNB/NW implementation what available information to use to decide on the need for triggering the post-deployment testing of a UE functionality/configuration. The RAN4 discussion shall focus on the UE behaviour and requirements when such post-deployment is triggered by the gNB/NW.

In the next subsections we provide our views and proposal on the descriptive naming of the Option 1 and Option 2.

      - Post-deployment - Option 1

For the option 1 we are proposing the descriptive naming “Post-deployment pre-activation functionality/configuration update testing”.

The RAN4 discussions to assume Option 1 described as “Post-deployment pre-activation functionality/configuration update testing”.

The reasoning for this proposal is that in the Option 1 the proposed functionality/configuration testing is to be carried out before it is explicitly activated by the gNB/NW for inference (RAN2). Hence here ‘pre-activation’ indicates that the functionality/configuration is available in (supported by) the UE device, based on UE capabilities, and it is not RRC/MAC activated for use in the UE.

The RAN4 discussions on the functionality/configuration which requires testing before it can be RRC/MAC activated by the NW for inference, to use “pre-activation” term to indicate “the functionality/configuration is available in (and supported by) the UE device, and it is not RRC/MAC activated”.

The details, and high-level steps, for Option 1 are explained further in Section 2.1.2.

      - Post-deployment - Option 2

For the option 2 we are proposing the descriptive naming “Post-deployment post-activation functionality update testing based on performance monitoring”.

The RAN4 discussions to assume Option 2 described as “Post-deployment post-activation functionality testing based on performance monitoring”

In our understanding, there are two key differences between Option 2 and Option 1. In Option 2 the performance monitoring mechanisms/procedures discussed in RAN1-2 are to be used. We believe this needs to be reflected in the label and description of the Option 2. Secondly, in Option 2, the use of the performance monitoring mechanisms/procedures implies that the functionality/configuration has been RRC/MAC activated by the gNB/NW and, the UE can provide the corresponding performance monitoring metrics to the gNB/NW.

For Option 2 we therefore of the opinion that two new clarifications are needed. Firstly, the obvious term of “post-activation” can be used for any functionality/configuration which is available in (supported by the) UE, it has been RRC configured by the gNB/NW and, has been RRC/MAC activated by the gNB/NW.

Secondly, we need to differentiate between the functionality/configuration which is being tested and any other functionality/configuration for the same UE feature (sub use-case) which has been configured for normal operation and reporting (RAN1-2). For this we propose to introduce the term “test-active” defined as “the functionality/configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

The RAN4 discussions to use “test-active” defined as “the functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

The details, and high-level steps, for Option 2 are explained further in Section 2.1.2.

Details of Description

      - Post-deployment - Option 1

In case of post-deployment testing using option 1, the main steps are as below. The first 3 steps are in accordance with currently discussed RAN2 procedures, Step 4 and Step 7 are common for the post-deployment Option 1 and Option 2, while Steps 5-6 are the post-deployment Option 1 testing related (high-level) steps:

- The UE is assumed to have provided its relevant (ML-enabled feature) capabilities to the gNB/NW.

- The gNB/NW has configured the UE for the ML-enabled feature, including in/applicability indications and the monitoring reports

- Based on the received in/applicability indications and/or monitoring reports the gNB/NW take ML management (“LCM”) actions (e.g. RRC re-configurations)

- The gNB/NW can enable the functionality for post-deployment testing. This is based on the applicability conditions (applicability report, RAN2) of the ML-enabled feature which the gNB/NW can use to decide on the tests that are required to test the feature.

Note: It is up to NW implementation what available information to use to decide on the need for triggering the post-deployment testing of the functionality/configurations declared as applicable by the UE.

- If the gNB/NW decides that a post-deployment test is required for the ML-enabled feature, then it will configure a test-active mode to run the post-deployment test for the feature.

        - The test-active configuration in Option 1 consists of configuration of RRC reports including ground-truth information for the feature being tested

- The test-active reports results are collected.

- Based on the test results:

    - If the KPIs are met, then the tested ML-enabled feature configuration (functionality) can be RRC/MAC activated to be used for inference

    - If the KPIs are not met, then the tested ML-enabled feature configuration (functionality) will not be activated, and another configuration/functionality or the fallback mechanism can be activated instead.

To realize the steps above a robust framework is necessary to enable the UE to be configured for test-active mode (Step 4-5) and then the post-deployment tests can be run (Step 6-7).

RAN4 should discuss on the framework related to the test-active mode of the ML-enabled feature to realize the post-deployment testing in Option 1.

Since the ML-enabled feature is already available in the UE and needs to be tested in the field, there can be various in-field conditions that govern the usage of the feature. And this will reflect in the tests that need to be conducted to ensure the feature works as expected.

In RAN4 context this translates to requirements that need to be defined as configurable conditions in which a ML-enabled feature is required to undergo a post-deployment test (Option 1)

RAN4 should further discuss the possible in-field configurable conditions and requirements for a post-deployment test of an ML-enabled feature in Option 1.

      - Post-deployment Option 2

The main steps for this Option 2 are described below. The first 3 steps are in accordance with currently discussed RAN2 procedures, Step 4 and Step 7 are common for the post-deployment Option 1 and Option 2, while Steps 5-6 are the post-deployment testing Option 2 related (high-level) steps:

- The UE is assumed to have provided its relevant (ML-enabled feature) capabilities to the gNB/NW

- The gNB/NW has configured the UE for the ML-enabled feature, including in/applicability indications and the monitoring reports

- Based on the received in/applicability indications and/or monitoring reports the gNB/NW take ML management (“LCM”) actions (e.g. RRC re-configurations)

- The post-deployment testing can be triggered by the NW when the UE indicates the applicability of a functionality/configurations which was previously indicated as in-applicable to the NW.

Note: It is up to NW implementation what available information to use to decide on the need for triggering the post-deployment testing of the functionality/configurations declared as applicable by the UE.

- If the gNB/NW decides that a post-deployment test is required for the ML-enabled feature, then it will configure a test-active mode to run the post-deployment test for the feature.

  - The test-active configuration in Option 2 consists of supported RRC reports and performance monitoring reports configurations for the feature being tested

- The performance monitoring results corresponding to the test-active configuration are collected.

- Based on the test results:

    - If the performance KPI(s) are met, then the tested ML-enabled feature configuration (functionality) can be RRC/MAC activated to be used for inference

    - If the performance KPI(s) are not met, then the tested ML-enabled feature configuration (functionality) will not be activated, and another configuration/functionality or the fallback mechanism can be activated instead.

In RAN4 context there are two related aspects to be considered for Option 2. First, we assume that RAN4 will address the requirements related to performance monitoring as appropriate for all use cases.

RAN4 to consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the configured, active and applicable functionality/ configuration, sent from the UE, and set requirements as needed.

Secondly, due to the similarity with the RAN2 discussed performance monitoring (for the configured, active and applicable functionality/ configuration), we propose to also address in RAN4 the similar requirements for the configured and test-active functionality/ configuration.

RAN4 to consider the need and feasibility of requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration, and set requirements as needed

The introduction of the test-active functionality/ configuration can lead to the need to define the corresponding procedures e.g., re-configuration messages of the performance monitoring to enable monitoring of the test-active functionality/configuration, in RAN2. In RAN4 context, however, the actual test requirements may be the same as for the normal applicable functionality/ configuration (Observation 1). This indicates a potentially easier adoption of Option 2 in the time-frame of Release 19.

Details on Further potential clarifications (not resolved)

In this section we discuss about issues that need further clarifications (as listed in the table row 4) and are not resolved in the past meetings as identified and noted down in the last meeting.

      - Post-deployment Option 2

Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.)

Considering the general scope of the RAN4 performance testing and post-deployment testing it is not to address explicit ML model performance (based on RAN1- RAN2 agreements on performance monitoring), the answer to this question is clearly: not possible.

Based on the current RAN1- RAN2 agreements for Release 19, RAN4 to not purse testing procedures for individual UE-side ML model performance testing purposes.

Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs.

Performance monitoring procedures, specific to ML-enabled features, are being designed in RAN1-RAN2 for release 19 use cases. Therefore, it is very reasonable to assume that RAN4 can address the necessity of testing requirements for performance monitoring procedures.

Based on the current RAN1- RAN2 agreements for Release 19 on performance monitoring, RAN4 to study the necessity and feasibility of testing requirements to ensure consistent performance monitoring between different UEs can be achieved.

Furthermore, the Option 2 solutions also require the monitoring procedures for the test-active configurations as described in Section 2.1.2.

      - Common between Post-deployment Option 1 and Post-deployment Option 2

Whether the testing of a new model is on a device or in a lab.

We are of the opinion that the UE device should be tested in the field and not in the lab. As a test cycle, testing of implementation changes/updates of functionalities are not feasible in the lab due to the following reasons:

- Frequent updates to the AI/ML models supporting the ML-enabled feature configurations: The companies can come up with very frequent ML model updates and this will increase the test frequency and duration proportionally.

- High turnaround time for the conformance tests: For the conformance tests to be done across different ML-enabled feature configurations before deployment, even if we assume that an abridged set of conformance tests impacting the functionality alone is decided for this purpose, it could increase the turnaround time for the test significantly as the testing is done on a per device basis.

Frequent updates to the UE specific ML models supporting the ML-enabled feature configurations may significantly prolong the turnaround time and testing overhead if tested the lab.

Conformance testing for changed/updated ML functionality before deployment (Option 1) in the lab can be time consuming and it must be de-prioritized.

Whether models that have been updated post conformance testing should be kept in a device.

In this case we are of the opinion that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance test should be kept in the device. In case of a malfunction is identified during performance monitoring, then the fallback/switch should be allowed only to the functionality or legacy (non-AI/ML) feature that has passed conformance testing. This would mean that the functionality that has passed conformance testing should be always available in the UE device.

RAN4 to ensure and to clarify that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance testing shall always be available in the device.

Whether parallel operating of one model and (in-device) testing of another can be assumed.

Parallel operating of one ML-enabled feature configuration along with another ML-enabled feature configuration can be a good solution for realizing post-deployment testing of an updated ML-enabled functionality. However, this must be supported by the device which is capable of running several inferences (and the underlying ML models) in parallel. If supported, then one functionality can be active, and another can be in test-active mode (discussed in earlier section) while it is undergoing post-deployment test.

This can be realized in different ways based on how inference for the active and test-active ML-enabled functionalities can be arranged.

- Time split inference scheme for post-deployment testing

In this case the UE can do inferences across the active ML-enabled functionality and test-active ML-enabled functionality that is being tested in tandem.

- Regular inference: The major percentage of total time is used/allocated to make inference of the currently active functionality (to support the respective use-case)

- Validation Inference: The remaining/smaller percentage of the total time is allocated to make inference of the updated/new model/functionality in the device for the purpose of testing/validation in test-active state before it can be activated.

Note that in this case the model switch time is not negligible, and it is necessary to allocate model switching slot between the regular inference slot and validation inference slot.

- Parallel inference scheme for post-deployment testing

If UE is capable of parallel inference for several ML-enabled functionality at once, then the inference made by the currently active functionality will be used to support the use-case whereas the inference made by the test-active functionality can be used for the testing/validation of the updated functionality before activation.

- Note: Both the schemes of post-deployment testing described above can be used as part of Model Test Framework (proactive testing approach) as well as Model Monitoring Framework (reactive testing approach).

In order to validate the new ML-enabled feature configuration before using it in the field, the UE need to support inference either in consecutive/time splitting manner or in parallel.

RAN4 to further discuss the option of using both active as well as test-active in parallel to realize post-deployment testing of updated ML-enabled configuration in the field.

Whether signaling from a UE of changes/updates are needed.

To realize the post-deployment of an ML-enabled functionality in the field, it is desirable for the NW to be notified by the UE on the update of its implementation supporting an ML-enabled feature configuration. Potentially, based on the “type” of update that the ML-enabled functionality has undergone, the NW can arrange the test suite that needs to be run as part of in-field post-deployment testing of the functionality.

An illustration of the signaling flow between the UE and the NW is illustrated in Figure 3.

The trigger for the post-deployment test itself can be based on the applicability report that the UE sends to the NW (based on agreed RAN2 applicability reporting procedures). This consists of including in the applicability report additional information about the update “type” that the feature has undergone. Based on this if the NW decides that a post-deployment test is required, then it can re-configure the UE accordingly (see Option 1 or Option 2). Based on the outcome of the test(s), it can be decided if the updated functionality can be activated, or a fallback is required.

Figure 3:Typical signaling exchange between the UE and NW to indicate the update to an ML-enabled functionality

It should be noted that the Option 1 is not fully supported by RAN2.

RAN4 should further discuss on the potential signaling requirements to realize the post-deployment tests in Option 1.

In order to realize post-deployment testing in Option 2, the signaling requirements can be part of the UE-side “LCM” signaling that are being discussed in RAN2.

Updates to current UE-side “LCM” signaling procedures are required to realize post-deployment testing of updated ML-enabled functionality in Option 2 as a part of performance monitoring. This must be done in alignment with RAN2 and impacts to RAN4 should be further discussed.

Whether there is any relation to GCF procedures.

Relation to GCF should be discussed in RAN5 when RAN5 discussions start.

Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.

For this option, we believe that a combination of Option 1 and Option 2 can also be viable way forward. The Option 1 ensures that the functionality configuration can be tested before it is activated (for inference and reporting) by the NW. And in Option 2 the performance monitoring at the NW-side is used to detect performance changes of the already activated UE-side AI/ML-enabled feature configurations. The combination of these mechanisms, supported by the signalling from the UE about the changes/updates in AI/ML-based feature, can ensure a high reliability of the post-deployment testing, hence increase the trustworthiness of the AI/ML operation in live networks.

RAN4 to consider a combination of Option 1 and Option 2 as the best way forward solution for the post-deployment testing.

Life-Cycle Management related aspects

The moderator suggested the following way forward in RAN4#114 for LCM related aspects.

## Sub-topic 1: Necessity of delay and tolerance requirements for monitoring There was no agreement on this topic. Here, a summary of some issues to consider is provided for the purpose of stimulating discussion in future meetings: Delay and tolerance requirement for input data for monitoring: - Is this related to the delay before a monitoring report update reflects the change in input conditions, or is it related to the delay before LCM has taken action due to the change in conditions ? - If it is related to the delay before LCM has taken action, if the LCM is on the UE side, is it visible in the 3GPP specifications ? If it is on the network side, how would a requirement be defined on the UE ? - What is meant by tolerance ? Is it the accuracy of the monitoring report ? Is it for monitoring operation or is it TE tolerance ? - Different AI models may have different generality; in one case a model may accommodate a change in input data with no actions; in another case there may be a need for LCM action when input data changes. How to take into account of this variation ?

- In case of NW sided and UE assisted performance monitoring, UE will either report measurements of the configured monitoring RS resources to the network in addition to the predictions or will directly report performance metrics to the network, respectively. In case measurement reports, UE is supposed to measure configured monitoring RS resources and report the measurement as in the legacy scenario, therefore, the delay requirement of such reports should not have any impact on legacy measurement period requirements.

In case of NW sided performance monitoring, UE should follow legacy measurement period requirements for the measurement of monitoring RS resources.

In case of UE assisted performance monitoring, UE will be configured by the network to report the performance metric after comparing the prediction with the measurement of configured monitoring RS resources. These reports are different from legacy reporting therefore, RAN4 should define requirement for delay for performance metric reporting.

In case of UE-assisted performance monitoring, RAN4 should consider defining delay requirement for performance metric reporting.

Tolerance can be defined as the level of performance degradation that needs to be reached before taking any LCM related action.  However, RAN4 doesn’t need to define any requirement for tolerance level, particularly for NW sided LCM. In our view, tolerance shouldn’t be taken as the accuracy of the monitoring report. For performance monitoring UE is supposed to report a degraded performance in case of inaccurate predictions and a good performance in case of accurate predictions. Therefore, to test such a metric, AI/ML model/functionality needs to be forced to perform certain type of predictions, which is not feasible to do in a meaningful way.

Tolerance shouldn’t be defined as the accuracy of the performance monitoring report.

Tolerance for performance monitoring should be defined as the level of performance degradation that needs to be reached before taking any LCM related action. RAN4 should not define any requirement for tolerance level for both NW and UE sided LCM.

## Sub-topic 2: Delay requirements for activation and de-activation of functionality There was no agreement on this topic. Here, a summary of some issues to consider is provided for the purpose of stimulating discussion in future meetings: - Alignment of terminology on “activation” of AI/ML functionality, “de-activation” of AI/ML functionality, “switching” AI/ML functionality etc… with RAN1/RAN2 - Definition of the starting point for <activation/de-activation> delay - What de-activation delay means; e.g. model no longer operative, or non-AI actived. Which use cases delay requirements are needed for (BM, CSI prediction, positioning)

Alignment of terminology with RAN1/RAN2

To align RAN4 terminology with RAN1/RAN2, we start by summarizing here the main agreements made by RAN2 and RAN1 regarding LCM aspects.

In RAN2#127, definitions of supported, applicable, and activated functionalities were agreed upon. These definitions are to be used in the LCM procedures of both UE-sided beam management and positioning use cases.

Agreements on definitions from RAN2#127: - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refer to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference

For the UE-sided beam management use case, RAN2 agreed as working assumption on a 5-step procedure, intended to cover the main signalling steps from UE capability enquiry up to NW provisioning and activation of the ML functionality/configuration.

We quote below the agreements related to this 5-step procedure from RAN2#127, RAN2#127-bis, RAN2#128, and RAN2#129 meetings.

Agreements on procedures from RAN2#127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig. - Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality. Agreements on procedures from RAN2#127-bis: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - FFS if inference configuration can be signalled in step3. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality). Agreements on procedures from RAN2#128: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable. - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. Verify this aligns with RAN1 configuration design. - Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover. - Source cell UAI (as is) can be sent from source cell to target cell using existing signaling. No further optimizations will be considered in RAN2 related to UAI. Agreements on procedures from RAN2#129: - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig). - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - The provided periodic CSI configuration should be consistent with reported UE capabilities. - FFS option B - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

Related to the Steps described above, RAN2 sent an LS to RAN1 on applicable functionality reporting for beam management UE-sided model [R2-2407848]. In response to this LS, RAN1 made the following agreements [R1-2410898].

Agreements from RAN1#119 related to RAN2 LS: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 RAN1 Conclusion: For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

AI/ML functionality activation:

Based on the above agreements, an activated functionality refers to a functionality already enabled for performing inference. Hence, according to RAN2, functionality activation refers to the process of enabling an applicable functionality to perform inference.

RAN4 to adopt that, according to RAN2, AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference.

AI/ML functionality deactivation:

RAN2 made the following agreement in RAN2#128: “When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.” Hence, based on this agreement, functionality deactivation refers to the process where the network deactivates an active functionality (e.g., when this functionality becomes non-applicable).

According to RAN2, one example of cases where the network deactivates a functionality is when it becomes non-applicable.

RAN4 to adopt that, according to RAN2, AI/ML functionality deactivation refers to the process of network deactivating an active functionality.

AI/ML functionality activation delay

According to the latest RAN1 and RAN2 agreements, the activation of AI/ML functionalities for UE-sided beam management differs based on the type of the CSI report (periodic, semi-persistent, or aperiodic).

RAN1 agreed that periodic CSI Report is considered as activated after RRCReconfigurationComplete. Subsequently, RAN2 agreed that, if option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in Step 4 (i.e., without need to wait RRCReconfiguration in Step 5).

For periodic CSI reporting in UE-sided beam management, the starting point of the AI/ML functionality activation delay is when UE receives RRCReconfiguration message with the configuration of the AI/ML functionality.

RAN1 agreed that aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. Subsequently, RAN2 agreed that semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework (i.e., semi-persistent reporting is activated by MAC CE/DCI, while aperiodic CSI reporting is activated by DCI).

For aperiodic and semi-persistent CSI reporting in UE-sided beam management, the starting point of the AI/ML functionality activation delay is when UE receives the MAC CE/DCI command to activate the functionality.

We believe that we can also start the discussion about the ending point of the activation delay.

The ending point of the AI/ML functionality activation delay is when UE starts reporting inference to the network.

AI/ML functionality deactivation delay

Compared to functionality activation, not many agreements were made by RAN1/RAN2 about the AI/ML functionality deactivation. The main agreement was made in RAN2#128: “When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.”

We believe that RAN4 can start the discussion on the starting point of the deactivation delay using this agreement. However, it is necessary that RAN4 keeps track of any new agreements made by RAN1/RAN2 about functionality deactivation.

The starting point of the AI/ML functionality deactivation delay is when UE receives the command to deactivate the functionality from the network.

RAN4 to monitor RAN1/RAN2 agreements about AI/ML functionality deactivation to revisit the starting point definition and define the ending point.

Delay requirements for one-sided use cases

- Alongside beam management, RAN2 has defined the following LCM procedure for UE-sided AI positioning case 1.

Agreements from RAN2#127-bis: - The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. - Step 5: The LMF requests the inferred location information using the LPP request location information message. - Step 6: UE reports the inferred location using LPP provide location information message. Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received. Agreements from RAN2#129: - RAN2 has not and will not study Case 2a and Case 2b in Rel-19. An LS was sent to SA2 to notify this agreement (LS available in R2-2501507) - Introduce AI/ML positioning Case 1 as a new positioning method - Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. - FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation. - The content of error cause is discussed while drafting stage3 CRs. - As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. - UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

Specifying different LCM procedures for the different one-sided use cases implies the need for defining use case-specific LCM-related delay requirements (e.g., activation/deactivation delays)

RAN4 to define use case-specific LCM-related delay requirements following the LCM procedures defined by RAN1/RAN2

On Generalization, number of tests and non-stationary conditions.

At the RAN4#112 meeting, it was agreed to introduce the new definition of the testing goals that is defined in the feature/functionality level:

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Based on the recent agreements in the RAN WG1 and WG2, we propose the following possible illustration of interplay in between AI/ML-enabled Feature functionalities (to be interpreted here as indicating the set of parameters and their values as supported for a particular Feature configuration based on the UE capabilities), Associated ID and applicability of the functionality (as discussed in RAN2), shown in Figure 1. The logical ML models are shown for illustration purposes only, as it was agreed in RAN4 that the requirements will be defined on the level of UE capabilities and functionalities, even though they can be provisioned by one or several UE-side ML models, and this is left up to UE implementation.

Figure 4: Illustration of simplified relationship in between AI/ML-enabled Feature (NW-UE), functionalities (UE), Associated IDs (NW) and logical ML models (UE) based on recent RAN1 and RAN2 agreements.

The point of the diagram in 4 is to understand how the requirement/test parameters/configurations will need to be defined in RAN4 so that the UE-side AI/ML-enabled features functionalities can be (adequately) tested. Following the diagram, the test configurations parameters for test requirements that needs to be specified can be split at least into the following groups:

- Parameters that are defined by the Feature functionality configurations based on the UE capabilities

- Parameters that are defined by the Associated IDs (i.e., NW-related parameters including NW side additional conditions) that help the UE to determine/check the functionality applicability (RAN2) and enable consistency between training and inference conditions (RAN1) for the UE-side AI/ML-enabled Features functionalities.

- Other/additional parameters that cannot be defined neither by the Feature functionality/configuration nor include in the Associated ID e.g., propagation conditions/channel model, UE-side configurations/additional conditions, etc.

Correspondingly, it can be expected that every specified AI/ML-enabled Feature, with functionalities/configurations from group (1) specific to it, shall be tested i.e., at least one requirement/test per functionality shall be defined. However, the total number of requirements per UE Feature configuration will be dependent on whether an explicit list of Associated ID will be specified or not. If yes, then it will be necessary to define requirements per each of those because which of the Associated IDs are applicable to a UE Feature configuration will be UE-specific. For example, some UEs might support best beam prediction for certain gNB beam codebooks, but others - not. Then, the explicit list of Associated IDs will not be specified, and the definition of requirements becomes more challenging because potentially it could be UEs without any applicable Associated IDs leaving the functionality without test. This should not be the case. Therefore, at least a minimum set of representative Associated ID(s), NW and UE vendor agnostic, shall be considered as mandatory/default and, be applicable and testable for the ML-enabled Feature configurations.

The way how test configurations/parameters and requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized in RAN1 and RAN2.

Each specified AI/ML-enabled Feature shall have RAN4 requirements and UE needs to pass at least one test for specific sub-set of applicable Feature configurations (matching functionalities) as part of conformance testing.

A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

The change of the Associated ID (parameter group (2) in the list above) may be indicated in a RRC reconfiguration message because UE will need to re/check the applicability/compatibility of the Associated ID to its AI/ML-enabled Feature. Thus, the UE should have sufficient time to check the new conditions and adapt if needed e.g., to switch its underlining ML models if needed or to report that such configuration is not applicable at all. Therefore, we do not see a need to define requirements that will cover a possible of change Associate ID during the test run/sequence.

A considerable change of NW-side conditions that require the change of Associated ID may require RRC reconfiguration for the UE.

RAN4 does not need to define performance requirements that imply the change of NW configuration/Associated ID during the test.

Lastly, we have a group of parameters/configurations that are not explicitly defined neither by the feature/functionality nor by the Associated ID (parameters in group (3) in the list above). For example, the capability of the UE to maintain the minimal level of performance in different radio propagation conditions is the core question of generalization (and valid problem regardless of the use/non-use of ML algorithms) and should be ensured by the sufficient test/requirement coverage. We envision two open issues related to such parameters:

- What should be the number of additional test requirements per given UE AI/ML-enabled Feature configuration and selected Associated ID(s) combination?

  - This will depend, for example, whether the deployment, UE configurations or channel model will be specified by those or not.

  - On top of that, it will be necessary to study how the performance of the AI/ML-enabled Feature is sensitive to variations in those test parameters in group (3), i.e., how well the functionality generalizes to changes in those.

- Are there any potential degradations in the performance when the change of test parameters/configurations happens dynamically, i.e., in non-static way?

  - We cannot expect that RRC reconfiguration happens when additional conditions / parameters / configurations (e.g. channel conditions) change. UE should use its internal mechanisms/implementation to ensure good performance of the functionality, e.g., by changing to the proper underlying model within functionality to match the conditions.

Regarding the first issue above, we think that more clarity from other WGs and a study of generalization UE capabilities to different scenarios will be needed in RAN4. It will still take some time.

The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in per use-case manner.

For the testing of Generalization aspects, RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Regarding the second issue above, when additional conditions might change without the change of the AI/ML-enabled Feature configuration and Associated ID, it brings us to the question of non-static test conditions. At the previous RAN4 meetings, we have already discussed such tests and definition of non-static conditions:

Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

The current RAN4 (and to large degree also RAN1) assumptions for UE feature requirements, testing and evaluations are based on static scenarios and configurations, which correspond to stationary radio channel conditions. In our understanding, for the ML-enabled features to be introduced in NR Release 19, these assumptions are still valid and can be used as baseline. Additional assumptions and conditions for testing e,g, in the form of non-static scenarios (for non-stationary radio channel conditions) can be introduced on a use case basis.

The static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID shall be assumed as baseline for RAN4 requirements purposes.

Conclusion

In this contribution, we addressed following topics:

- Post-deployment testing

- Life-Cycle Management related aspects

- Generalization, number of tests and non-stationary conditions

Following Observations and Proposals were made in this paper:

Proposal 1: The RAN4 discussions to use “post-deployment testing” defined as “the testing of functionality/configuration in a UE which is operating in the field, connected to a mobile network”.

Proposal 2: The RAN4 discussions to assume Option 1 described as “Post-deployment pre-activation functionality/configuration update testing”.

Proposal 3: The RAN4 discussions on the functionality/configuration which requires testing before it can be RRC/MAC activated by the NW for inference, to use “pre-activation” term to indicate “the functionality/configuration is available in (and supported by) the UE device, and it is not RRC/MAC activated”.

Proposal 4: The RAN4 discussions to assume Option 2 described as “Post-deployment post-activation functionality testing based on performance monitoring”

Proposal 5: The RAN4 discussions to use “test-active” defined as “the functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”

Proposal 6: RAN4 should discuss on the framework related to the test-active mode of the ML-enabled feature to realize the post-deployment testing in Option 1.

Proposal 7: RAN4 should further discuss the possible in-field configurable conditions and requirements for a post-deployment test of an ML-enabled feature in Option 1.

Observation 1: RAN4 to consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data, for the configured, active and applicable functionality/ configuration, sent from the UE, and set requirements as needed.

Proposal 8: RAN4 to consider the need and feasibility of requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration, and set requirements as needed

Proposal 9: Based on the current RAN1- RAN2 agreements for Release 19, RAN4 to not purse testing procedures for individual UE-side ML model performance testing purposes.

Proposal 10: Based on the current RAN1- RAN2 agreements for Release 19 on performance monitoring, RAN4 to study the necessity and feasibility of testing requirements to ensure consistent performance monitoring between different UEs can be achieved.

Observation 2: Frequent updates to the UE specific ML models supporting the ML-enabled feature configurations may significantly prolong the turnaround time and testing overhead if tested the lab.

Proposal 11:  Conformance testing for changed/updated ML functionality before deployment (Option 1) in the lab can be time consuming and it must be de-prioritized.

Proposal 12: RAN4 to ensure and to clarify that at least one version of the ML-enabled feature configuration (including the supporting ML models), that has passed conformance testing shall always be available in the device.

Observation 3: In order to validate the new ML-enabled feature configuration before using it in the field, the UE need to support inference either in consecutive/time splitting manner or in parallel.

Proposal 13: RAN4 to further discuss the option of using both active as well as test-active in parallel to realize post-deployment testing of updated ML-enabled configuration in the field.

Observation 4: It should be noted that the Option 1 is not fully supported by RAN2.

Proposal 14: RAN4 should further discuss on the potential signaling requirements to realize the post-deployment tests in Option 1.

Observation 5: In order to realize post-deployment testing in Option 2, the signaling requirements can be part of the UE-side “LCM” signaling that are being discussed in RAN2.

Proposal 15: Updates to current UE-side “LCM” signaling procedures are required to realize post-deployment testing of updated ML-enabled functionality in Option 2 as a part of performance monitoring. This must be done in alignment with RAN2 and impacts to RAN4 should be further discussed.

Proposal 16: Relation to GCF should be discussed in RAN5 when RAN5 discussions start.

Proposal 17: RAN4 to consider a combination of Option 1 and Option 2 as the best way forward solution for the post-deployment testing.

Proposal 18: In case of NW sided performance monitoring, UE should follow legacy measurement period requirements for the measurement of monitoring RS resources.

Proposal 19: In case of UE-assisted performance monitoring, RAN4 should consider defining delay requirement for performance metric reporting.

Proposal 20: Tolerance shouldn’t be defined as the accuracy of the performance monitoring report.

Proposal 21: Tolerance for performance monitoring should be defined as the level of performance degradation that needs to be reached before taking any LCM related action. RAN4 should not define any requirement for tolerance level for both NW and UE sided LCM.

Proposal 22: RAN4 to adopt that, according to RAN2, AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference.

Observation 6: According to RAN2, one example of cases where the network deactivates a functionality is when it becomes non-applicable.

Proposal 23: RAN4 to adopt that, according to RAN2, AI/ML functionality deactivation refers to the process of network deactivating an active functionality.

Proposal 24: For periodic CSI reporting in UE-sided beam management, the starting point of the AI/ML functionality activation delay is when UE receives RRCReconfiguration message with the configuration of the AI/ML functionality.

Proposal 25: For aperiodic and semi-persistent CSI reporting in UE-sided beam management, the starting point of the AI/ML functionality activation delay is when UE receives the MAC CE/DCI command to activate the functionality.

Proposal 26: The ending point of the AI/ML functionality activation delay is when UE starts reporting inference to the network.

Proposal 27: The starting point of the AI/ML functionality deactivation delay is when UE receives the command to deactivate the functionality from the network.

Proposal 28: RAN4 to monitor RAN1/RAN2 agreements about AI/ML functionality deactivation to revisit the starting point definition and define the ending point.

Observation 7: Specifying different LCM procedures for the different one-sided use cases implies the need for defining use case-specific LCM-related delay requirements (e.g., activation/deactivation delays)

Proposal 29: RAN4 to define use case-specific LCM-related delay requirements following the LCM procedures defined by RAN1/RAN2

Observation 8: The way how test configurations/parameters and requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized in RAN1 and RAN2.

Proposal 30: Each specified AI/ML-enabled Feature shall have RAN4 requirements and UE needs to pass at least one test for specific sub-set of applicable Feature configurations (matching functionalities) as part of conformance testing.

Proposal 31: A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

Observation 9: A considerable change of NW-side conditions that require the change of Associated ID may require RRC reconfiguration for the UE.

Proposal 32: RAN4 does not need to define performance requirements that imply the change of NW configuration/Associated ID during the test.

Observation 10: The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in per use-case manner.

Proposal 33: For the testing of Generalization aspects, RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Proposal 34: The static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID shall be assumed as baseline for RAN4 requirements purposes.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.