3GPP TSG-RAN WG4 Meeting #114bis	R4-2504233

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On RAN4 Impacts for Measurement Event Prediction in AIML Mobility

Agenda item:	7.21.2.2

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114 within the scope of Rel-19 SI ‎‎[1] and ‎[2]. Agreements achieved during this meeting are summarized in the WF ‎‎[3], where it was agreed that RAN4 would start the discussion on the measurement event prediction use case in the RAN4#114bis meeting.

In this paper, we provide our high-level views on measurement event prediction and outline the main initial steps that need to be agreed upon and discussed by RAN4.

Discussion

Implementation aspects of measurement event prediction

Regarding measurement event predictions, RAN2 considers two methods for predicting measurement events: indirect and direct measurement event prediction, as illustrated in Figure 1 and Figure 2, respectively.

Figure 1: Indirect measurement event prediction.

Figure 2: Direct measurement event prediction

In indirect measurement event prediction for intra-frequency scenarios—such as temporal domain case A, temporal domain case B, or the spatial domain—a Radio Resource Management (RRM) measurement prediction model first predicts the measurement result(s), potentially utilizing AI/ML models. Subsequently, the predicted results, along with optionally included actual historical measurement data from the same cell, are used to determine whether a measurement event will occur at a specific future time instance. The post-processing phase operates without further involvement of an AI/ML model.

A similar approach can be applied to frequency-domain indirect measurement event prediction. In this case, the process begins with an RRM measurement prediction model estimating the measurement result(s) for a different frequency layer. The predicted results, optionally combined with historical measurement data from the serving cell, are then used to determine whether a measurement event will occur at a particular time instance, again without further AI/ML model involvement.

As shown in Figure 2, direct measurement event prediction follows a different approach. While it uses the same input data as indirect prediction (as illustrated in Figure 1), it also allows for additional inputs. Unlike the indirect method, which first predicts intermediate measurement results before determining the occurrence of an event, the direct method predicts measurement events directly without relying on intermediate measurement results.

Although the choice between direct and indirect measurement event prediction is expected to be a UE implementation aspect, these methods may impose different requirements and performance constraints, requiring further study within the RAN4 SI.

For instance, in the case of Event A3, the UE is expected to transmit the measurement result once the reporting conditions are fulfilled. It may need to be investigated whether the same measurement reporting delay should apply to both indirect and direct prediction methods.

Additionally, while indirect measurement prediction can accommodate different Layer 3 filtering coefficients, a direct measurement model may only function with specific L3 coefficients. It might also be necessary to retain certain L3 filtering parameters after each L3 update to ensure model consistency.

Further evaluations are needed to analyze performance and requirements of these methods, particularly in terms of prediction accuracy, and reporting delay.

Direct and indirect measurement event prediction may have different requirements and performance characteristics.

RAN4 to study the potential differences in performance and requirements between direct and indirect measurement event prediction methods.

RAN4 to prioritize between studying direct and indirect measurement event predictions if they exhibit distinct performance and requirement differences. As a possible approach, RAN4 may follow RAN2 prioritization or alternatively focus on indirect measurement event prediction.

Temporal domain measurement event prediction

Figure 3 provides example of time domain case-A RRM measurement prediction.

Figure 3: Time domain case A RRM measurement prediction

In indirect measurement event prediction with time domain case A RRM measurement prediction, an A3 measurement event may be predicted before it actually occurs and the prediction potentially used in optimizing mobility decisions. Two options for handover execution decision have been documented in TR 38.744 by RAN2:

Option 1: Relying on legacy measurement event

Option 2: Relying on predicted measurement event

In both options, the decision for an early handover preparation may be done based on the predicted measurement event.

In time domain case B RRM measurement prediction, the number of required downlink radio measurements is reduced by the UE skipping specified measurements periods and predicting the skipped measurements from the past measurements using an ML model. Two examples for skipping patterns are shown in Figure 4.

Figure 4: Examples of time domain case B measurement prediction

It has been agreed in RAN2 to study the measurement event prediction also with time domain case B RRM measurement prediction in FR1. In this case, the PW is typically short, and predictions are not made for every measurement period, so any gains that can be expected from the future predictions is low. On the other hand, all mobility decisions are made based on measurement-event-triggered reporting based on actual measurements and predictions, which may set higher requirements for the accuracy of the predictions. Comparing the deployment and uses cases, case A and case B impose different requirements and performance metrics, necessitating further RAN4 evaluation.

In time domain case B measurement event prediction, the PW is typically short, and predictions are not made for every measurement period.

In time domain case B measurement event prediction, all mobility decisions are made based on measurement-event-triggered reporting based on measurements and predictions. Therefore, the goal of time domain case B is measurement reduction without causing any significant impact on the mobility performance.

The following scenarios can be considered for indirect measurement event prediction using temporal domain predictions:

- I: Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction.

- II: Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance.

For indirect measurement event prediction, the choice between temporal domain Case A and Case B may impose different requirements and performance metrics, necessitating further RAN4 evaluation.

RAN4 to prioritize, based on RAN2 outcome, from the available cases for indirect measurement event prediction using temporal domain Case A or Case B, and conduct a study on the performance and requirements for the selected case.

Based on TS 38.133, for event-triggered measurement reporting, the measurement reporting delay is defined as the time between the occurrence of an event that triggers a measurement report and the point when the UE begins transmitting the measurement report over the air interface.

The reported RSRP measurements in event-triggered measurement reports must comply with the requirements specified in 38.133, specifically in Clauses 10.1.2.1 (RSRP for FR1) and 10.1.3.1 (RSRP for FR2).

The UE shall send one Event A3-triggered measurement report within the defined measurement reporting delay. The UE shall not send event-triggered measurement reports if the reporting criteria are not met. The rate of correct events observed during repeated tests shall be at least 90%.

RAN4 to study the applicability of current measurement event criteria and reporting requirements/performance metrics for measurement event prediction and explore the need for new metrics.

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility measurement event prediction scenario to be studied in RAN4 working group.

The following Observations and Proposals were made:

Observation 1: Direct and indirect measurement event prediction may have different requirements and performance characteristics.

Proposal 1: RAN4 to study the potential differences in performance and requirements between direct and indirect measurement event prediction methods.

Proposal 2: RAN4 to prioritize between studying direct and indirect measurement event predictions if they exhibit distinct performance and requirement differences. As a possible approach, RAN4 may follow RAN2 prioritization or alternatively focus on indirect measurement event prediction.

Observation 2:  In time domain case B measurement event prediction, the PW is typically short, and predictions are not made for every measurement period.

Observation 3: In time domain case B measurement event prediction, all mobility decisions are made based on measurement-event-triggered reporting based on actual measurements and predictions. Therefore, the goal of time domain case B is measurement reduction without causing any significant impact on the mobility performance.

Observation 4: The following scenarios can be considered for indirect measurement event prediction using temporal domain predictions:

I: Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction.

II: Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance.

Observation 5: For indirect measurement event prediction, the choice between temporal domain Case A and Case B cases may impose different requirements and performance metrics, necessitating further RAN4 evaluation.

Proposal 3: RAN4 to prioritize, based on RAN2 outcome, from the available cases for indirect measurement event prediction using temporal domain Case A or Case B, and conduct a study on the performance and requirements for the selected case.

Proposal 4: RAN4 to study the applicability of current measurement event criteria and reporting requirements/performance metrics for measurement event prediction and explore the need for new metrics.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2502594, WF on AI/ML Mobility, Nokia, RAN4#114, Athens, Greece, 17th – 21st February, 2025.