# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504237

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.19.4

Source: 	OPPO

Title: 	RRM core requirement and testing framework for Positioning

Document for:	Approval

- Discussion

In RAN1 #110bis meeting, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 		UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In RAN #107 meeting, case 2a and case 2b have been removed from the Revised WID[1], and following objectives have been agreed.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the above use cases, as agreed in RAN4 #110 meeting, RAN4 will not define positioning accuracy requirements for case 3a/3b.

For case1, in RAN4 #112 bis meeting, RAN4 agreed that “companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible”. Whether and how to define positioning accuracy requirements for case1 is still highly controversial.

For case1(for direct AI/ML and AI/ML assisted positioning), companies have concerns on how to obtain the positioning label if we use positioning accuracy as a RAN4 test metric. The feasibility on defining positioning accuracy requirements for case1 is not clear.

For case1(for AI/ML assisted positioning), the model output includes RSTD, identification of LoS/NLoS, and other metrics that were agreed in RAN1. We need to note that these intermediate measurement results (RSTD, identification of LoS/NLoS, and other metrics) are not realistic measurement results. They are from the output of AI/ML models with non-linear processing. The feasibility of using these AI-processed intermediate results for case 1 tests is also not clear.

- Taking RSTD as an example, we need to think about and check what is the reference/ideal/label value for RSTD in case1(note, RSTD in case1 is obtained through AI model with nonlinear processing)? And, if there is already a method to obtain the reference/ideal/label RSTD value, why not use this ideal RSTD for positioning instead of the non-ideal RSTD obtained from the AI case1? This creates a chicken-and-egg problem.

In RAN4 discussions, some companies proposed that there are metrics like RSTD in legacy requirements. However, it should be emphasized that if meeting legacy RSTD metrics could achieve good positioning results, why couldn't legacy methods achieve high-precision positioning, especially in NLOS environments? If new requirements need to be determined using an “RSTD-type” metric, what would these new requirements be, and what would the reference/ideal/label RSTD be?

So, from our understanding, for case1(for AI/ML assisted positioning), the feasibility of tests on model outputs is not clear.

Based on the above discussion, and taking into account the limited time budget and the priority of different positioning cases in R19 WI, we proposed that:

Proposal 1: For case1, RAN4 will not define positioning accuracy requirements in R19 WI.

Observation 1: For case1(AI/ML assisted positioning), the feasibility of using intermediate results(output of AI/ML models with non-linear processing) for RAN4 tests is not clear.

- how to get the label data for intermediate results(e.g. RSTD, identification of LoS/NLoS and other metrics that agreed in RAN1) and test these intermediate results is not clear.

- Conclusions

In this contribution, we have got the following proposals:

Proposal 1: For case1, RAN4 will not define positioning accuracy requirements in R19 WI.

Observation 1: For case1(AI/ML assisted positioning), the feasibility of using intermediate results(output of AI/ML models with non-linear processing) for RAN4 tests is not clear.

- how to get the label data for intermediate results(e.g. RSTD, identification of LoS/NLoS and other metrics that agreed in RAN1) and test these intermediate results is not clear.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface