3GPP TSG-RAN WG4 Meeting #114bis                                 R4-2504240

Wuhan, China, 7th – 11th April, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	7.1.1.2.1

Document for:	Approval

- Introduction

In RAN#114, a WF was agreed upon in [1]. Agreements were made on the following:

- Boundary for Spurious Emission: Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW.

- Integral region for OOBE: Option 1: The OOBE requirements are based on UE CBW.

- MPR reduction with extended CBW is an optional feature with UE capability.

  - FFS details of UE capability

Other issues remained FFS. In this paper, we propose our views and discuss the remaining FFS issues.

- Summary of our Proposed Concept

The following is proposed as the concept to be specified in Rel-19:

- Both Symmetric and Asymmetric shifted OOBE are specified.

- Both Symmetric and Asymmetric shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms, at least for PC3.

- Shifted OOBE requirement specified for the following:

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a few discrete points.

- Shifted OOBE can be defined for any existing specified UE CBW up to the maximum CBW supported by the UE.

- Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE. For RBs outside of NRB, a “virtual extended NRB” term applies.

- Define dedicated RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, to allow mobile operators the choice of whether to allow it or not. It would be also be helpful if network signalling could also indicate what kind of “shift of OOBE” is permitted.

NOTE: We change the term from “extended CBW” to “shifted OOBE” because in our view the OOBE requirements apply based on the original UE CBW, and we select both asymmetrical and symmetrical approaches. Therefore, it seems easier to view it from the perspective of a shift of OOBE requirements.

- 4. Our detailed comments and justification for the above

Width/RB numbers of extended CBW at each side of UE CBW

We believe that defining both Symmetric and Asymmetric extensions of 0.5 * CBW on both sides or one side of the UE CBW is sufficient to cover most cases. This provides sufficient flexibility in case the UE is located at the edge of the spectrum block. Given the moderate gains of the feature, the additional complexity of defining a granular extension vs the complexity both in terms of specification, device testing and signalling does not seem very well justified. Furthermore, there appears to be NO benefit to MPR performance of extending beyond 0.5 * UE CBW on either side. We have also analyzed NRB/2 extension and identified that this could also be acceptable (assuming “ceil(NRB/2)” with guardband also added before the start of the OOBE domain).

Regarding whether to apply this for just the maximum CBW or smaller CBWs, we are open minded. There may be cases where the relaxed emission may apply in guardband, where the UE cannot be configured with a wider bandwidth, so it may make sense here to allow for smaller CBWs.

Proposal 1: Both Symmetric and Asymmetric shifted OOBE are specified.

Proposal 2: Shifted OOBE requirement to at least be specified for:

- 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

- 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

NOTE: It is also acceptable to replace 0.5 * UE CBW with “ceil(NRB/2 ) + UE CBW guardband”.

Proposal 3: Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a 1 or 2 discrete points.

Proposal 4: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

InBand Emissions (IBE) between UE CBW edge and extended edge

In terms of how to define the extended IBE requirement, we believe that applying an extension of the NRB term (NRB_EXT) would be sufficient. The following that the following approach would work:

- For Symmetrical extension (0.5 * UE CBW on each side), NRB_EXT = NRB of CBW corresponding to UE CBW*2 (for same SCS)

- For asymmetrical extension (0.5 * UE CBW on one side), NRB_EXT = NRB of CBW corresponding to UE CBW*1.5 (for same SCS)

- In case no defined CBW corresponds to UE CBW*2, NRB_EXT = floor(UE CBW*2 / (SCS * 12)) RBs.

- In case no defined CBW corresponds to UE CBW*1.5, NRB_EXT = floor(UE CBW*1.5 / (SCS * 12)) RBs.

It was pointed out in the last meeting in [2] that in the full RB allocation case, applying IBE in the extended region outside of the guardband could enable improved MPR performance (seemingly improved compared to even inner RB allocations). However, given that those emissions are most likely to be inside the operator spectrum block, we think it is more reasonable to apply the IBE as it would apply to a partial RB allocation within UE CBW today. However one could consider some exceptions for full RB allocation in the case that the UE CBW edge aligns with the spectrum block edge.

Proposal 5: Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE, for RBs outside of NRB, a “virtual extended NRB” term applies.

Proposal 6: Use the approach proposed in this document to derive NRB_EXT.

Why to specify the extension as “shifted OOBE” rather than “extended CBW”

When defining the SEM/ACLR requirements as integrated according to UE CBW, but where the starting point is shifted from the channel edge, this is not the same as the approach taken for scaling SEM/ACLR integration over a wider CBW.

It cannot be assumed that the existing MPR rules can be applied for any combination of UE CBW and wider overlaid CBW in the same way as for a partial RB allocation operating in such a wider CBW. This would need all of the cases (including all UE CBW locations within each wider CBW) to be simulated for every combination of CBW and wider CBW, which appears to be a lot of effort for very limited gain.

Proposal 7: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

What we have done is to simulate the Symmetrical and Asymmetrical shifted OOBE assuming 0.5 * CBW shift applying the SEM/ACLR according to UE CBW. The 20MHz results below are for 15kHz SCS which is a worst case than 30kHz SCS in terms of NRB and spectrum utilization. Therefore, the equivalent ratios can apply for 30kHz SCS too.

Table 1a: Evaluation results for Symmetric shifted OOBE (20MHz CBW with 0.5* CBW shift both sides

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 1b: Evaluation results for Asymmetrical shifted OOBE (20MHz CBW with 0.5 * CBW shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |

Table 1c: Evaluation results for Asymmetrical shifted OOBE (100MHz CBW with 0.5 * CBW shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |

Proposal 8: Both Symmetrical and Asymmetrical shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms for PC3. Application for PC2 is FFS.

For Symmetrical case, it can be observed that the maximum inner RB allocation is LCRB = NRB.

For the Asymmetrical case, it can be observed that the maximum inner RB allocation is LCRB = NRB*2/3, as also shown in the figure 1 below.

Figure 1: Asymmetric shifted OOBE

Proposal 9: Agree that the maximum inner RB allocation is LCRB = NRB with symmetrical shift of 0.5 * UE CBW on both sides, and that maximum inner RB allocation is LCRB = ceil(2*NRB/3) with asymmetrical shift of 0.5 * UE CBW on one side.

NOTE: It is also acceptable to replace 0.5 * UE CBW with “ceil(NRB/2 ) + UE CBW guardband” and apply the same proposal.

The following shows example modifications to the MPR formula to cover both Symmetrical and asymmetrical OOBE shifts, applying only for shifts of 0.5 * UE CBW.

RBStart,Low = min(max(1, floor(LCRB/2)), OOBExt,Low),

where OOBExt,Low:

= 0, if OOBE shift from lower UE CBW edge applied,

= ∞ otherwise

RBStart,High = NRB – min(max(1, floor(LCRB/2)), OOBExt,High) – LCRB,

where OOBExt,High:

= 0, if OOBE shift from upper UE CBW edge applied,

= ∞ otherwise

The RB allocation is an inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(NRB/2 * LCRB_ Ext) , where LCRB_ Ext:

= 2, if OOBE shifted by 0.5 * UE CBW on both upper and lower edge

= 4/3, if OOBE shifted by 0.5 * UE CBW on only one of either upper or lower edge

= 1 otherwise

Proposal 10: Consider the example spec changes in this section as a starting point for spec drafting for modification to the MPR section of TS 38.101-1.

Network-UE signalling aspects

It is important to quickly identify what sort of network-UE interactions are needed and inform RAN2 before the end of this quarter, given that RAN2 will functionally freeze RRC in September this year.

There are some different potential motivations for network to UE signalling, and in past meetings they have been somewhat bundled together in discussions. We believe it would be best to separate them.

Potential Motivations (1,2,3) are:

- To indicate to the UE that the MNO permits the application of relaxed OOBE in its spectrum block. This may be needed due to any geographically specific regulatory conditions not allowing the relaxed OOBE, or it may just be because the operator has some concern about the UE applying IBE requirement levels instead of OOBE outside of its CBW but inside the operator’s spectrum block.

Our view: This makes sense and de-risks deployment/adoption of the feature.

- To avoid the UE from having to check whether it is in a frequency location within the spectrum block that would be acceptable in terms of relaxing emissions. It is true that if the network provided this information to the UE then it would make the life simpler for the UE.

Our view: We think it would be simpler if the UE could rely on the network to indicate to UE what sort of shift would be permitted. Optionally the UE could also verify that it is adhering to regulations.

- To indicate exactly what is the level of extension permitted by the operator. This would assume that there is some network configurability of this relaxation.

Our view: If such configuration possibilities are limited, then this it could be acceptable.

Suggested RRC (network to UE) signalling concept:

Network to convey to UE (UE specific) whether shifted OOBE is permitted, and include the following additional information:

- Whether the shifted OOBE is permitted to be “symmetrical” or “asymmetrical”.

- The value of the shift permitted, may be default 0.5 x UE CBW.

- In case of asymmetrical shift, whether the shift applies to lower or upper side of UE CBW.

- FFS: In case of asymmetrical shift, in case RAN4 were to agree that both sides may apply a non-zero shift simultaneously, the permitted shift on each side would need to be signalled.

Proposal 11: Consider the following as a starting point for requirements for RRC signalling support:

- Network to convey to UE (UE specific) whether shifted OOBE is permitted, and include the following additional information:

  - Whether the shifted OOBE is permitted to be “symmetrical” or “asymmetrical”.

  - The value of the shift permitted, may be default 0.5 x UE CBW.

  - In case of asymmetrical shift, whether the shift applies to lower or upper side of UE CBW.

  - FFS: In case of asymmetrical shift, in case RAN4 were to agree that both sides may apply a non-zero shift simultaneously, the permitted shift on each side would need to be signalled.

UE capability signalling

We would propose the following as a starting point for the UE capability framework:

- UE can indicate support of symmetric and asymmetric shifted OOBE of 0.5 * UE CBW (or ceil(NRB/2 ) + guardband).

- FFS whether any a per UE CBW capability is needed.

- If other allowed frequency shifts were defined, the ue should be able to separately indicate support for those.

Proposal 12: Consider the following UE capabilities:

- UE to indicate support of shifted OOBE with symmetrical and asymmetrical with default supported frequency shift = 0.5 * UE CBW (or approximate equivalent).

- FFS whether any a per UE CBW capability is needed.

- If other allowed frequency shifts were defined, the ue should be able to separately indicate support for those.

A-MPR considerations

For permitted usage of A-MPR within a channel, we believe the interaction with enhanced MPR with shifted OOBE should be kept simple. Some simple rules can be considered for the case where NS values and A-MPR is permitted in the cell and where shifted OOBE requirements are also permitted:

- Where A-MPR is defined, then it should be allowed for the UE to apply the full A-MPR in the allocations for which A-MPR has been defined.

- Where A-MPR is not defined, it can be up to UE implementation as to whether to apply MPR or enhanced MPR, in consideration of the shifted OOBE permission.

Proposal 13: Agree the following rules for the case where NS values and A-MPR is permitted in the cell and where shifted OOBE requirements are also permitted:

- Where A-MPR is defined, then it should be allowed for the UE to apply the full A-MPR in the allocations for which A-MPR has been defined.

- Where A-MPR is not defined, it can be up to UE implementation as to whether to apply MPR or enhanced MPR, in consideration of the shifted OOBE permission.

- 5. Conclusion & Proposals

Proposal 1: Both Symmetric and Asymmetric shifted OOBE are specified.

Proposal 2: Shifted OOBE requirement to at least be specified for:

- 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

- 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

NOTE: It is also acceptable to replace 0.5 * UE CBW with “ceil(NRB/2 ) + UE CBW guardband”.

Proposal 3: Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a 1 or 2 discrete points.

Proposal 4: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

Proposal 5: Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE, for RBs outside of NRB, a “virtual extended NRB” term applies.

Proposal 6: Use the approach proposed in this document to derive NRB_EXT.

Proposal 7: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

Proposal 8: Both Symmetrical and Asymmetrical shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms for PC3. Application for PC2 is FFS.

Proposal 9: Agree that the maximum inner RB allocation is LCRB = NRB with symmetrical shift of 0.5 * UE CBW  on both sides, and that maximum inner RB allocation is LCRB = ceil(2*NRB/3) with asymmetrical shift of 0.5 * UE CBW on one side.

NOTE: It is also acceptable to replace 0.5 * UE CBW with “ceil(NRB/2 ) + UE CBW guardband” here.

Proposal 10: Consider the example spec changes in this section as a starting point for spec drafting for modification to the MPR section of TS 38.101-1.

Proposal 11: Consider the following as a starting point for requirements for RRC signalling support:

- Network to convey to UE (UE specific) whether shifted OOBE is permitted, and include the following additional information:

  - Whether the shifted OOBE is permitted to be “symmetrical” or “asymmetrical”.

  - The value of the shift permitted, may be default 0.5 x UE CBW.

  - In case of asymmetrical shift, whether the shift applies to lower or upper side of UE CBW.

  - FFS: In case of asymmetrical shift, in case RAN4 were to agree that both sides may apply a non-zero shift simultaneously, the permitted shift on each side would need to be signalled.

Proposal 12: Consider the following UE capabilities:

- UE to indicate support of shifted OOBE with symmetrical and asymmetrical with default supported frequency shift = 0.5 * UE CBW (or approximate equivalent).

- FFS whether any a per UE CBW capability is needed.

- If other allowed frequency shifts were defined, the ue should be able to separately indicate support for those.

Proposal 13: Agree the following rules for the case where NS values and A-MPR is permitted in the cell and where shifted OOBE requirements are also permitted:

- Where A-MPR is defined, then it should be allowed for the UE to apply the full A-MPR in the allocations for which A-MPR has been defined.

- Where A-MPR is not defined, it can be up to UE implementation as to whether to apply MPR or enhanced MPR, in consideration of the shifted OOBE permission.

- 6. References

- R4-2503013, “WF on power domain enhancements”, Moderator (Huawei)

- R4-2501832, “On power domain enhancements for single carrier”, Huawei