3GPP TSG-RAN WG4 Meeting #114bis	R4-2504245

# Wuhan, China, 7th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance of R19 ATG

Agenda Item:	7.12.5

Document for:	Discussion

- Introduction

In RAN plenary 106 meeting, a revision WI of NR Rel-19 ATG led by RAN4 was approved in [1]. We summarize the objectives of the WI as below:

4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify the TAE requirements for support of DL MIMO for BS. [RAN4]. - Specify signaling and UE capabilities if necessary [RAN2]. 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

RAN4 had achieved good progress on the core part of R19 ATG. From this meeting, the performance part of ATG is to be kicked off in this meeting. In this paper, we provide our further analysis on remaining RRM aspects of R19 ATG.

- Discussion

- The discussion of R18 single carrier ATG performance part and the final test case design could provide meaningful reference for R19 ATG.

- 2.1 General

- General principle

- One general principle from our side is that RAN4 does not need to repeat the R18 ATG performance verification, and only focus on the verification of new functions or features introduced in R19 ATG, i.e. mainly the DL CA relevant features and antenna array relevant features.

- Proposal 1: The performance verification should focus on the new functionalities introduced for R19 ATG that were not defined in R18 single carrier ATG, i.e. mainly the DL CA relevant features and antenna array relevant features.

- Test configuration

- Regarding the test configuration, besides the intra-band CA and/or inter-band CA configuration, actually all test configuration defined in R18 ATG could be reused, such as general test configuration, TDD pattern, RMC, OCNG, channel model, BS location, number of neighbour cells for test of antenna array.

Proposal 2: Reuse the general test configuration defined in R18 ATG, and the intra-band CA and/or inter-band CA relevant configuration is additional considered.

- The general test configuration includs bandwidth/duplexing, TDD pattern, RMC, OCNG, channel model, BS location, number of neighbour cells for test of antenna array.

- Test method for UE with antenna array

- Regarding the test method for antenna array case, due to it is new challenge and not involved in legacy FR1, RAN4 had held quite sufficient discussion during R18 ATG performance part with both RRM and RF delegates involved in. All progress during the whole discussion currently are shown as below:

Test method for UE with antenna array in R18 ATG Agreement: - On the test method for UE with antenna array, further discuss: Whether OTA test is feasible. Whether/how if conducted test is to be used. FFS whether scaling factor needs to be considered in the test requiremetns. Agreement: - Use the test method of conducted test Endorse the CRs without scaling factor (scaling factor = [1]), and close the WI performance part in this meeting. For the draft CRs, add an Editor’s note that: “the scaling factor may be revisited if needed”. Agreement: - Conducted test without scaling factor (Scaling factor = 1) Agreement: - For ATG UE with the antenna array, if scaling factor is 1 in the test cases, no need to consider the sharing factor between L1 and L3 measurement.

- Here for the antenna array assymption for R19 ATG, actually we can classify all allowed cases into 2 categories:

- Category 1: common Rx beam in PCC and SCC band(s), including intra-band antenna array and Case 4 of inter-band

- Category 2: separate Rx beam in PCC and SCC bands, including other cases

- For Category 1, it is similar as the antenna array in R18 ATG, so directly reuse the above method defined in R18 ATG.

- For Category 2, based on our analysis in core part, the L1/L3 sharing factor could be assumed as 1 since UE could perform RRM measurement and L1 measurement in parallel. It seems all above agreements are applicable as well.

- Proposal 3: Reuse the test method defined in R18 ATG for all antenna array cases including intra-band CA and inter-band CA.

- Accuracy requirement

- We can not see any potential impact on the accuracy requirements. So it is preferred to reuse L3 and L3 accuracy requirements of legacy FR1 TN, which is applied in R18 ATG.

- Proposal 4: Reuse the L3 and L1 accuracy requirement of legacy FR1 TN

- 2.2 Test case design

For the exact test case, based on the progress achieved in core part, we defined core part requirements on the following aspects:

- Timing

  - No MTTD defined for intra-band/inter-band ATG

  - MRTD is defined for inter-band ATG, same as legacy FR1 inter-band CA

- IDLE/INACTIVE

  - ΔRIB,c, is introduced to the exception of side conditions for cell selection measurements, same as RF session

- Measurement procedure

  - Introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, same as legacy TN

  - L1/L3 measurement sharing factor is defined for RRM measurement

  - Psharing factor is defined for L1 measurement

  - Scheduling restriction stategy is defined for intra-band/inter-band ATG

  - Deactivated SCC measurement, [reuse legacy methodology]

- Charicteristics

  - RLM/BFD/CBD, reuse legacy procedure and requirements

  - Active BWP switching, reuse legacy cases and requirements

  - Interruptions

- For the requirements same as legacy, such as MRTD, pre-configured measurement gap activation/deactivation, [deactivated SCC measurement], RLM/BFD/CBD and active BWP switching, we prefer not to define test cases.

- For the requirements different from legacy and introduced for intra/inter-band CA ATG, such as RRM measurement(with scheduling restriction), L1 measurement and some of interruptions, we prefer to verify the UE behavior through test cases.

- Proposal 5: Based on the summaries of what we specified in core part and whether the exact requirements are reused from legacy TN, we suggest to verify the following test cases:

- TC1: Intra-frequency measurement(with scheduling restriction verified as well)

- TC2: Inter-frequency measurement(with scheduling restriction verified as well)

- TC3: L1-RSRP measurement

- TC4: The interruption of SCell addition/release

- TC5: The interruption of SCell activation/deactivation

- RAN4 discusses whether all these test cases need to verify two categories of UE antenna assumption:

- Category 1: common Rx beam in PCC and SCC band(s), including intra-band antenna array and Case 4 of inter-band

- Category 2: separate Rx beam in PCC and SCC bands, including other cases

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- Proposal 1: The performance verification should focus on the new functionalities introduced for R19 ATG that were not defined in R18 single carrier ATG, i.e. mainly the DL CA relevant features and antenna array relevant features.

Proposal 2: Reuse the general test configuration defined in R18 ATG, and the intra-band CA and/or inter-band CA relevant configuration is additional considered.

- The general test configuration includs bandwidth/duplexing, TDD pattern, RMC, OCNG, channel model, BS location, number of neighbour cells for test of antenna array.

- Proposal 3: Reuse the test method defined in R18 ATG for all antenna array cases including intra-band CA and inter-band CA.

- Proposal 4: Reuse the L3 and L1 accuracy requirement of legacy FR1 TN

- Proposal 5: Based on the summaries of what we specified in core part and whether the exact requirements are reused from legacy TN, we suggest to verify the following test cases:

- TC1: Intra-frequency measurement(with scheduling restriction verified as well)

- TC2: Inter-frequency measurement(with scheduling restriction verified as well)

- TC3: L1-RSRP measurement

- TC4: The interruption of SCell addition/release

- TC5: The interruption of SCell activation/deactivation

- RAN4 discusses whether all these test cases need to verify two categories of UE antenna assumption:

- Category 1: common Rx beam in PCC and SCC band(s), including intra-band antenna array and Case 4 of inter-band

- Category 2: separate Rx beam in PCC and SCC bands, including other cases

- Reference

- RP-243285, New WID: New WID on Enhancements for Air-to-ground network for NR, 3GPP TSG RAN Meeting #106.