3GPP TSG-RAN WG4 Meeting #114                                   R4-2504261

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on UE RF requirements for 7MHz CBW

Agenda Item:	7.8.2

Document for:	Approval

- Introduction

In RAN4#114, a WF [1] for 7MHz CBW was agreed and there are some issues need to be resolved. In this paper, we will provide our views on UE REFSENS for 7 MHz CBW for band n5 and n26.

2-1-1 Spectrum utilization Agreement: - 35 RBs will be supported.

- Discussion

  - REFSENS for 7MHz

In the last RAN4 meeting, RAN4 reached an agreement to apply NRB =35 for 7 MHz with 15 kHz SCS. Therefore, we can derive REFSENS for 7 MHz by NRB scaling. Table 1 shows the REFSENS of bands n5 and n26 from 38.101-1. Table 2 shows the scaling factor, which takes 5 MHz as the reference for Table 1. For n5, the REFSENS of 10 MHz and 15 MHz align with the scaling factor. The scaling factor for 7 MHz is 10*log10(35/25) = 1.5 dB. Therefore, the REFSENS of 7 MHz for n5 is -98 + 1.5 = -96.5 dBm. For n26, the REFSENS of 3 MHz also aligns with the NRB scaling. Therefore, the REFSENS of 7 MHz for n26 is -97.5 + 1.5 = -96 dBm.

Table 1 REFSENS of band n5 and n26 from 38.101-1

| Operating Band | SCS kHz | 3MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | -98 | -94.8 | -93 |
| n26 | 15 | -99.7 | -97.5 | -94.5 | -92.7 |

Table 2 Scaling factor for REFSENS of band 5 and n26 refer 5MHz

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | 0 | 3.2 | 5.0 |
| n26 | 15 | -2.2 | 0 | 3.0 | 4.8 |

Table 3 REFSENS of band n5 and n26 for 7MHz with 15kHz SCS

| Operating Band | SCS kHz | 7MHz (dBm) |
| --- | --- | --- |
| n5 | 15 | -96.5 |
| n26 | 15 | -96.0 |

Proposal 1: RAN4 to consider REFSENS as -96.5 dBm for 7MHz CBW with SCS 15kHz for band n5.

Proposal 2: RAN4 to consider REFSENS as -96 dBm for 7MHz CBW with SCS 15kHz for band n26.

- 3	Conclusion

In this contribution, we provide our views on UE REFSENS for 7 MHz CBW for band n5 and n26. The proposals are summarized as follows:

Proposal 1: RAN4 to consider REFSENS as -96.5 dBm for 7MHz CBW with SCS 15kHz for band n5.

Proposal 2: RAN4 to consider REFSENS as -96 dBm for 7MHz CBW with SCS 15kHz for band n26.

- 4	References

- R4-2502870, “WF on requirements for 7MHz channel bandwidth”, Ericsson, RAN4#114.