# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504262

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.1.1.2.1

Source: 	Xiaomi

Title: 	Power domain enhancements for single carrier

Document for:	Discussion

- Introduction

In Rel-19 UE RF WI, one of major objective was to study and specify power enhancement on single carrier with emission requirements relaxtion:

- Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

In this contributions, views on remaning open issues were provided.

- Discussion

Sub-topic 1-1: Applicable requirements

IBE requirements

#### RAN4#114 #### Issue 1-1-4: How to derive the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified with ΔRB =1 or ΔRB =0. FFS the specific offset value. - IBE is not used in the GB - Option 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions (Moderator: NRB,V and LCRB,V could be replaced by other terms, but the concept is similar) max-25-10⋅log10(NRB,V/LCRB,V), 20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V, -57dBm+10log10SCS/15kHz-PRB - Option 3: Others - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW

RAN4 agreed that IBE shall be applied in the range between the edge of extended CBW and edge of UE CBW instead of SEM.  The opening question is whether IBE applied for the guard-band in UE CBW and/or extend CBW.

From UE implementation perspective, we didn’t see the difficulty to meet the IBE within guard-band of UE CBW or extend CBW. The original reason why existing IBE not applied during guard-band was due to no RB is expected to be scheduled on guard-band which was served as transient frequency gap for channel filtering. However, in Rel-15 NR for wider BW operation with asymmetric CBW in network and UE side, it’s not precluded NW can schedule transmission in the guard-band from one carrier to another carrier with partially overlapped CHBWs for different UEs.

Proposal 1: Extension of the IBE is used in the guard band of original UE CBW, but not used in the guard-band of extended CBW.

For the applicable of IBE during the range between the edge of extended CBW and edge of UE CBW shall be same value as last unused RB in original UE CHBW with flat value; and the NRB and LRB shall follow original UE CHBW allocation.

For fully allocation scenario, IBE can be transformed as following:

max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

Proposal 2: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

- NRB and LRB shall follow original UE CHBW allocation.

- For fully RB allocation case: IBE following the equation as following: max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

RAN4#114 #### Issue 1-1-5: Modulation order for the extended IBE - Proposals - For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. Table. Comparison of emission level @ the first 1MHz adjacent to the UE CBW UE CBW (MHz). 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. SEM (dBm/30kHz); -18.2; -20.0; -21.2; -22.2; -23.0; -23.6; -24.2; -24.8; -24.0. IBE QPSK 26dBm (dBm/30kHz); -16.7; -18.7; -20.0; -21.0; -21.8; -22.5; -23.2; -23.7; -24.1; -25.0; -25.7; -26.3; -26.8; -27.2. IBE 16QAM 26dBm (dBm/30kHz); -19.6; -21.6; -22.9; -24.0; -24.8; -25.5; -26.1; -26.6; -27.1; -28.0; -28.6; -29.2; -29.7; -30.2. IBE BPSK 26dBm (dBm/30kHz); -12.0; -14.0; -15.3; -16.4; -17.2; -17.8; -18.5; -19.0; -19.5; -20.3; -21.0; -21.6; -22.1; -22.6. - WF - FFS whether the extended IBE with high order EVM could have impact on MPR for new inner RB allocation

On this issue, we share similar observation from proposal above, IBE requirements are more stringent than SEM requirements at first 1MHz for some cases with 16QAM/QPSK modulation order.  One possible way as suggested by company is to fix the EVM parameter as used by BPSK.

Proposal 3: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK.

Sub-topic 1-2: Approaches to enable MPR reduction for both scenario 1 and scenario2

RAN4#114 agreement #### Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible

Extended CBW approach was adopted for MPR reduction in last RAN4 meeting. From deployment scenario and network scheduling perspective, extended CBW/extended RB numbers on each side of UE CHBW can have wide range with the granularity from zero to half of maximum CHBW in FR1 i.e., 50MHz CHBW.  On the other side, the concept of extended CBW was used to introduce MPR reduction by converting outer RB allocation to inner RB allocation. From RAN4 requirements perspective, we shall consider the actual impact to MPR reduction, evaluation work and test effort to decide confined extend CBW range or fixed values for RAN4 requirement introduction.

Observation 1: Extended CBW approach adopted to serve following purpose

- Specify MPR applicability from outer RB allocation to inter RB allocation

- Specify requirement applicability between IBE and OOBE

Proposal 4: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules.

Based on evaluated results from several companies in previous RAN4 meetings, half BW extension on each side can enable inner RB MPR for whole region of UE CHBW. In order to convert all outer RB allocation to inner RB allocation on channel edge, the extended BW on one side shall not less than half of UE CBW. Otherwise, there are still inner RB and outer RB allocation under extended CBW, RAN4 needs to spend lots of effort to evaluate different cases to judge the suitable MPR or change the inner RB/outer RB definition with multiple breaking points. This will complicate the specification, and test and make the feature difficult to be implemented and deployed.

The intrinsic reason behind of MPR reduction was to revise the applicable frequency range between OBE (ALCR and SEM) and IBE i.e., from the channel edge of UE CBW to extended CBW with channel edge extension without degrading the system performance under target scenarios (scenario 1-1,1-2 and scenario 2 mentioned in the WF [1]).

In ITU regulation, 250% of actual bandwidth criteria (from transmission bandwidth center) was used to decide the frequency boundary between SEM/ACLR and spurious emission.  In existing RAN4 requirements, OOB boundary FOOB was specified as CBW+5MHz from channel edge.  Shifting the SEM applicable frequency range shall not conflict with ITU regulation which means the upper bound of extended BW on one side shall not exceed CBW-5MHz.

Proposal 5: The extended BW on one side shall be in the range of CBW/2 ~ CBW – 5MHz.

Proposal 6: Fixed extended BW on one side as CBW/2 (option 1 with asymmetric allowed).

- Power enhancement with extend CHBW only applicable for UE CHBW larger than or equal to 10MHz

Sub-topic 1-3: Signaling and others

On signalling aspects i.e., how to indicate extended RBs/BW location is strongly depending on the discussion/decision on Sub-topic 1-1 for possible extended BW/RB number on one side and associated MPR applicability.

From deployment perspective, the extended BW needs to be combined with BS channel bandwidth or aggregated bandwidth for multi-carrier depending deployment scenario. However, from UE perspective, UE only needs to know the applicable requirements and MPR values with extended BW, the deployment scenario and BS channel bandwidth or aggregated bandwidth can be transparent to UE. Network shall take the responsibility to guarantee system performance without degradation on adjacent system and take proper scheduling to enable the feature or not.

If RAN4 agreed to fix the extended BW on one side as half/BW of UE CBW for inner RB MPR applicability, then UE only needs to know whether enabling power boosting or not and/or on which sides or both sides as well as network can guarantee the actual BS bandwidth or spectrum holding can be equal or large than the gap specified in RAN4 for extended BW.

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 7: Postpone the decision and discussion on the detailed signalling on extend CBW location and width which pending on decision of extended UE CBW for requirements as fixed value (on/off) or variable values.

3		Conclusion

In this controbution, views on remaining open issues are provided.

Sub-topic 1-1: Applicable requirements

IBE requirements

Proposal 1: Extension of the IBE is used in the guard band of original UE CBW, but not used in the guard-band of extended CBW.

Proposal 2: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

- NRB and LRB shall follow original UE CHBW allocation.

- For fully RB allocation case: IBE following the equation as following: max-25,20⋅log10EVM-3,-57dBm+10log10SCS/15kHz-PRB

Proposal 3: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK.

Sub-topic 1-2: Approaches to enable MPR reduction for both scenario 1 and scenario2

Observation 1: Extended CBW approach adopted to serve following purpose

- Specify MPR applicability from outer RB allocation to inter RB allocation

- Specify requirement applicability between IBE and OOBE

Proposal 4: RAN4 shall decide confined fixed value(s) for extended CBW (fixed ratio to UE CHBW) with associated MPR and requirement applicability rules.

Proposal 5: The extended BW on one side shall be in the range of CBW/2 ~ CBW – 5MHz.

Proposal 6: Fixed extended BW on one side as CBW/2 (option 1 with asymmetric allowed).

- Power enhancement with extend CHBW only applicable for UE CHBW larger than or equal to 10MHz

Sub-topic 1-3: Signaling and others

Observation 2: Actual BS channel bandwidth/aggregated bandwidth and deployment scenarios can be transparent to UE.

Proposal 7: Postpone the decision and discussion on the detailed signalling on extend CBW location and width which pending on decision of extended UE CBW for requirements as fixed value (on/off) or variable values.

Reference

[1] R4-2420527, WF on Power domain enhancement, Huawei

[2] R4-2503013	WF on power domain enhancement, Huawei