3GPP TSG RAN WG4 Meeting #114bis	R4-2504268

Wuhan, P. R. China, 7th – 11th April 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	7.19.3

Document for: 	Discussion

# Introduction

From the WF in [16], the following analysis is required with regards to the test setup for AI/ML based beam management:

Issue 4-1: Baseline test setup

Agreement:

- Discuss both single AoA based and multiple AoA based set up

  - Continue to study the two multiple AoA candidate setups

    - One option to continue the discussion proposed as following:

      - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability

        - AI/ML BM performance centric, not CDL channel emulation centric.

        - Define test system candidates which are already feasible

      - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance.

  - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

Issue 4-2: Channel model

Agreement:

- Use CDL-based channel model as starting point

In this contribution, we elaborate further our views on single-AoA and multi-AoA test setups.

# Discussion

## Single-AoA testing

Regarding the single-AoA test setup, many proposals and discussions have been provided in the last RAN4 meetings, including R&S contributions in [8]-[12]. In our understanding the discussion has come to a phase, where a constructive progress depends mainly on the clarification of certain assumptions for the AI/ML BM requirement. In the following we try to identify, assess and summarize shortly these questionable and unclarified assumptions.

As per the WF in [16], the channel model to be used needs to be based on CDL. In the previous meetings there has been discussions whether such a channel model can be supported in a single-AoA system or not. In our understanding the channel model type (TDL or CDL) refers only to the model taken as reference from TR 38.901 [2] or TR 38.870 [3] and not how they are finally generated in the test equipment. The spatial related parameters (AoD, ZoD, AoA, ZoA) in the CDL models describe the clusters, but not the implementation (e.g. it is very clear that no AoD/ZoD can be physically emulated in either single-AoA or multi-AoA chamber).

The single/multi-AoA emulation of the channel models in the test systems depends on the UE Rx antenna assumption, which act as a filter of the AoA/ZoAs in space, converting them into amplitude taps for the UE BB. We can differentiate two cases:

- If the UE Rx antenna pattern is known (i.e. assumed, declared or measured), it is emulated by the TE by convoluting it with the channel model. The resulting signal can be fed to the UE using a single-AoA link.

- If the UE Rx antenna pattern is unknown, it cannot be emulated by the TE. In this case, the signal needs to be provided to the UE from multiple AoAs, based on assumptions met for the channel clusters.

Observation 1: (Single-AoA) CDL channel model can be emulated in a single-AoA setup, if UE Rx antenna pattern is known, while a multi-AoA setup is to be considered, if UE Rx antenna pattern is unknown.

When discussing about knowing or not the UE Rx antenna, another question is whether the antenna performance is considered as an integral part of the UE AI/ML BM requirement or not. This question arises since many companies see the AI/ML BM requirement only based on L1-RSRP scalars, and not necessarily considering how they are measured in the UE (i.e. taking into account the UE Rx antenna performance). In this sense:

- If the UE Rx antenna performance is not considered for the AI/ML BM requirement, the knowledge of the UE Rx antenna pattern is not required. An UE antenna assumption can be used instead.

- If the UE Rx antenna performance is indeed part of the AI/ML BM requirement, it is required to consider the UE Rx antenna pattern during testing.

Observation 2: (Single-AoA) An UE Rx antenna pattern assumption can be used for testing, if UE Rx antenna performance is not part of AI/ML BM requirement, while a real UE Rx antenna pattern is to be considered, if UE Rx antenna performance is part of the requirement.

Another aspect affecting the UE Rx antenna pattern, is the assumption whether the UE AI/ML BM requirement considers the UE Rx beam sweeping as an input to the AI/ML model. So:

- If UE AI/ML BM requirement includes the UE Rx beam sweeping performance, the UE Rx antenna pattern definition becomes challenging.

- If UE AI/ML BM requirement does not include the UE Rx beam sweeping performance, a static UE Rx antenna pattern can be considered.

Observation 3: (Single-AoA) A static UE Rx antenna pattern can be used for testing, if UE Rx beam sweeping performance is not part of AI/ML BM requirement, while an UE Rx antenna pattern definition becomes challenging, if UE Rx beam sweeping performance is part of the requirement.

Finally, the last aspect which was extensively discussed when evaluating the single-AoA vs. multi-AoA setup is the question whether the UE Rx beam ID is considered as an input to the AI/ML model when deriving the requirements or not. In this case:

- If UE Rx beam index is not considered as input for the AI/ML model, a single-AoA test setup can be defined as baseline using one of the options presented in [12].

- If UE Rx beam index is considered as input to the AI/ML model, the test should give the possibility to the UE to use different UE Rx beams for measurement, which leads more to a multi-AoA test setup.

Observation 4: (Single-AoA) Single-AoA testing is sufficient, if only L1-RSRP values are relevant for the UE AI/ML BM requirement, while multi-AoA testing is rather required, if also UE Rx beam index is needed (as input for the AI/ML model.

To facilitate the discussion for single/multi-AoA testability we propose:

Proposal 1: (Single-AoA) RAN4 to discuss and conclude whether following aspects shall be part of the UE AI/ML BM requirement and/or testing: a) UE Rx antenna pattern knowledge, b) UE Rx antenna performance, c) UE Rx beam sweeping performance, d) UE Rx beam index (as input for the AI/ML model).

## Multi-AoA Testing

Regarding the multi-AoA test setup, we follow in this contribution the guidance agreed in the WF [16], in order to reach to a base line system proposal.

### Candidate system

The first step is to define a feasible candidate system.

The current definition of the multi-AoA legacy RRM baseline system based on IFF is called eIFF and is specified in TS 38.508-1 [7], Annex B.2.6. It prescribes the support of 2 simultaneous active probes with relative angular offsets of 30°, 60°, 90°, 120° and 150°. These are required for the test cases using Angular Setups 1-5 in TS 38.133 [6], Annex A.3.15.1-5.

However, as described in our previous contribution in [12], most current commercial implementations of eIFF test systems deploy 4x CATRs with the layout (or an equivalent) as shown in Figure 1. This is the resulting layout to support all (6) the required angular offsets with the minimal number (4) of static CATRs.

Figure 1 – Commercial implementation of eIFF system with 4 IFF probes / CATRs ( feIFF)

The implicit understanding of such implementation has been already used to define the Angular Setups 6-7 in TS 38.133 [6], Annex A.3.15.6-7. These angular setups require up to 3 active probes and are used in the test cases defined for Enhanced MIMO and Multi Rx test cases (Rel-19).

For the purpose of multi-AoA CDL emulation, the system can be further enhanced to enable the simultaneous activation of all the 4 available probes / CATRs. For the purpose of AI/ML BM testability discussion, this system can be called “feIFF” (further enhanced Indirect Far Field). Such enhancement is feasible, thus we can consider it as a candidate to be evaluated for the AI/ML BM multi-AoA baseline system.

Proposal 2: (Multi-AoA) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes with probes (CATRs) layout at 0°, 30°, 90°and 150° as feIFF test setup.

Proposal 3: (Multi-AoA) Consider the feIFF test setup as candidate for the AI/ML BM baseline system and analyze its suitability.

In summary, the feIFF test setup has the following technical capabilities and advantages:

- 4 Active 2x2 MIMO probes, with distinctive angular separation (max 150°)

- Big quiet zone of at least 30cm for all AoAs  real black box testing

- Best far field conditions with lowest MU (compared to DFF-based systems)  highest (L1-RSRP) accuracy measurements

- Reference for legacy requirements  side conditions (RF-RRM / cross-feature) ensured

Observation 5: (Multi-AoA) The feIFF test setup supports 4 Active 2x2 MIMO probes with distinctive angular separation of up to150°, overall big quiet zone of at least 30cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

In addition, the feIFF test setup has practical advantages as a superset of the eIFF setup:

- eIFF is available and matured as already broadly used for FR2 testing

- (f)eIFF supports all RF/ Demod / RRM FR2 tests up to release 19 (incl. Multi-Rx, eMIMO)

- If feIFF concluded to be suitable for AI/ML  one single system for full conformance testing coverage

- Continuity (eIFFfeIFF) in the cost & complexity-sensitive FR2 testing ecosystem

Observation 6: (Multi-AoA) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

### Simplified CDL-C channel model

The second step is to define a simplified CDL based channel model supported by the candidate system.

Using the UMi CDL-C channel model Table 7.2.2-3 from TR 38.827 [3] as a reference, the following simplifications are applied:

- The clusters that have the same AoAs can be merged, following Observation 11 in [17]. To do so, the delay for the first tap is kept and the power of the three taps is combined, resulting in +3dB for clusters #2 and #6.

- The weak clusters that have a limited impact on the beam management (i.e. <-10dB) can be removed, following Observation 8 to Observation 10 in [17], in order to reduce the number of clusters.

Implementing #1 results in a power scaling of all clusters. This is done by adjusting the power for the strongest cluster (i.e. #6) to 0dB and reducing the remaining clusters by the difference to cluster #6. With this scaling, there are more clusters falling below -10dB from the strongest cluster and then they are also removed. The resulting channel model parameters are presented in Table 1.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz 
after merging clusters with similar AoAs and removing weak clusters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 |  | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 |  | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 |  | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 |  | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -10.4318 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -10.1318 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -13.7318 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -14.1318 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -11.7318 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -16.2318 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -16.9318 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -16.9318 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -18.8318 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -20.1318 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -19.0318 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -18.7318 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -24.6318 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -25.8318 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

Proposal 4: (Multi-AoA) Use channel model parameters for UMi CDL-C at 28 GHz presented in Table 1 (i.e merging clusters with similar AoAs and removing weak clusters) as starting point for further channel model simplifications.

The maximum angular spread required to emulate the down selected clusters in Table 1 is close to 150°, which can be covered by the probe layout of the proposed candidate setup feIFF.

Observation 7: (Multi-AoA) The required total AoA spread of 150° can be emulated with the feIFF probe layout.

Looking at further channel model simplifications to fit the candidate test system feIFF described in the previous section, there is one clear change to be implemented: the elevation spread (i.e. ZoA) and the intra-cluster elevation angle spread (i.e. CZSA) must be flattened, to fit the probe layout placed in a single plane. Further discussion could determine whether a flat 90º (i.e. as done for FR1 MIMO OTA as described in TR 38.827 [3]) should be used, or it could be possible to select the weighted average of the ZoA for the remaining clusters (i.e. 74.51º). In Table 2, the weighted average is used for further discussion. It can be decided later whether this single elevation is tested in the chamber by selecting the appropriate DUT testing conditions/orientations.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Looking further to how to this channel model can be implemented in the candidate test system feIFF in previous section, we see 3 possible options:

- Option A: Using the simplified channel model in Table 2, the strongest cluster (i.e. cluster #6) is aligned to one of the available probes (i.e. 30º). In this approach, like done for MIMO OTA, the test system calculates the weights to simulate the AoAs in between the probes (i.e. reflectors). With this option, there is no need to further modify the channel model table. On the other hand, the main drawback is that there are several clusters falling outside the probe spread.

- Option B: Using the simplified channel model in Table 2, the mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes (i.e. 75º). With this option, there is no need to further modify the channel model table, and all clusters (nearly) fall into the probe spread.  The main drawback is that the strongest cluster fall between probes separated 60º apart.

- Option C: Using Option B as starting point, AoAs in the channel model parameter table are modified to align with the closest probe location. Here, every cluster will be aligned with one probe location and allows the intra-cluster angle spread (i.e. CASA) to be set to 0º for a very much simplified channel model. This results in the simplified channel model in Table 3.

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Observation 8: (Multi-AoA) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options: Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

The resulting alignment of the clusters with respect to the reference probe layout for feIFF is presented in Figure 2. Please note that in the figure the channel model AoAs have been transformed to the chamber probe coordinates, and that the strongest cluster (i.e. cluster #6) has been highlighted with a different marker (▲).

Figure 2 – Simplified UMi CDL-C at 28 GHz alignment to the candidate test system feIFF

Observation 9: (Multi-AoA) The evaluation of Options A and B (Table 2) becomes more complex, if the effect of the test setup / probe layout has to be included.

In this case, it is required first to define the metric to compare with the channel model parameter table, in order to determine accordingly the probe weights.

It has to be noted that these probe weights are different per BS Tx beam, in order to simulate the resulting Power Angular Spectrum (PAS) based on the mean cluster power. The distribution of the PAS can be calculated by directly filtering the clusters in Table 2 with the full set of BS Tx beams (i.e. Set A). In addition to the mean PAS distribution, the power fluctuations over time for a given BS Tx beam from the different AoAs, is determined by the cluster angular spread, the intra-cluster angular spread and the relative ray angles, as well as the initial ray phases defined in TR 38.827 [3].

To emulate these power fluctuations properly, the relative ray phases should be emulated properly by the test setup. Any phase inaccuracy could reduce the validity of the test approach. To avoid this, a reliable relative phase emulation among the rays is required, which is ensured by using an IFF based approach.

Observation 10: (Multi-AoA) The emulation of the relative ray phases with the feIFF is highly accurate and should satisfy the requirement for the accuracy for the cluster power fluctuations over time.

As mentioned in previous contributions ([8] - [12]), the metric used so far for MIMO OTA (which is CDL channel emulation centric) is not suitable and new metric(s) have to be defined.

These metric(s), should characterize the following aspects:

- Sufficient power variation in the angular domain, dependent on the BS Tx beam.

- Sufficient power variation in the time domain, for a given BS Tx beam.

- Power accuracy in the test volume.

Proposal 5: (Multi-AoA) Further discuss the metric(s) to evaluate channel model implementation with the candidate solutions.

On the other hand, Option C (Table 3) presents an extreme case of the CDL simplification, where the the clusters are fully aligned with the probes. As such, there is no need to optimize the probe weights, which was required above for Options A and B. Therefore, the performance of the AI/ML BM prediction with this simplified channel model can be evaluated right away, since the channel model parameters in Table 3 contain all the required information.

Observation 11: (Multi-AoA) For Option C (Table 3), there is no need to optimize the probe weights, and channel model parameters in Table 3 contain all the required information.

Proposal 6: (Multi-AoA) Evaluate the AI/ML BM prediction performance when using the simplified channel model parameters for Option C (Table 3).

### AI/ML BM Simulations

The third step is to evaluate the candidate system and the simplified CDL channel for AI/ML BM suitability, by using system level simulations.

Similarly to our contribution in RAN4#113 [13], we performed simulations for our candidate system (feIFF, section 2.2.2) and proposed channel (simplified CDL, section 2.2.3) with the detailed configuration described in Table 4:

Table 4 – Simulation configuration

| Parameter | Value | Notes |
| --- | --- | --- |
| Radio Link | 28 GHz Carrier 120 kHz SCS 8 RBs |  |
| Antenna Assumptions | BS Tx Array 8x4 UE Rx Array 4x1 |  |
| AI/ML Model | CNN DNN | Trained with the same channel model as used for prediction |
| Set A beams (BS Codebook) | 32 |  |
| Set B beams | 8 | Selection as in Figure 3 |
| Channel Model Type | CDL-C Umi |  |
| Channel Model Details (1) | 24 Clusters (Reference) | Table 7.2.2-3 from TR 38.827 [3] |
| Channel Model Details (2) | 6 Clusters (Option A/B) | Table 2 |
| Channel Model Details (3) | 6 Clusters (Option C) | Table 3 |

Figure 3 – Selection of 8 Set B Beams to predict 32 Set A Beams

The prediction accuracy results have been summarized in Table 5 and visualized in Figure 4.

Table 5 – Prediction accuracy probability

|  | CNN 24 Clusters (Reference) | CNN 6 Clusters (OptionA/B) | CNN 6 Clusters (OptionC) | DNN 24 Clusters (Reference) | DNN 6 Clusters (OptionA/B) | DNN 6 Clusters (OptionC) |
| --- | --- | --- | --- | --- | --- | --- |
| Top 3 Set A Beams | 98.68% | 99.34% | 99.38% | 98.97% | 99.30% | 99.38% |
| Top 1 Set A Beam | 87.74% | 88.52% | 88.76% | 87.17% | 88.20% | 87.91% |

Figure 4 – Prediction accuracy probability

The results of these simulations, show that the prediction accuracy probability improves just slightly when the number of clusters reduces from the full theoretical CDL model (as per Table 7.2.2-3 from TR 38.827 [3]) to the simplified channel model (as per Table 2 or Table 3 in this contribution), however at a negligible scale (less than 1%).

Observation 12: (Multi-AoA) Based on simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (improvement of ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Certainly, further simulations can be done and provided, by varying more parameters from the simulation assumptions in Table 4. Due to time constraints, such simulations can be provided only in the coming meetings. However, at least from the results acquired at this stage, we would consider the channel models for feIFF in Table 2 and Table 3 as suitable for the AI/ML BM testing.

Proposal 7: (Multi-AoA) Consider the simplified channel models for feIFF with 6 Clusters in Table 2 and Table 3 as suitable for AI/ML BM testing.

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: (Single-AoA) CDL channel model can be emulated in a single-AoA setup, if UE Rx antenna pattern is known, while a multi-AoA setup is to be considered, if UE Rx antenna pattern is unknown.

Observation 2: (Single-AoA) An UE Rx antenna pattern assumption can be used for testing, if UE Rx antenna performance is not part of AI/ML BM requirement, while a real UE Rx antenna pattern is to be considered, if UE Rx antenna performance is part of the requirement.

Observation 3: (Single-AoA) A static UE Rx antenna pattern can be used for testing, if UE Rx beam sweeping performance is not part of AI/ML BM requirement, while an UE Rx antenna pattern definition becomes challenging, if UE Rx beam sweeping performance is part of the requirement.

Observation 4: (Single-AoA) Single-AoA testing is sufficient, if only L1-RSRP values are relevant for the UE AI/ML BM requirement, while multi-AoA testing is rather required, if also UE Rx beam index is needed (as input for the AI/ML model.

Observation 5: (Multi-AoA) The feIFF test setup supports 4 Active 2x2 MIMO probes with distinctive angular separation of up to150°, overall big quiet zone of at least 30cm, lowest MUs and ensures preservation of (RF-RRM) side conditions.

Observation 6: (Multi-AoA) The feIFF test setup is a feasible upgrade (superset) of the existing legacy eIFF setup. As such, if feIFF is concluded as capable for AI/ML BM testing, a single system covers the the whole conformance testing, ensuring continuity in the cost sensitive FR2 testing ecosystem.

Observation 7: (Multi-AoA) The required total AoA spread of 150° can be emulated with the feIFF probe layout.

Observation 8: (Multi-AoA) The simplified channel model parameters for UMi CDL-C can be implemented in the candidate test system feIFF, with 3 possible options: Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

Observation 9: (Multi-AoA) The evaluation of Options A and B (Table 2) becomes more complex, if the effect of the test setup / probe layout has to be included.

Observation 10: (Multi-AoA) The emulation of the relative ray phases with the feIFF is highly accurate and should satisfy the requirement for the accuracy for the cluster power fluctuations over time.

Observation 11: (Multi-AoA) For Option C (Table 3), there is no need to optimize the probe weights, and channel model parameters in Table 3 contain all the required information.

Observation 12: (Multi-AoA) Based on simulations with 8 Set B & 32 Set A beams, there is only a negligible effect (improvement of ~1%) on the prediction accuracy for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for feIFF (Options A/B/C - 6 clusters).

Proposal 1: (Single-AoA) RAN4 to discuss and conclude whether following aspects shall be part of the UE AI/ML BM requirement and/or testing: a) UE Rx antenna pattern knowledge, b) UE Rx antenna performance, c) UE Rx beam sweeping performance, d) UE Rx beam index (as input for the AI/ML model).

Proposal 2: (Multi-AoA) Define the implementation of eIFF test setup with 4x active 2x2 MIMO probes with probes (CATRs) layout at 0°, 30°, 90°and 150° as feIFF test setup.

Proposal 3: (Multi-AoA) Consider the feIFF test setup as candidate for the AI/ML BM baseline system and analyze its suitability.

Figure 1 – Commercial implementation of eIFF system with 4 IFF probes / CATRs (  feIFF)

Proposal 4: (Multi-AoA) Use channel model parameters for UMi CDL-C at 28 GHz presented in Table 1 (i.e merging clusters with similar AoAs and removing weak clusters) as starting point for further channel model simplifications.

Proposal 5: (Multi-AoA) Further discuss the metric(s) to evaluate channel model implementation with the candidate solutions.

Proposal 6: (Multi-AoA) Evaluate the AI/ML BM prediction performance when using the simplified channel model parameters for Option C (Table 3).

Proposal 7: (Multi-AoA) Consider the simplified channel models for feIFF with 6 Clusters in Table 2 and Table 3 as suitable for AI/ML BM testing.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

# References

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V18.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.3.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.101-4 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.133 V18.8.0, “NR; Requirements for support of radio resource management”

- 3GPP TS 38.508-1 V18.5.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2418941, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2501726, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2407333, “On AI/ML RAN4 Use Cases: Beam Management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2500403, “Testability for AI/ML beam management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025